| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Problems in idle Mode specifications |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Vladimir Yanover Alvarion Ltd. | E-mail: vladimir.yanover@alvarion.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot on the Draft Amendment IEEE P802.16m/D6 |  |
| Abstract | Language clarification in idle Mode section |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Problems in idle Mode specifications

Vladimir Yanover

Alvarion Ltd.

# Problem 1

- There are multiple terms interchangeably applied to the ABS and the AMS to indicate their relationship with certain PGs (belong to the group, be assigned to the group, be within the group etc.)

- The text is using term “operation mode” which has no definition in the draft.

- Paging cycle and paging offset are not properties of a paging group; they are assigned to specific AMS at the time of deregistration or location update. So it is incorrect to say that “paging groups may have different paging cycles and paging offsets”

See remedy 3 for support of several paging groups.

# Remedy 1

[Change in 16.2.18, p.391, line 55]

16.2.18 Idle mode

An ABS may be statically assigned to a member of one or more paging groups that may have different paging cycles and paging offsets. Being assigned to a paging group, the ABS shall advertise the paging group ID (PGID) in the PGID_Info message.

An AMS are assigned during deregistration or location update, to one or more paging groups and, per paging group, a specific paging cycle and paging offset. The values of paging cycle and paging offset can be different among AMSs assigned to same paging group.

When an AMS capable of WirelessMAN-OFDMA Advanced co-existing System operations selects a Mixed Mode ABS as a preferred ABS, the AMS may stay in the Lzone and perform the legacy Idle Mode operation as specified in 6.3.23, which is referred to as the AMS being in Legacy Idle operation mode. If An AMS in Legacy Idle operation mode may decides to change its operation mode, then the AMS shall perform network reentry from Idle Mode as specified in 6.3.23 in the new operation mode. The operation mode change includes the AMS’s movement between and handover to a WirelessMAN-OFDMA Advanced co-existing System BSs operating in different mode or perform zone switching from LZone to MZone of a the mixed mode ABS. And the This decision may be based on the detection of a new operation mode. When an AMS is paged in the Lzone of a mixed mode ABS, the AMS shall perform the network reentry in the LZone of the ABS and may switch to the MZone of the ABS using Lzone to Mzone handover handoff procedures as defined in 16.2.6.4.1.2.1.

# Problem 2

Text in p.392, line 25 shows that the AMS must differentiate between the primary paging group and secondary paging groups. For example, in p.402 line 62

In case an AMS belongs to multiple paging groups, it starts Paging Group Location Update Timer (PG_LU_TIMER) when it leaves primary paging group and enters a secondary paging group.

However there is no indication in DREQ-RSP message or elsewhere how the AMS is informed on the primary / secondary status of the PG.

# Remedy 2

Left to 802.16m TG to resolve the problem.

# Problem 3

Need a consistent explanation of how this primary / secondary paging groups system works. Remove numerous duplications.

# Remedy 3

[Change in 16.2.18, p.392, line 1]

An AMS may be assigned to one or more paging groups. If an AMS is assigned to multiple paging groups, it may be assigned a common paging cycle with different multiple paging offsets within a paging cycle where each paging offset correspondings to a separate different paging groups; in this case the smallest offset shall be assigned to the primary paging group.  The  AMS  is  not  required  to  perform  location update  when  it  moves  within  its assigned paging groups. The assignment of multiple paging offsets to an AMS allows the AMS to monitor paging message from different paging groups.

[Change in 16.2.18, p.392, line 25]

By monitoring the PGIDs advertised by its preferred ABS during a paging listening interval, an AMS determines if it is within its primary paging group or within a secondary paging group. The AMS determines that it is within a Paging Group if the PGID of that Paging Group is advertised by the AMS's preferred ABS. Otherwise the AMS determines that it is not within said Paging Group. If the AMS determines that it is in its primary paging group, the AMS wakes up at its primary paging offset and responds to paging messages that are sent in the primary paging offset and are addressed to it. If the AMS determines that it is not in it's primary paging group, and that one or multiple secondary paging groups are present, the AMS wakes up at the shortest paging offset and responds to paging messages that are sent in during this paging offset and are addressed to it. If the AMS determines that none of the paging groups it has been assigned to are present, the AMS shall perform a location update.

# Problem 4

Paging Groups, Paging Cycle(s) and paging offsets are assigned to the AMS by the Paging Controller in the network; different AMSs assigned to same Paging Group may be handled by multiple Paging Controllers.  So there is no network entity that can be designed / configured to fulfill requirements of the 3rd paragraph of the page 392.

# Remedy 4

Clarify or remove the paragraph

[Change in 16.2.18, p.392, line 8]

When the AMS is assigned to multiple paging groups with the same paging cycle and different paging offsets, the primary paging group is the one with the smallest offset. The time difference between two adjacent paging offsets should be long enough so that the ABS can (i) send a paging message to the AMS in the primary paging offset within the paging cycle, (ii) when the AMS is in the primary paging group, receive a response to the paging message by the AMS before the secondary paging offset, and (iii) retransmit the paging message to the AMS at the secondary offset within the same paging cycle only if a response to the paging message in the primary paging offset is not received.

# Problem 5

Here is the citation from 16.2.18.2.3, p.400 line 53

The AMS derives the start of the paging listening interval based on the paging cycle and paging offset. The

paging listening interval shall comprise of the superframe whose superframe number meets the condition.

modulo PAGING_CYCLE == PAGING_OFFSET

…

At the beginning of the paging listening interval, the AMS shall scan and synchronize on the A-PREAM-

BLE of its preferred ABS and decode the P-SFH of the ABS.

The ABS shall transmit the PGID_Info at a predetermined location in the paging listening interval in order

to advertise the paging group(s) that is supported by the ABS. The PGID_Info shall be transmitted by the

ABS regardless of whether or not there any notifications for AMSs.

The green highlighted part is meaningless as “the paging listening interval” means the paging listening interval discussed previously, in the yellow highlighted part. However, the yellow highlighted part is about operations of specific AMS in its paging listening interval. How does the ABS know that this specific AMS selected the ABS as the preferred one? How does it know the paging listening intervals assigned to the AMS in all paging groups the AMS is assigned to?

# Remedy 5

Left to 802.16m TG to resolve the problem.