| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on the E-MBS System Configuration in IEEE P802.16m (16.2.3.30 & 16.9.3.1) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D6,” in response to the IEEE 802.16 Working Group Sponsor Ballot |  |
| Abstract | Cleanup for the E-MBS Configuration on AAI_SCD message supporting E-MBS on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text on the E-MBS System Configuration in IEEE P802.16m (16.2.3.30 & 16.9.3.1)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Instructions

IEEE 802.16m Amendment Draft Standard [1] describes the Enhanced Multicast Broadcast Service (E-MBS).

According to the IEEE 802.16m Amendment Draft Standard [1], some E-MBS Configuration Parameters transmitted via AAI_SCD message have been proposed as follows.

- Zone_Allocation-Bitmap

- ZF

- MSI Length(NMSI)

- E-MBS AAI frame offset

Those parameters are essential for the E-MBS operation, but they do not support well in the case of following scenario.

Figure 1 shows the current zone allocation bitmap where subband index 1~5 are used for E-MBS zone#1 and subband index 6~8 are used for E-MBS zone #2. As shown the Figure 1, some ABSs dependent on the belonging to the E-MBS Zone, Zone Allocation Bitmap may set according to using SLRUs for the E-MBS Zone as follows;

- Any ABS, which belongs to E-MBS Zone #1 only, may set subband index 1~5 for E-MBS region as well as E-MBS Zone #1

- Any ABS, which belongs to E-MBS Zone #1 and #2, may set subband index 1~8 for E-MBS region as well as 1~5 for E-MBS Zone #1 and 6~8 for E-MBS Zone #2

- Any ABS, which belongs to E-MBS Zone #2 only, may set subband index 6~8 for E-MBS region as well as E-MBS Zone #2

However, current Zone Allocation Bitmap does not support E-MBS region as well as E-MBS Zone especially in the case of any ABS belonging to E-MBS Zone #2 only. As shown the Figure 1, subband index 1~5 are not used neither E-MBS region nor non E-MBS region.

Figure 1 – Zone allocation bitmap for the E-MBS (subbnad index 1~5 are for zone #1 and subband index 6~8 are for zone #2)

Therefore, in this contribution, E-MBS configuration parameters (i.e., ZF, Zone Allocation Bitmap) are modified more efficiently and zone is divided using TDMed manner.

# E-MBS Region and E-MBS Zone Allocation Bitmaps for E-MBS Operation

In order to support E-MBS avoiding the problem mentioned above, E-MBS Zone is assigned with following procedure as shown Figure 2.

- Step 1: Check whether current carrier is mixed or dedicated carrier

- Step 2: Divide the subbands into unicast region and multicast region where the multicast region may be used for E-MBS – only for the mixed carrier

- Step 3: Divide the multicast region into E-MBS region and non-E-MBS region

Figure 2 – flow chart to assign E-MBS Zone

As mentioned above, to divide the subbands into unicast region and multicast region, number of subbands for unicast (UNsubbnad) can indicate the subands to be used unicast and the length of UNsubbnad is 5/4/2 for 20/10/5MHz. Over the 1 MSI, region bitmap indicates the E-MBS region (i.e, set to “1”) and non E-MBS region (i.e., set to “0”) and the length is 2/4/8/16 according to the MSI length. The bits set to “1” for E-MBS region can be divided into several E-MBS Zones which the ABS belongs to. If the use of resource is changed between one superframe and next superframe, the bit “1” is set in the location the same as the superframe in the E-MBS zone allocation bitmap. Otherwise, the bit “0” is set in the E-MBS zone allocation bitmap. The length of E-MBS zone allocation bitmap is the number of bits set to “1” in the E-MBS region bitmap – 1 and the total number of E-MBS zones which the ABS belongs to is the number of bits set to “1” in the E-MBS Zone Allocation bitmap.

Figure 3 shows an example of proposed E-MBS zone allocation in mixed carrier in 10MHz system bandwidth where 1 MSI is 16superframes. In the example, let the unicast subbands and multicast subbands be 4 and 6 each. Therefore, the value of UNsubband is 4. Consider the E-MBS region bitmap is 1111111100001111 and E-MBS Zone Allocation bitmap is 00001001000. In this case superframe 0~7 and 12~15 are used for E-MBS and the number of E-MBS zones is 3.

Figure 3 – Proposed Region Bitmap and E-MBS Zone Allocation Bitmap in mixed carrier (E-MBS Region allocation bitmap: 1111111100001111; E-MBS Zone Allocation bitmap: 00001001000)

# Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Modify the following rows in Table 716-AAI_SCD Message of 802.16m/D6]

Table 716 – AAI_SCD Message

| M/O | Attributes/Array of attributes |  | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| … | ... |  | … | … | … |
| O | E-MBS Configuration Parameters | Zone_Allocation-BitMap | DSACMAX-1 | See Section 16.9.3.1 | Present when E-MBS is configured |
|  |  | ZF | 1 | Zone Flag bit. Indicates the use of the last zone. 0b0: Unicast 0b1: E-MBS See section 16.9.3.1 |  |
|  |  | # of subbands for unicast (UNsubband) | variable | Number of subbands for unicast zone and the size of UNsubband is as below. 5bits for 20MHz 4bits for 10MHz 2bits for 5MHz |  |
|  |  | MSI | 2 | The Length of an MSI in units of the number of superframes 0b00: 2 superframes, 40ms (NMSI =2) 0b01: 4 superframes, 80ms (NMSI=4) 0b10: 8 superframes, 160ms (NMSI=8) 0b11: 16 superframes, 320ms (NMSI=16) |  |
|  |  | E-MBS AAI frame offset | variable | The location of the AAI frame where the E-MBS data burst ends MSI length == 0b00: 3bits MSI length == 0b01: 4bits MSI length == 0b10: 5bits MSI length == 0b11: 6bits |  |
|  |  | E-MBS Region Bitmap | Variable | The length is determined based on the length of MSI. MSI length == 0b00: 2bits MSI length == 0b01: 4bits MSI length == 0b10: 8bits MSI length == 0b11: 16bits See section 16.9.3.1 |  |
|  |  | E-MBS Zone Allocation Bitmap | Variable | The length is determined based on the number (M) of bits set to “1” in the E-MBS region bitmap. The length is M-1 See section 16.9.3.1 |  |
| … | ... |  | … | … | … |

[Remedy2: Modify the line 1-50 in page 850 of 16.9.3.1 of 802.16m/D6]

Number of Subbands for unicast (UNsubband) : Number of Subbands for Unicast indicates the last some subband indics used for unicast and the length of UNsubbnad is 5/4/2 for 20/10/5MHz.

E-MBS Region Bitmap : E-MBS Region Bitmap consists of superframe indices reserved for E-MBS region over 1 MSI, E-MBS Region bitmap indicates the E-MBS region (i.e, set to “1”) and non E-MBS region (i.e., set to “0”) and the length is 2/4/8/16 according to the MSI length. The bits set to “1” for E-MBS region can be divided into several E-MBS Zones which the ABS belongs to.

E-MBS Zone Allocation Bitmap : E-MBS Zone Allocation Bitmap consists of superframe indices to assign the E-MBS Zone. If the use of resource is changed between one superframe and next superframe, the bit “1” is set in the location the same as the superframe in the E-MBS zone allocation bitmap. Otherwise, the bit “0” is set in the E-MBS zone allocation bitmap. The length of E-MBS zone allocation bitmap is the number of bits set to “1” in the E-MBS region bitmap – 1 and the total number of E-MBS zones which the ABS belongs to is the number of bits set to “1” in the E-MBS Zone Allocation bitmap.

Figure xxx shows an example of proposed E-MBS zone allocation in mixed carrier in 10MHz system bandwidth where 1 MSI is 16superframes. In the example, let the unicast subbands and multicast subbands be 4 and 6 each. Therefore, the value of UNsubband is 4. Consider the E-MBS region bitmap is 1111111100001111 and E-MBS Zone Allocation bitmap is 00001001000. In this case superframe 0~7 and 12~15 are used for E-MBS and the number of E-MBS zones is 3.

Figure xxx – Proposed Region Bitmap and E-MBS Zone Allocation Bitmap in mixed carrier (E-MBS Region allocation bitmap: 1111111100001111; E-MBS Zone Allocation bitmap: 00001001000)

Zone_Allocation Bit-MAP: Zone_Allocation Bit-MAP consists of sub-band indices reserved for all E-MBS zones the BS belongs to. The Zone Allocation Bit-MAP in the AAI_SCD message identifies the use of the resource comprising a set of contiguous subbands in a DL AAI subframe. The Bit-MAP determines the size (S) of each E-MBS zone in number of contiguous SLRUs in frequency domain within the subframe, the index (L) of each E-MBS zone from where the allocated zone begins and the total number (Num_E-MBS_Zones) of allocated zones.

The Zone Allocation Bit-MAP is constructed using the following rules:

- If the use of the resource is changed between one sub-band and the next sub-band, the bit '1' is set in the location the same as the index of the sub-band in the Bit-MAP.

- Otherwise, the bit '0' is set in the Bit-MAP.

For a given system bandwidth with maximum number of subbands, DSACmax = 21, 10, 4 for 20, 10 and 5 MHz respectively, the size of the Zone Allocation Bit-MAP is (DSACmax -1). Let the total number of the bit '1' in the Bit-MAP be denoted as K and the index in which the bit '1' in the Bit-MAP is located be denoted as Ji, where i is the E-MBS zone index which is from 1 to Num_E-MBS_Zones. J0 is fixed to 0.

For example, consider the case where the Zone Allocation Bit-MAP field = 000010010. The system bandwidth is 10MHz and DSACmax is 10. This is illustrated pictorially below in Figure 607. In this case, ZF=0b0 because the last zone is the unicast zone. Num_E-MBS_Zones = 2, Li = {1, 6}, Si = {5, 3} from K = 2, Ji = {0, 5, 8}. Hence, the number of zones is 2. Each index of two zones is 1, 6 respectively and each size of two zones is 5, 2 respectively.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE P802.16m/D6, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” May 2010