| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup for AAI_E-MBS-CFG Message (16.9.3.1) |  |
| Date Submitted | 2010-07-13 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D6,” in response to the IEEE 802.16 Working Group Sponsor Ballot |  |
| Abstract | ASN.1 format of the AAI-E-MBS-CFG message on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Cleanup for AAI_E-MBS-CFG Message (16.9.3.1)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Instructions

This contribution reformats the current AAI_E-MBS-CFG message in [1] as the ASN.1 format.

# Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Replace the Table 980 with the following table in 16.9.3.1 of 802.16m/D6]

Table 980—AAI_E-MBS-CFG_Message format

| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | AAI_E-MBS-CFG | N.A. |
| M | E-MBS_CFG_LIFETIME (m) | 4 | Indicates the duration of E-MBS_CFG_LIFETIME for which the E-MBS configuration information of the zone do not change. AMS shall decode the E-MBS configuration message when its Nsuperframe meets the following condition. Nsuperframe modulo 16(m+1) == 0 |  |
| M | fullNumEMBSZoneID[1..8] |  |  |  |
| M | A) E-MBS_Zone_ID | 7 | The E-MBS_Zone_ID to which this E-MBS MAP applies. |  |
| M | B) E-MBS MAP Resource Index | 11 | Resource index includes location and allocation size. |  |
| M | C) E-MBS MAP ISizeOffset | 5 | Offset used to compute burst size of E-MBS MAP |  |
| O | D) E-MBS ID and FID Mappings List |  |  | Present when the serving ABS is in the zone boundary and to support inter-zone service continuity |
| M | D.1) Num_Neighbor_E-MBS_Zones | 3 | Number of neighbor E-MBS Zones |  |
| M | D.2) AAI_NBR-ADV Change Count | 3 | Indicates the value of AAI_NBR-ADV change count |  |
| O | D.3) fullNumNeighborEMBSZones[1…8] |  |  | Present when the Num_Neighbor_E-MBS_Zones > 0 |
| M | D.3.a) Neighbor_E-MBS_Zone_ID | 7 | Neighbor E-MBS Zone ID |  |
| O | D.3.b) Physical Carrier Index | 6 | Target carrier which the AMS switches or is redirected by ABS to | Present only if the neighbor E-MBS Zone ID is served on a different carrier from the previous one at the neighbor ABS |
| M | D.3.c) Num_Neighbor_ABS | 8 | Number of Neighbor ABSs |  |
| O | D.3.d) fullNumNeighborABS[1…256] |  |  | Present when the Num_Neighbor_ABS > 0 |
| M | D.3.d.1) ABSIndex | 8 | Index of Neighbor ABS |  |
| M | D.3.e) Number of E-MBS_FID Mappings (M) | 4 | Number of E-MBS ID_FID Mappings between serving E-MBS Zone and neighbor E-MBS Zone |  |
| O | D.3.f) fullNumEMBSFidMappings[1..16] |  |  | Present when the Number of E-MBS_FID Mappings > 0 |
| M | D.3.f.1) Current_E-MBS ID and FID and New_E-MBS_ID and FID | Variable (32xM) | Mapping of current E-MBS ID and FID and new E-MBS ID and FID between serving E-MBS Zone and neighbor E-MBS Zone. Based on the value of Number of E-MBS ID_FID Mappings |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE P802.16m/D6, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” May 2010