| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MIMO midamble on/off option to support inter-cell Multi-BS MIMO (Section 16.3.5 of the IEEE 802.16m/D6) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Alexei Davydov, Gregory Morozov, Huaning Niu, Yuan Zhu Intel Corporation | alexei.davydov@intel.com |
| Re: | Comments on IEEE P802.16m/D6 for IEEE 802.16 WG SB_16m Area: Chapter 16.3.5 |  |
| Abstract | The contribution proposes MIMO midamble on/off option to support inter-cell Multi-BS MIMO |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 7) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MIMO midamble on/off option to support inter-cell Multi-BS MIMO (Section 16.3.5 of the IEEE 802.16m/D6)

Alexei Davydov, Gregory Morozov

Intel Corporation

# Introduction

To support Multi-BS MIMO operation accurate channel measurements from interfering BS are required. Currently IEEE 802.16m MIMO midamble physical structure supports reuse-3 thus enabling channel measurements from three sectors of the same cell only. In this case only intra-cell Multi-BS MIMO can be supported. To extend operation of multi-BS to inter-cell scenario and minimize changes in the IEEE 802.16m specification MIMO midamble on-off option is proposed. For this option with help of DL control channel (e.g. using AAI_SCD message), the serving BS can inform the MS about MIMO midamble presence in the frame(s) within superframe transmitted by the serving cell. If the MIMO midamble of the serving cell is not present the MS can measure the channel from sectors of neighboring cell and report the inter-cell Multi-BS MIMO feedback information. It is expected that turning off MIMO midamble transmission will be infrequent to minimize the impact on the MS operating in Single-BS MIMO mode.

Figure 1. The strongest interferes distribution

To illustrate the rational of supporting inter-cell Multi-BS MIMO option a simple system level analysis of the strongest interferer distribution has been carried out. The results are presented in Figure 1. The dots in the figure show the MS positions within serving sector of the center cell. All the MS associated with the serving sector are subdivided into three groups. The first two groups of the MS that experience the strongest interference from the same cell (i.e. from the remaining two sectors of the same cell) are shown with red and green color. It is expected that these MS will operate in intra-cell Multi-BS MIMO mode and measurements from IEEE 802.16m MIMO midamble will be sufficient. The third group of the MS shown with blue dots experiences the strongest interference from the remaining sectors of the other cells. For these MS inter-cell Multi-BS MIMO mode is more suitable and requires MIMO midamble on/off option. The percentage of the strongest interferes distribution is summarized in the Table 1

Table 1. Strongest interference distribution

|  | Strongest interference from the 1st sector of the same cell | Strongest interference from the 2nd sector of the same cell | Strongest interference from the other sectors of the other cell |
| --- | --- | --- | --- |
| Relative number of MS | 25% | 25% | 50% |

It can be seen that relative amount of MS experiencing the strongest interference from other cell is equal to 50%.

# Text proposal for inclusion in the 802.16m/D7

--------------------------------------------------  Text Start #1  ------------------------------------------------

{Modify text on page 489, line 27, section 16.3.5.4.2 as follows}

16.3.5.4.2 MIMO midamble

MIMO midamble is used for PMI selection in closed loop MIMO. For OL MIMO, midamble can be used to calculate CQI. MIMO midamble shall be is transmitted every frame on the second DL AAI subframe. The midamble signal occupies the first OFDMA symbol in a DL type-1 or type-2 AAI subframe. For the type-1 AAI subframe case, the remaining 5 consecutive symbols form a type-3 AAI subframe. For the type-2 AAI subframe case, the remaining 6 consecutive symbols form a type-1 AAI subframe. To assist AMS downlink channel measurements from neighboring ABS an empty OFDM symbol can be transmitted on the place of MIMO midamble in the second and fourth frames of the superframe. In this case MIMO midamble transmission is controlled by AAI_SCD message and applicable when ABS is configured in Multi-BS MIMO mode only.

--------------------------------------------------  Text End #1 ------------------------------------------------

--------------------------------------------------  Text Start #2  ------------------------------------------------

{Modify table 716 on page 157, line 29, section 16.2.3.20 as follows}

Table 716—AAI_SCD Message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| ... |  |  |  |  |
| M | Trigger definitions | Variable | Refer to Table 774- | N.A. |
| O | mimoMidambleFrameBitmap | 2 | Ones in the bitmap indicates presence of MIMO midamble in the second and fourth frames of the superframe (MSB corresponds to the second frame) | N.A. |
| ... |  |  |  |  |

--------------------------------------------------  Text End #2 ------------------------------------------------