| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Fix to CTC Interleaver Parameters to Enable Efficient Parallel Decoder Designs (16.3.11.1.5.1) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Brian F. Johnson, Slobodan Jovanovic, Steven Tretter , Cory Modlin, Alek Purkovic, Arnon Friedmann, Eko Onggosanusi Texas Instruments Mark Cudak Motorola | Voice: + 1 240 821 4324 Email: bfjohnson@ti.com, eko@ti.com Email: Mark.Cudak@motorola.com |
| Re: | Comments and Change Request on 802.16m Draft Amendment, IEEE P802.16m/D6 Section: 16.3.11.1.5.1 Convolutional Turbo Codes in response to the Sponsor Ballot circulation. |  |
| Abstract | The current CTC interleaver parameters do not generally support parallelism in the turbo decoder implementations for block sizes below 1024. This will lead to significantly lower throughput CTC decoding for the smaller block sizes or significantly increased complexity and cost to implement turbo decoding to deliver high throughputs, especially in the uplink base station decoder when there can be a large number of small blocks to decode with tight latency budget. This contribution proposes small changes to the interleaver parameter table maintaining the existing well-designed block sizes and performance while also improving the design so that all block sizes below 1024 have at least parallelism order 2 and all except 2 block sizes have parallelism order 4. This will significantly lower the cost and complexity to implement high throughput turbo decoding for 802.16m and facilitate efficient multi-standard implementations. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D7 Draft as a remedy for Disapprove comments in the Sponsor Ballot circulation. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Fix to CTC Interleaver Parameters to Enable Efficient Parallel Decoder Designs (16.3.11.1.5.1)

Brian F. Johnson, Slobodan Jovanovic, Steven Tretter , Cory Modlin, Alek Purkovic, Arnon Friedmann, Eko Onggosanusi
Texas Instruments

Mark Cudak
Motorola

- Introduction

The current CTC interleaver parameters do not generally support parallelism in the turbo decoder implementations for block sizes below 1024. Of the 25 block sizes below 1024, there are 10 that do not support parallelism order 2.  This will lead to significantly lower throughput CTC decoding for the smaller block sizes or significantly increased complexity and cost to implement turbo decoding to deliver high throughputs, especially in the uplink base station decoder when there can be a large number of small blocks to decode with tight latency budget. It is noted that the 802.16-2009 CTC code supports at least parallelism order 2 for all block sizes except one. It is also noted that in other air interface standards that may be considered for IMT-Advanced such as [5] the CTC encoder interleaver supports parallelism orders 2, 4, and 8, for all block sizes, and parallelism order 16 for all block sizes above 512. This allows very efficient implementation of very high speed turbo decoding.  In this respect the current design compares poorly to the existing 802.16-2009 standard and to [5].

It cannot be considered good for the evolution of the 802.16 standard or the evolution of the 4G technology in general to introduce an 802.16m amendment with a CTC specification that has poor support for efficient parallel implementation of the turbo decoder.  This contribution proposes small changes to the interleaver parameter table maintaining the existing well-designed block sizes and performance while also improving the design so that all block sizes below 1024 have at least parallelism order 2 and all except 2 block sizes have parallelism order 4. This will significantly lower the cost and complexity to implement high throughput turbo decoding for 802.16m and facilitate efficient multi-standard implementations.

- Previous Work

The convolutional turbo code (CTC) used for 802.16-2009 FEC scheme with modifications is adopted as a mandatory channel coding scheme for data channels in the current IEEE 802.16m draft amendment [1]. In [2] there was an attempt to improve the design compared to 802.16-2009 with respect to padding overhead. A new set of block sizes along with the appropriate inner interleaver parameters and subblock interleaver parameters is proposed.  The result is a significant reduction in padding overhead due to the finer granularity of block sizes. However the design does not consider the contention-free property of the interleaver or the implementation of efficient, parallel structures for high throughput turbo decoding.

In [3] it is noted that the almost regular permutation (ARP) interleaver [4] is used as the CTC interleaver. It is also noted in [3] that the ARP interleaver is a contention free interleaver that can allow efficient implementation of parallel structures in the CTC to achieve very high throughput decoding. In [3] in order to satisfy the required data throughput, the CTC block sizes, interleaver parameters, and subblock interleaver parameters are adjusted to achieve parallelism order 4 for block sizes over 1024.  However, no consideration was made for the parallelism order of the block sizes below 1024.

- Lack of Parallelism with Current Interleaver Parameters.

Table 1 depicts the CTC encoder input sizes (NFB) and their possible parallelism orders between 2 and 8.  The red entries that say “No” indicate that there is a contention which prevents support for parallelism at that order.

It is observed that 10 block sizes do not support parallelism order 2 as a minimum which is highly concerning.  Another highly concerning observation is that many block sizes do not support any order of parallelism between 2 and 8 including block sizes as large as 568.

Table 1 CTC encoder block sizes and the supported parallelism orders up to order 8. “Yes” means supported.

This lack of parallelism has serious implications for the complexity of the turbo decoder implementation.  When implementing high throughput turbo decoding, especially for the base station, the typical requirement is to support as close to the theoretical peak speeds as possible for all possible combinations of the block sizes. There are typically tight latency requirements for completion of all the turbo decoding on a per frame basis.  The turbo decoding typically represents the single highest load in terms of the complexity of number of operations per second of any processing block in the receiver. Even with a very efficient, high performance design, the turbo decoder processing is also typically the largest contributor to the receiver latency.  The contention free interleaver property allows for a simple, efficient decoder design using parallel MAP decoders running at full speed in the decoder to achieve the high throughput decoding.  The current design could reduce the throughput, or conversely increase the complexity of the design by as much as 2x, 4x, or 8x. The worst case scenario would be if the complete lack of parallelism makes it very difficult to achieve the peak throughput per base station of the 802.16m standard.

As mentioned previously, 802.16-2009 supports contention-free parallelism order 2 for all block sizes except one.  The LTE standard supports contention free parallelism order 2, 4, 8, and 16 for block sizes above 512, and parallelism order 2, 4, and 8 for all block sizes below 512.  The current design for 802.16m compares poorly in this respect with 802.16-2009 and with LTE. This will also make it extremely difficult to achieve a low complexity multi-standard implementation.

- Proposed Solution Through Modified Interleaver Parameters.

In this contribution, we propose a simple solution that maintains the current block sizes and almost all of the current interleaver parameter settings without compromising performance.   We propose to change the CTC encoder interleaver parameters for the 10 block sizes that currently do not support parallelism order 2.  The list of the block sizes and the new interleaver parameters are shown in table 2.

Table 2 Proposed Changes to the CTC encoder interleaver parameters to support parallelism order 2 for all block sizes

As can be seen from Table 2, the proposed interleaver parameters provide support for parallelism order 2 for all block sizes and also provide support for parallelism 4 for all except 2 block sizes.  The table also summarizes the Frame Error Rate performance comparison.  The details of the simulation and performance comparison are in the next section.  The conclusion is that the new interleaver parameters show nearly identical performance as the current parameters.  The last section of this document describes the proposed text changes required to adopt the new interleaver parameters.

- Simulation results

The simulation results show the Block Error Rate (BLER), also referred to as Frame Error Rate(FER) performance comparison between the current interleaver parameter settings (P0, P1, P2, P3) and the proposed interleaver parameter settings. Figures 1 through 10 show the results. The Max-log-MAP algorithm with 8 iterations is used with the rate 1/3 CTC decoder in the AWGN channel environment.  Simulation results are reported as FER vs SNR where for this simulation setup SNR is equivalent to (2/3)(Eb/N0).  The conclusions of the simulation results are that the new interleaver parameters show nearly identical performance as the current parameters using the same performance analysis methods as used in [2], and [3].  This means that the CTC interleaver parameters are well-designed and that the proposed design can support parallelism order 2 for all block sizes, and parallelism 4 for all except 2 block sizes without any performance loss.

Figure 1 FER performance comparison for NFB = K = 88.  
Current interleaver parameters are Red O, versus proposed parameters BLUE *.

Figure 2 FER performance comparison for NFB = K = 104.
Current interleaver parameters are Red o, versus proposed parameters BLUE *.

Figure 3 FER performance comparison for NFB = K = 120.
Current interleaver parameters are Red o, versus proposed parameters BLUE *.

Figure 4 FER performance comparison for NFB = K = 136.
Current interleaver parameters are Red o, versus proposed parameters BLUE *.

Figure 5 FER performance comparison for NFB = K = 152.
Current interleaver parameters are Red o, versus proposed parameters BLUE *.

Figure 6 FER performance comparison for NFB = K = 200.
Current interleaver parameters are Red o, versus proposed parameters BLUE *.

Figure 7 FER performance comparison for NFB = K = 216.
Current interleaver parameters are Red o, versus proposed parameters BLUE *.

Figure 8 FER performance comparison for NFB = K = 248.
Current interleaver parameters are Red o, versus proposed parameters BLUE *.

Figure 9 FER performance comparison for NFB = K = 456.
Current interleaver parameters are Red o, versus proposed parameters BLUE *.

Figure 10 FER performance comparison for NFB = K = 568.
Current interleaver parameters are Red o, versus proposed parameters BLUE *.

- References

[1] IEEE P802.16m/D6, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems,” May 2010.

[2] IEEE C802.16m-09/0313, “IEEE 802.16m CTC block sizes and interleaver,” January 2009.

[3] IEEE C802.16m-09/1726r1, “Proposed Changes to CTC Encoder Input Sizes,” January 2009.

[4] C. Berrou, Y. Saouter, C. Douillard, S. Kerouédan, and M. Jézéquel, “Designing good permutations for turbo codes: towards a single model,” IEEE International Conference on Communications, pp.341-345, June 2004.

[5] 3GPP TS 36.212 v9.2.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding,” June 2010.

- Proposed changes for 802.16m draft amendment

The proposed text changes are confined to a few entries in Table 962 in Section 16.3.11.1.5.1

---------------------------------------------------  Text Start  ---------------------------------------------------

<Modify Table 962 as follows >

- Table 962─Interleaver Parameters

| Index | NFB (bit) | P0 | P1 | P2 | P3 | Index | NFB (bit) | P0 | P1 | P2 | P3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 48 | 5 | 0 | 0 | 0 | 21 | 568 | 19 13 | 102 34 | 140 0 | 226 34 |
| 2 | 64 | 11 | 12 | 0 | 12 | 22 | 640 | 23 | 84 | 296 | 236 |
| 3 | 72 | 11 | 18 | 0 | 18 | 23 | 720 | 23 | 130 | 156 | 238 |
| 4 | 80 | 7 | 4 | 32 | 36 | 24 | 800 | 23 | 150 | 216 | 150 |
| 5 | 88 | 13 | 36 22 | 36 0 | 32 22 | 25 | 912 | 29 | 14 | 264 | 94 |
| 6 | 96 | 13 | 24 | 0 | 24 | 26 | 1024 | 29 | 320 | 236 | 324 |
| 7 | 104 | 7 19 | 4 26 | 8 0 | 48 26 | 27 | 1152 | 31 | 534 | 372 | 246 |
| 8 | 120 | 11 13 | 30 | 0 | 34 30 | 28 | 1312 | 31 | 214 | 160 | 506 |
| 9 | 136 | 13 19 | 58 34 | 4 0 | 58 34 | 29 | 1440 | 41 | 288 | 556 | 672 |
| 10 | 152 | 11 17 | 38 | 12 0 | 74 38 | 30 | 1632 | 29 | 334 | 564 | 66 |
| 11 | 176 | 17 | 52 | 68 | 32 | 31 | 1856 | 47 | 576 | 212 | 728 |
| 12 | 200 | 11 13 | 76 50 | 0 | 24 50 | 32 | 2112 | 43 | 96 | 720 | 980 |
| 13 | 216 | 11 17 | 54 | 56 0 | 2 54 | 33 | 2368 | 47 | 228 | 440 | 724 |
| 14 | 248 | 13 | 6 62 | 84 0 | 46 62 | 34 | 2624 | 47 | 378 | 1092 | 1250 |
| 15 | 288 | 17 | 74 | 72 | 2 | 35 | 2944 | 41 | 338 | 660 | 646 |
| 16 | 320 | 17 | 84 | 108 | 132 | 36 | 3328 | 37 | 258 | 28 | 1522 |
| 17 | 352 | 17 | 106 | 56 | 50 | 37 | 3776 | 53 | 772 | 256 | 408 |
| 18 | 400 | 19 | 142 | 0 | 142 | 38 | 4224 | 59 | 14 | 668 | 1474 |
| 19 | 456 | 17 13 | 184 194 | 0 | 48 194 | 39 | 4800 | 53 | 66 | 24 | 2 |
| 20 | 512 | 19 | 64 | 52 | 124 |  |  |  |  |  |  |

---------------------------------------------------  Text End  ---------------------------------------------------