| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARS frame structure clean up |  |
| Date Submitted | 2010-07-14 |  |
| Source(s) | Alexander Maltsev, Alexey Khoryaev, Andrey Chervyakov, Mikhail Shilov Intel Corporation Youngbin Chang, Jaeweon Cho Samsung Electronics Jinyoung Chun, Jin Sam Kwak LG Electronics | E-mails: alexander.maltsev@intel,com alexey.khoryaev@intel.com E-mail: yb.chang@samsung.com E-mail: jiny.chun@lge.com |
| Re: | Comments on IEEE P802.16m/D6 for IEEE 802.16 Working Group Sponsor Ballot Topic: Relay (Section 16.6.3.3) |  |
| Abstract | This contribution proposes technical and editorial changes |  |
| Purpose | To be discussed and adopted by TGm for theP802.16m/D7. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARS frame structure clean up and clarification on AAI Relay zone durations

Alexander Maltsev, Alexey Khoryaev, Andrey Chervyakov
Mikhail Shilov

Intel Corporation

Youngbin Chang, Jaeweon Cho

Samsung Electronics

Jinyoung Chun, Jin Sam Kwak

LG Electronics

- Introduction

This contribution provides technical and editorial changes on ARS basic frame structure. The proposed text is developed so, that it is compliant to the IEEE 802.16m SRD [2] and the IEEE 802.16m SDD [3], and it follows the style and format guidelines in [4].

- References

- IEEE P802.16Rev2/D9a, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,” March 2009.

- IEEE 802.16m-07/002r8, “802.16m System Requirements”

- IEEE 802.16m-08/0034r2, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- IEEE P802.16m/D6, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Proposed Text

Instruction to editor:

Black text: existing D6 text

Red and strike out text: to be deleted

Blue and underline text: to be added

=========================Start of Proposed Text 1 ==============================

[Note to Editor: Modify the following text in section 16.6.3.1 Basic frame structure supporting ARS (page 827, line 2) as follows]

The zone configuration of AAI Access zone and AAI Relay zones is informed transmitted to the ARS by the ABS in AAI_ARS-CONFIG-CMD RS_config-CMD message.

========================== End of Proposed Text 1 =============================

=========================Start of Proposed Text 2 ==============================

[Note to Editor: Delete the following text in section 16.6.3.1 Basic frame structure supporting ARS (page 827, line 20)]

The position of AAI DL/UL relay zone is indicated for AMS using AAI_System Configuration Descriptor message when AAI_Relay_zone_AMS_allocation_indicator is equal to 0.

========================== End of Proposed Text 2 =============================

=========================Start of Proposed Text 3 ==============================

[Note to Editor: Modify the following text in section 16.6.3.1 Basic frame structure supporting ARS (page 827, line 31) as follows]

The duration and the positions of R-TTI and R-RTI are defined in FDD (see 16.6.3.2.1) and TDD (see 16.6.3.2.2), respectively. The each R-TTI and R-RTI shall be equal or less than one OFDMA symbol.

The calculation of duration and the positions of R-TTI and R-RTI at the ARS frame in FDD and TDD modes are provided in section 16.6.3.2.1 and 16.6.3.2.2 respectively.

========================== End of Proposed Text 3 =============================

========================Start of Proposed Text 4  ==============================

[Note to Editor: The MAC message RS_config-CMD was  renamed to AAI_ARS-CONFIG-CMD. Please replace the RS_config-CMD with AAI_ARS-CONFIG-CMD in section 16.6.3 and it subsections]

RS_config-CMD  AAI_ARS-CONFIG-CMD

========================== End of Proposed Text 4 =============================