| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of Change Indicator in E-MBS MAP (16.3.6.2.3) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Kaushik Josiam, Hai Wang, JaeJeong Shim, Taeyoung Kim Samsung Electronics | E-mail: k.josiam@samsung.com, wang.hai@samsung.com;Brian.shim@samsung.com; ty33.kim@samsung.com |
| Re: | Comments on IEEE P802.16m/D6 for IEEE 802.16 WG Sponsor Ballot SB_16m |  |
| Abstract | This contribution proposes to make the changes to the fields in E-MBS MAP. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of Change Indicator in E-MBS MAP (16.3.6.2.3)

Kaushik Josiam, Hai Wang, JaeJeong Shim and Taeyoung Kim

Samsung Electronics

- Introduction

E-MBS MAP should not be used to indicate changes in the E-MBS_CFG message. The MAP is a message whose identical copies carried at all ABSs in the E-MBS zone while the E-MBS_CFG message is ABS specific message that is different at every ABS in the zone.  The E-MBS MAP currently carries the AAI_E-MBS-CFG Change Indicator which this contribution proposes to remove.  The AAI_E-MBS-CFG message contains a field called CFG Lifetime which indicates when the configuration message is likely to change.

- Text proposal for the 802.16m DRAFT amendment

---------------------------------------------------------Start of the Proposed Text ---------------------------------------------------------

[Remedy: Modify “16.3.6.2.3 Enhanced Multicast Broadcast Service MAP” section in P 802.16m/D6 as follows:]

16.3.6.2.3 Enhanced Multicast Broadcast Service MAP (E-MBS MAP)

Table 837: E-MBS MAP Structure

| Syntax | Size (bits) | Description/Notes |
| --- | --- | --- |
| E-MBS-MAP () { | - | - |
| AAI_E-MBS-CFG Change Indication | 1 | AAI_E-MBS-CFG Change Indication indicates any change in the parameters of AAI_E-MBS-CFG MAC Control Message at its next transmission instance. If it is set (AAI_E-MBS-CFG Change Indication = 1), parameters in the AAI_E-MBS-CFG message will change and AMS shall decode the next AAI_E-MBS-CFG message. Otherwise, the AAI_E-MBS-CFG message does not change and the AMS resets the Allocation Lifetime as defined in the previous AAI_E-MBS-CFG message and may not decode the next transmission. |
| S-SFH SP2 update indicator | 1 | Indicates whether the changed S-SFH SP2 will be transmitted in this MSI. If this field is set to 1, the time offset which the changed S-SFH SP2 is transmitted will be included in this E-MBS MAP. |
| If (S-SFH SP2 update indicator ==1) { |  |  |
| S-SFH SP2 transmission time offset | Variable | Indicates the superframe offset which the changed S-SFH SP2 is transmitted. The size of this field depends on MSI If MSI == 0b00: 1 bits If MSI == 0b01: 2 bits If MSI == 0b10: 3 bits If MSI == 0b11: 4 bits |
| } |  |  |
| E-MBS_DATA_IE() | - | - |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |

-----------------------------------------------------------End of the Proposed Text   --------------------------------------------------------