| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on periodicity of MIMO-Midamble transmission in AAI DL Relay zone |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Alexander Maltsev, Alexey Khoryaev, Andrey Chervyakov, Mikhail Shilov Intel Corporation | E-mails: alexander.maltsev@intel,com alexey.khoryaev@intel.com |
| Re: | Comments on IEEE P802.16m/D6 for IEEE 802.16 Working Group Sponsor Ballot Topic: Relay (Section 16.6.3.3) |  |
| Abstract | This contribution proposes to explicitly define 20 ms periodicity of additional MIMO Midamble transmission on Relay link when ARS is deployed |  |
| Purpose | To be discussed and adopted by TGm for theP802.16m/D7. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on MIMO-Midamble transmission in AAI DL Relay zone

Alexander Maltsev, Alexey Khoryaev, Andrey Chervyakov
Mikhail Shilov

Intel Corporation

- Introduction

This contribution provides clarification on transmission of MIMO midamble on relay link when ARSs are deployed. The proposed text is developed so, that it is compliant to the IEEE 802.16m SRD [2] and the IEEE 802.16m SDD [3], and it follows the style and format guidelines in [4].

In Draft 6 of the IEEE 802.16m specification the additional MIMO midamble may be transmitted on relay link when ARS is attached to ABS. It is assumed that this additional MIMO midamble is transmitted in the first frame of superframe and it period can be configured to be 0 (switched off), 1, 2, or 4 superframes. In the current text of the IEEE 802.16m D6 [5], the information about MIMO midamble periodicity is transmitted in AAI_ARS-CONFIG-CMD only. This MAC control message is ARS specific can be transmitted very infrequently. AMSs are not supposed to read the content of AAI_ARS-CONFIG-CMD. In this case the AMSs served by ABS will not be aware about MIMO-Midamble periodicity, that may lead to failure of data and control channel reception in subframe where MIMO-Midamble is transmitted since it transmission may require change of subframe type. In order to inform AMSs about the transmission of additional MIMO-Midamble on Relay link the signaling of MIMO-Midamble periodicity will need to be inserted to superframe header which IEs have limitation on information size.

- Proposal Description

To resolve the mentioned issue it is proposed to explicitly define that when ARS is deployed the ABS shall transmit MIMO-Midamble with the periodicity equal to 20 ms (i.e. once per superframe). In this case when AMS becomes aware about presence of ARS in the cell and decode frame configuration index from SFH message the position of MIMO-Midamble can be determined for TDD and FDD modes using the procedure defined in section 16.6.3.3.1 of the Draft 6.

Note that non-transmission of MIMO-Midamble on relay link will not allow accurate measurement of MIMO channel that is necessary to ensure high spectral efficiency of ABS-ARS link. In TDD mode the MIMO channel can be measured only relying on reciprocity that requires accurate calibration of RF chains which is not a general assumption. In FDD mode the MIMO channel can not be measured without transmission of MIMO-Midamble since the channel is not reciprocal. From the other side the MIMO-Midamble transmission with the periodicity equal to 40 and 80 ms will always have less accurate MIMO channel measurements comparing to 20 ms periodicity.

- References

- IEEE P802.16Rev2/D9a, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,” March 2009.

- IEEE 802.16m-07/002r8, “802.16m System Requirements”

- IEEE 802.16m-08/0034r2, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- IEEE P802.16m/D6, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Proposed Text

Instruction to editor:

Black text: existing D6 text

Red and strike out text: to be deleted

Blue and underline text: to be added

=========================Start of Proposed Text 1 ==============================

[Note to Editor: Change the following text in section 16.6.3.3.1 MIMO Midamble on page 832 (line 1) as follows]

The ABS may transmit an additional MIMO midamble in the AAI DL Relay zone of the first frame of each superframe when the ARS starts operation phase during network entry. The periodicity of MIMO midamble is provided in AAI_ARS-CONFIG-CMD.

When ABS has at least one attached subordinate ARS the ABS shall transmit an additional MIMO midamble in the 16m DL Relay zone of the first frame of each superframe to enable MIMO channel measurements on ABS-ARS link.

========================== End of Proposed Text 1 =============================

=========================Start of Proposed Text 2 ==============================

[Note to Editor: Delete the row in Table 763 AAI_ARS-CONFIG-CMD message format on page 224]

Table 763—AAI_ARS-CONFIG-CMD message format

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | 16m_Relay_zone_AMS_allocation_indicator |  | 1 | The indicator which signalizes if the ABS may allocate resources to the AMS in the 16m DL Relay zone and 16m UL Relay zone. It has 1 bit to represent the values {0, 1}. If it is set to 1 then the AMSs are allocated in Relay zones | N.A. |
| M | MIMO Midamble Periodicity |  | 2 | MIMO Midamble periodicity in 16m DL Relay Zone. It has 2 two bits to represent the values {0, 1, 2, 4}. Unit of periodicity is a superframe. | N.A. |
| O | 16m_DL_Relay_zone_Cell_Specific_Parameters | R_DCASSB0 | 5/4/3 | See 16.6.3.3.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits | Present if 16m_Relay_zone_AMS_allocation_indicator == 0 |

========================== End of Proposed Text 2 =============================