| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Control signaling for support of relay operation |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Alexander Maltsev, Alexey Khoryaev, Andrey Chervyakov, Mikhail Shilov Intel Corporation | E-mails: alexander.maltsev@intel,com alexey.khoryaev@intel.com |
| Re: | Comments on IEEE P802.16m/D6 for IEEE 802.16 Working Group Sponsor Ballot Topic: Relay (Section 16.6.3.3) |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for theP802.16m/D7. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Control signaling for support of relay operation

Alexander Maltsev, Alexey Khoryaev, Andrey Chervyakov
Mikhail Shilov

Intel Corporation

- Introduction

This contribution introduces 2 bits in S-SFH SP2 IE required for support of ARS operation. The proposed text is developed so, that it is compliant to the IEEE 802.16m SRD [2] and the IEEE 802.16m SDD [3], and it follows the style and format guidelines defined in [4].

- Description

All AMSs (served by ABS and ARS) have to be informed about the presence of ARS in the cell and shall know duration of the AAI Access and AAI Relay zone when ARS is attached to ABS. This information is required since:

- ABS transmits additional MIMO-Midamble on relay link that results in change of the subframe type and therefore impacts frame structure configuration.

- AMS served by ARS shall know ARS frame structure parameters to start communication with ARS.

- Different cell-specific permutations can be configured in AAI Access and AAI Relay zones (if ABS decides to use Relay Zone for communication with ARS only).

- Interference environment in Access and Relay Zone is different that may be taken by AMS into account for CQI report.

- References

- IEEE P802.16Rev2/D9a, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,” March 2009.

- IEEE 802.16m-07/002r8, “802.16m System Requirements”

- IEEE 802.16m-08/0034r2, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- IEEE P802.16m/D6, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Proposed Text

Instruction to editor:

Black text: existing D6 text

Red and strike out text: to be deleted

Blue and underline text: to be added

=========================Start of Proposed Text 1 ==============================

[Note to Editor: Add the following underlined blue fields into Table 841 S-SFH SP2 IE format section 16.3.6.5.1.2 S-SFH IE (page 547, line 2)]

| AMS Transmit Power Limitation Level | 5 | Unsigned 5-bit integer. Specifies the maximum allowed AMS transmit power. Values indicate power levels in 1 dB steps starting from 0 dBm |
| --- | --- | --- |
| EIRxPIR,min | 5 | Unsigned integer from -133 to -102 in units of 1dBm, where 0b00000=-133 dBm and 0b11111=-102 dBm. |
| ARS Indication | 1 | If ARS indication bit = 0b1, the ABS has attached ARSs, else if 0b0 cell does not have ARSs |
| The value of this field depends on type of the station (ABS or ARS) that transmits SP2 ABS frame: Relay Specific Permutation ARS frame: ARS R-TTI value | 1 | If Relay Specific Permutation bit = 0b1 then cell specific permutation is applied in AAI Relay Zone and ABS does not communicate with AMS in AAI Relay zone, else the cell specific permutations of AAI Access zone and AAI Relay zone are the same and ABS may communicate in AAI Relay zone with ARS and AMSs If ARS R-TTI value bit = 1b1 then R-TTI duration is equal to 1 OFDMA symbol else the R-TTI duration is zero. |
| Reserved |  |  |
| } |  |  |

========================== End of Proposed Text 1 =============================