| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Radio specifications for IEEE P802.16m |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Reza Arefi Intel Corporation* Roger Marks WiMAX Forum* | reza.arefi#intel.com r.b.marks@ieee.org *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE Sponsor Ballot of P802.16m/D6 |  |
| Abstract | This contribution proposes the addition of two annexes to the IEEE P802.16m draft to contain radio specifications. The content is drawn from IEEE’s IMT-Advanced candidate radio interface technology proposal. |  |
| Purpose | This is proposed as the basis of a P802.16m/D6 Sponsor Ballot comment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Radio specifications for IEEE P802.16m

Reza Arefi, Intel Corporation

Roger Marks, WiMAX Forum

# 1. Introduction: Radio Specifications in IMT

Radio specifications, including information on supported frequency bands and associated unwanted emissions, form an important part of the ITU’s detailed IMT specifications, including IMT-Advanced radio interface technology specifications. Such information is essential to regulatory agencies around the world. It is also a key element of ITU-R provisions enabling global circulation of terminals.

# 2. Comparison to IMT-2000

Frequency bands and their associated unwanted emission characteristics are an integral part of Recommendation ITU-R M.1457, the detailed specifications of IMT-2000 radio interfaces. Both IEEE and the WiMAX Forum support the IMT-2000 OFDMA TDD WMAN specification (based on IEEE 802.16) in Rec. M.1457-9. The WiMAX Forum Mobile Radio Specification is an element of the “Global Core Specifications” (GCS) of IMT-2000 OFDMA TDD WMAN in Rec. M.1457-9; it has also been “transposed” as a WiMAX Forum specification. The WiMAX Forum Mobile Radio Specification addresses unwanted emissions and other regulatory information.

In the case of the IEEE’s IMT-Advanced candidate in IMT-ADV/4, the IEEE is the sole proponent of the radio interface technology and expects to submit IEEE Std 802.16m as the IMT-Advanced GCS. According to the planned process, external Standard Development Organizations (SDOs), could transpose of the content of the GCS into their own standards, with allowance for regional variations but maintaining “close consistency” with the GCS. Radio specification information included in IEEE Std 802.16m and then into the IMT-Advanced GCS will provide a basis for external SDOs, such as ARIB, TTA, and the WiMAX Forum, to transpose their own radio specifications.

# 3. Detailed radio specifications

The IEEE 802.16 Working Group has already taken a position regarding the detailed radio specifications appropriate to its IMT-Advanced proposal. This information was included in the IEEE radio interface technology (RIT) proposal IMT-ADV/4. Namely, in that contribution:

*Subclause 4.2.3.2.8.3 answers the question “What are the frequency bands supported by the RIT?” with a text answer as well as Table 6-10 (“Some of supported frequency bands”).

*Subclause 4.2.3.2.23.5 answers the question “Does the proposal satisfy a specific spectrum mask?” with a reference to Table 6-10 and with a large set of additional tables (Tables 6-17 through 6-76).

# 4. Proposal

This contribution proposes the addition of two new normative annexes to the P802.16m draft based on the radio specifications in Subclauses 4.2.3.2.8.3 and 4.2.3.2.23.5 of IEEE’s contribution IMT-ADV/4 (“Submission of a Candidate IMT-Advanced RIT based on IEEE 802.16”). The specific content proposal is contained below as Appendices 1 and 2.

Appendix 1

[Authors’ Note: Tables numbering and other references are drawn from IMT-ADV/4 and need to be adjusted as appropriate to a new annex in P802.16m.]

Annex T

(Normative)

Supported frequency bands

The standard supports deployment in all bands identified for IMT in ITU-R Radio Regulations. In addition, the standard supports non-IMT bands below 6 GHz allocated to the Fixed Service and/or Mobile Service.

See below for more information on some of the bands in which systems based on the standard can be deployed.

Table 6-10: Some of supported frequency bands

| Band Class | UL AMS Transmit Frequency (MHz) | DL AMS Receive Frequency (MHz) | Duplex Mode |
| --- | --- | --- | --- |
| 1 | 2300-2400 | 2300-2400 | TDD |
| 2 | 2305-2320, 2345-2360 | 2305-2320, 2345-2360 | TDD |
|  | 2345-2360 | 2305-2320 | FDD |
| 3 | 2496-2690 | 2496-2690 | TDD |
|  | 2496-2572 | 2614-2690 | FDD |
| 4 | 3300-3400 | 3300-3400 | TDD |
| 5L | 3400-3600 | 3400-3600 | TDD |
|  | 3400-3500 | 3500-3600 | FDD |
| 5H | 3600-3800 | 3600-3800 | TDD |
| 6 | 1710-1770 | 2110-2170 | FDD |
|  | 1920-1980 | 2110-2170 | FDD |
|  | 1710-1755 | 2110-2155 | FDD |
|  | 1710-1785 | 1805-1880 | FDD |
|  | 1850-1910 | 1930-1990 | FDD |
|  | 1710-1785, 1920-1980 | 1805-1880, 2110-2170 | FDD |
|  | 1850-1910, 1710-1770 | 1930-1990, 2110-2170 | FDD |
| 7 | 698-862 | 698-862 | TDD |
|  | 776-787 | 746-757 | FDD |
|  | 788-793, 793-798 | 758-763, 763-768 | FDD |
|  | 788-798 | 758-768 | FDD |
|  | 698-862 | 698-862 | TDD/FDD |
|  | 824-849 | 869-894 | FDD |
|  | 880-915 | 925-960 | FDD |
|  | 698-716, 776-793 | 728-746, 746-763 | FDD |
| 8 | 1785-1805, 1880-1920, 1910-193, 2010-2025, 1900-1920 | 1785-1805, 1880-1920, 1910-193, 2010-2025, 1900-1920 | TDD |
| 9 | 450-470 | 450-470 | TDD |
|  | 450.0-457.5 | 462.5-470.0 | FDD |

Appendix 2

[Authors’ Note: Tables numbering and other references are drawn from IMT-ADV/4 and need to be adjusted as appropriate to a new annex in P802.16m.]

Annex U

(Normative)

Unwanted emissions for WirelessMAN-Advanced Air Interface

AMS spectral masks:

Unless otherwise specified for specific bands, the spectrum masks of the following three tables (6-17, 6-18, 6-19) are applicable to AMSs.

Table 6-17: AMS Spectrum Emission Mask for 5 MHz Bandwidth

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/integration BW) at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to < 3.5 | 50 | -13 |
| 2 | 3.5 to  12.5 | 1000 | -13 |

- Table 6-18: AMS Spectrum Emission Mask for 10 MHz Bandwidth

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to < 6 | 100 | -13 |
| 2 | 6 to  25 | 1000 | -13 |

- Table 6-19: AMS Spectrum Emission Mask for 20 MHz Bandwidth

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <11 | 200 | -13 |
| 2 | 11 to  50 | 1000 | -13 |

AMS Band Class 1:

Table 6-20: AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 1

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/integration BW) at the antenna port. |
| --- | --- | --- | --- |
| 1 | 2.5 to < 3.5 | 50 | -13 |
| 2 | 3.5 to < 7.5 | 1000 | -13 |
| 3 | 7.5 to < 8 | 500 | -16 |
| 4 | 8 to < 10.4 | 1000 | -25 |
| 5 | 10.4 to ≤ 12.5 | 1000 | -25 |

Notes:

- f is defined as the frequency offset in MHz from the center frequency of a 5 MHz channel.

- Integration Bandwidth refers to the frequency range over which the emission power is integrated.

Table 6-20-1: AMS Spectrum Emission Mask for 8.75 MHz Bandwidth Band Class 1, Ptx ≤23 dBm

| Segment Number | ffset from channel center (MHz) | Integration Bandwidth (KHz) | Allowed Emission Level as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 4.77 to  9.27 | 100 | -[26+7×{(Δf¦-4.77 MHz)/4.5 MHz }] dB |
| 2 | 9.27 to 13.23 | 100 | -[33+4×{(¦Δf¦-9.27 MHz)/3.96 MHz }] dB |
| 3 | 13.23 to  17.73 | 100 | -[37+2×{(Δf¦-13.23 MHz)/4.5 MHz }] dB |
| 4 | 17.73 to  22.5 | 100 | -39 dB |

Notes:

f is defined as the frequency offset in MHz from the center frequency of the 8.75 MHz channel.

2.  PTx is the measured power in dBm into the antenna.

3. Integration Bandwidth refers to the frequency range over which the emission power is integrated.

Table 6-20-2: AMS Spectrum Emission Mask for 8.75 MHz Bandwidth Band Class 1, Ptx >23 dBm

| Segment Number | ffset from channel center (MHz) | Integration Bandwidth (KHz) | Allowed Emission Level as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 4.77 to  9.27 | 100 | -[{(PTx-23)+26}+7×{(Δf-4.77 MHz)/4.5 MHz }] dB |
| 2 | 9.27 to 13.23 | 100 | -[{(PTx-23)+33}+4×{(¦Δf¦-9.27 MHz)/3.96 MHz }] dB |
| 3 | 13.23 to  17.73 | 100 | -[{(PTx-23)+37}+2×{(¦Δf¦-13.23 MHz)/4.5 MHz }] dB |
| 4 | 17.73 to  22.5 | 100 | -[(PTx-23)+39] dB |

Notes:

f is defined as the frequency offset in MHz from the center frequency of the 8.75 MHz channel.

2. PTx is the measured power in dBm into the antenna.

3. Integration Bandwidth refers to the frequency range over which the emission power is integrated.

Table 6-21: AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 1

| Segment Number | ffset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to <6 | 100 | -13 |
| 2 | 6 to <10 | 1000 | -13 |
| 3 | 10 to <11 | 1000 | -13-12(f -10) |
| 4 | 11 to <15 | 1000 | -25 |
| 5 | 15 to <20 | 1000 | -25 |
| 6 | 20 to ≤ 25 | 1000 | -25 |

Notes:

- f is defined as the frequency offset in MHz from the center frequency of a 10MHz channel.

Integration Bandwidth refers to the frequency range over which the emission power is integrated.

AMS Band Class 3 – TDD

Table 6-22: AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/integration BW) at the antenna port. |
| --- | --- | --- | --- |
| 1 | 2.5 to <3.5 | 50 | -13 |
| 2 | 3.5 to <7.5 | 1000 | -13 |
| 3 | 7.5 to <8 | 500 | If PTx  +23 and (2547.5  fc  2622.5) then -23-2.28(∆f -7.5) else -16 |
| 4 | 8 to <10.4 | 1000 | -25 |
| 5 | 10.4 to  12.5 | 1000 | If PTx  +23 and (2547.5  fc  2622.5) then -21-1.68(∆f - 8) else -25 |

Table 6-23: AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to <6 | 100 | -13 |
| 2 | 6 to <10 | 1000 | -13 |
| 3 | 10 to <11 | 1000 | -13-12(f -10) |
| 4 | 11 to <15 | 1000 | -25 |
| 5 | 15 to <20 | 1000 | If PTx +23 dBm and (2550  fc  2620) then -21 - 32(f –10.5)/19 else -25 |
| 6 | 20 to  25 | 1000 | If PTx  +23 dBm and (2550  fc  2620) then -37 else -25 |

Table 6-24: AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 3

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <11 | 200 | -13 |
| 2 | 11 to <15 | 1000 | -13 |
| 3 | 15 to <16 | 1000 | -13-12(f -15) |
| 4 | 16 to  50 | 1000 | -25 |

AMS Band Class 3 - FDD

Table 6-25: AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/integration BW) at the antenna port. |
| --- | --- | --- | --- |
| 1 | 2.5 to <3.5 | 50 | -13 |
| 2 | 3.5 to <7.5 | 1000 | -13 |
| 3 | 7.5 to <8 | 500 | -16 |
| 4 | 8 to <10.4 | 1000 | -25 |
| 5 | 10.4 to  12.5 | 1000 | -25 |

Table 6-26: AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to <6 | 100 | -13 |
| 2 | 6 to <10 | 1000 | -13 |
| 3 | 10 to <11 | 1000 | -13-12(f -10) |
| 4 | 11 to <15 | 1000 | -25 |
| 5 | 15 to <20 | 1000 | -25 |
| 6 | 20 to  25 | 1000 | -25 |

- Table 6-27: AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 3

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <11 | 200 | -13 |
| 2 | 11 to <15 | 1000 | -13 |
| 3 | 15 to <16 | 1000 | -13-12(f -15) |
| 4 | 16 to  50 | 1000 | -25 |

AMS Band Class 5 – TDD

Table 6-28: AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 5

| Frequency offset f (MHz) | Maximum Emission Level (dBc) | Integration Bandwidth |
| --- | --- | --- |
| 2.5 to < 3.5 | -33.5-15(∆f-2.5) | 30 kHz |
| 3.5 to < 7.5 | -33.5-1(∆f-3.5) | 1 MHz |
| 7.5 to < 8.5 | -37.5-10(∆f-7.5) | 1 MHz |
| 8.5 to  12.5 | -47.5 | 1 MHz |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 2.515 MHz; the last is at f equals to 3.485 MHz.

- The first measurement position with a 1 MHz filter is at f equals to 4 MHz; the last is at f equals to 12 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the Integration Bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the Integration Bandwidth. When the resolution bandwidth is smaller than the Integration Bandwidth, the result should be integrated over the Integration Bandwidth in order to obtain the equivalent noise bandwidth of the Integration Bandwidth.

- Note that equivalent PSD type mask can be derived by applying 10*log ((5 MHz)/(30  kHz))= 22.2 dB and 10*log((5 MHz)/(1 MHz))= 7 dB scaling factor for 30 kHz and 1 MHz Integration Bandwidth respectively.

Table 6-29: AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 5

| Frequency offset f (MHz) | Maximum Emission Level (dBc) | Integration Bandwidth |
| --- | --- | --- |
| 5.0 to <7.0 | -33.5-9(∆f-5.0) | 30 kHz |
| 7.0 to <15.0 | -36.5-0.5(∆f-7.0) | 1 MHz |
| 15.0 to <17.0 | -40.5-5(∆f-15.0) | 1 MHz |
| 17.0 to 25.0 | -50.5 | 1 MHz |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 510.015 MHz; the last is at f equals to 6.985 MHz.

- The first measurement position with a 1 MHz filter is at f equals to 7.5 MHz; the last is at f equals to 24.5 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the Integration Bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the Integration Bandwidth. When the resolution bandwidth is smaller than the Integration Bandwidth, the result should be integrated over the Integration Bandwidth in order to obtain the equivalent noise bandwidth of the Integration Bandwidth.

- Equivalent PSD type mask can be derived by applying 10*log ((10 MHz)/(30 kHz))= 25.2 dB and 10*log((10 MHz)/(1 MHz))= 10 dB scaling factor for 30 kHz and 1 MHz Integration Bandwidth respectively.

AMS Band Class 5 – FDD

Table 6-30: AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 5 (3402.5 fc  3497.5)

| Frequency offset f (MHz) | Maximum Emission Level (dBc) | Integration Bandwidth |
| --- | --- | --- |
| 2.5 to <3.5 | -33.5-15(∆f-2.5) | 30 kHz |
| 3.5 to <7.5 | -33.5-1(∆f-3.5) | 1 MHz |
| 7.5 to <8.5 | -37.5-10(∆f-7.5) | 1 MHz |
| 8.5 to 12.5 | -47.5 | 1 MHz |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 2.515 MHz; the last is at f equals to 3.485 MHz.

- The first measurement position with a 1 MHz filter is at f equals to 4 MHz; the last is at f equals to 12 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the Integration Bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the Integration Bandwidth. When the resolution bandwidth is smaller than the Integration Bandwidth, the result should be integrated over the Integration Bandwidth in order to obtain the equivalent noise bandwidth of the Integration Bandwidth.

- Note that equivalent PSD type mask can be derived by applying 10*log ((5 MHz)/(30  kHz))= 22.2 dB and 10*log((5 MHz)/(1 MHz))= 7 dB scaling factor for 30 kHz and 1 MHz Integration Bandwidth respectively.

Table 6-31: AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 5 (3405 fc  3495)

| Frequency offset f (MHz) | Maximum Emission Level (dBc) | Integration Bandwidth |
| --- | --- | --- |
| 5.0 to <7.0 | -33.5-9(∆f-5.0) | 30 kHz |
| 7.0 to <15.0 | -36.5-0.5(∆f-7.0) | 1 MHz |
| 15.0 to <17.0 | -40.5-5(∆f-15.0) | 1 MHz |
| 17.0 to 25.0 | -50.5 | 1 MHz |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 510.015 MHz; the last is at f equals to 6.985 MHz.

- The first measurement position with a 1 MHz filter is at f equals to 7.5 MHz; the last is at f equals to 24.5 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the Integration Bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the Integration Bandwidth. When the resolution bandwidth is smaller than the Integration Bandwidth, the result should be integrated over the Integration Bandwidth in order to obtain the equivalent noise bandwidth of the Integration Bandwidth.

- Equivalent PSD type mask can be derived by applying 10*log ((10 MHz)/(30 kHz))= 25.2 dB and 10*log((10 MHz)/(1 MHz))= 10 dB scaling factor for 30 kHz and 1 MHz Integration Bandwidth respectively.

# AMS Band Class 6 – Sub-band 1710-1770/2110-2170 MHz

Table 6-32: AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 6 (1710-1770/2110-2170 MHz)

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/integration BW) at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to < 3.5 | 50 | -13 |
| 2 | 3.5 to  12.5 | 1000 | -13 |

- Table 6-33: AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 6 (1710-1770/2110-2170 MHz)

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to < 6 | 100 | -13 |
| 2 | 6 to  25 | 1000 | -13 |

Table 6-34: AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 6 (1710-1770/2110-2170 MHz)

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <11 | 200 | -13 |
| 2 | 11 to 50 | 1000 | -13 |

# AMS Band Class 6 – Sub-bands other than 1710-1770/2110-2170 MHz

Table 6-35: AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 6

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to <3.5 | 50 | -13 |
| 2 | 3.5 to < 7.5 | 1000 | -10 |
| 3 | 7.5 to <8.5 | 1000 | -13 |
| 4 | 8.5 to 12.5 | 1000 | -25 |

Table 6-36: AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 6

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to <6 | 100 | -13 |
| 2 | 6 to <10 | 1000 | -10 |
| 3 | 10 to <15 | 1000 | -13 |
| 4 | 15 to 25 | 1000 | -25 |

Table 6-37: AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 6

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <11 | 200 | -13 |
| 2 | 11 to <15 | 1000 | -10 |
| 3 | 15 to <30 | 1000 | -13 |
| 4 | 30 to 50 | 1000 | -25 |

# AMS Band Class 7 – TDD

- Table 6-38: AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7 (700.5 ≤fc  795.5)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to <2.6 | 30 | -13 |
| 2 | 2.6 to 12.5 | 100 | -13 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 2.515 MHz; the last is at f equals to 2.585 MHz. The first measurement position with a 100 kHz filter is at f equals to 2.650 MHz; the last is at f equals to 12.450 MHz.

Table 6-39: AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7 (799.5 fc  859.5)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (MHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to <7.5 | 5 | 1.6 |
| 2 | 7.5 to 12.5 | 2 | -10 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The measurement position with a 5 MHz filter is at f equals to 5 MHz. The first measurement position with a 2 MHz filter is at f equals to 8.5 MHz; the last is at f equals to 11.5 MHz.

- Table 6-40: AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 (703 fc  793)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5.0 to <5.1 | 30 | -13 |
| 2 | 5.1 to 25.0 | 100 | -13 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 5.015 MHz; the last is at f equals to 5.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 5.150 MHz; the last is at f equals to 24.950 MHz.

Table: 6-41: AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 (802 fc  857)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (MHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to <10 | 5 | 1.6 |
| 2 | 10 to 25 | 2 | -10 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The measurement position with a 5 MHz filter is at f equals to 7.5 MHz. The first measurement position with a 2 MHz filter is at f equals to 11 MHz; the last is at f equals to 24 MHz.

- Table 6-42: AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 (703 fc  793)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10.0 to <10.1 | 30 | -13 |
| 2 | 10.1 to 50.0 | 100 | -13 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 10.015 MHz; the last is at f equals to 10.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 10.150 MHz; the last is at f equals to 50.950 MHz.

Table 6-43: AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 (807 fc  852)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (MHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <15 | 5 | 1.6 |
| 2 | 15 to 50 | 2 | -10 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The measurement position with a 5 MHz filter is at f equals to 12.5 MHz. The first measurement position with a 2 MHz filter is at f equals to 16 MHz; the last is at f equals to 49 MHz.

# AMS Band Class 7 – FDD

- Table 6-44: AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7 (700.5 fc < 834.5)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to <2.6 | 30 | -13 |
| 2 | 2.6 to 12.5 | 100 | -13 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 2.515 MHz; the last is at f equals to 2.585 MHz. The first measurement position with a 100 kHz filter is at f equals to 2.650 MHz; the last is at f equals to 12.450 MHz.

Table 6-45: AMS Spectrum Emission for 5 MHz Bandwidth Band Class 7 (834.5 fc  859.5)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (MHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to <7.5 | 5 | 1.6 |
| 2 | 7.5 to 12.5 | 2 | -10 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The measurement position with a 5 MHz filter is at f equals to 5 MHz. The first measurement position with a 2 MHz filter is at f equals to 8.5 MHz; the last is at f equals to 11.5 MHz.

- Table 6-46: AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 (703 fc < 837)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5.0 to <5.1 | 30 | -13 |
| 2 | 5.1 to 25.0 | 100 | -13 |

Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 5.015 MHz; the last is at f equals to 5.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 5.150 MHz; the last is at f equals to 24.950 MHz.

- Table 6-47: AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 (837 fc  857)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (MHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to <10 | 5 | 1.6 |
| 2 | 10 to <15 | 2 | -10 |
| 3 | 15 to 25 | 1 | -25 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The measurement position with a 5 MHz filter is at f equals to 7.5 MHz. The first measurement position with a 2 MHz filter is at f equals to 11 MHz; the last is at f equals to 14 MHz. The first measurement position with a 1 MHz filter is at f equals to 15.5 MHz; the last is at f equals to 24.5 MHz.

Table 6-48: AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 (708 fc < 842)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10.0 to <10.1 | 30 | -13 |
| 2 | 10.1 to 50.0 | 100 | -13 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 10.015 MHz; the last is at f equals to 10.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 10.150 MHz; the last is at f equals to 49.950 MHz.

Table 6-49: AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 (842 fc  852)

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (MHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <15 | 5 | 1.6 |
| 2 | 15 to <20 | 2 | -10 |
| 3 | 20 to 50 | 1 | -25 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The measurement position with a 5 MHz filter is at f equals to 12.5 MHz. The first measurement position with a 2 MHz filter is at f equals to 16 MHz; the last is at f equals to 19 MHz. The first measurement position with a 1 MHz filter is at f equals to 20.5 MHz; the last is at f equals to 49.5 MHz.

# AMS Band Class 8

Table 6-50: AMS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 8

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/integration BW) at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to < 3.5 | 50 | -13 |
| 2 | 3.5 to  12.5 | 1000 | -13 |

- Table 6-51: AMS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 8

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to < 6 | 100 | -13 |
| 2 | 6 to  25 | 1000 | -13 |

Table 6-52: AMS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 8

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <11 | 200 | -13 |
| 2 | 11 to <15 | 1000 | -10 |
| 3 | 15 to <30 | 1000 | -13 |
| 4 | 30 to 50 | 1000 | -25 |

ABS spectral masks:

The spectrum masks in the following three tables (6-53, 6-54, 6-55) are applicable to all bands and all regions unless specific mask for a band or a country/region is specified.

Table 6-53: ABS Spectrum Emission Mask for 5 MHz Bandwidth

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to <7.5 | 100 | -7-7(∆f-2.55)/5 |
| 2 | 7.5 to 12.5 | 100 | -14 |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 100 kHz filter is at f equals to 2.550 MHz; the last is at f equals to 12.450 MHz.

- Integration Bandwidth refers to the frequency range over which the emission power is integrated.

Table 6-54: ABS Spectrum Emission Mask for 10 MHz Bandwidth

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to <10 | 100 | -7-7(∆f-5.05)/5 |
| 2 | 10 to <15 | 100 | -14 |
| 3 | 15 to 25 | 1000 | -13 |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 100 kHz filter is at f equals to 5.05 MHz; the last is at f equals to 14.95 MHz. The first measurement position with a 1 MHz filter is at f equals to 15.5 MHz; the last is at f equals to 24.5 MHz.

- Integration Bandwidth refers to the frequency range over which the emission power is integrated.

Table 6-55: ABS Spectrum Emission Mask for 20 MHz Bandwidth

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <15 | 100 | -7-7(∆f-10.05)/5 |
| 2 | 15 to <20 | 100 | -14 |
| 3 | 20 to 50 | 1000 | -13 |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 100 kHz filter is at f equals to 10.05 MHz; the last is at f equals to 19.95 MHz. The first measurement position with a 1 MHz filter is at f equals to 20.5 MHz; the last is at f equals to 49.5 MHz.

- Integration Bandwidth refers to the frequency range over which the emission power is integrated.

# ABS Band Class 1

Table 6-56: ABS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 1-United States

| Segment Number | Offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to < 3.5 | 50 | 13 |
| 2 | 3.5 to  12.5 | 1000 | 13 |

Table 6-57: ABS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 1-United States

| Segment Number | Offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to  6 | 100 | 13 |
| 2 | 6 to  25 | 1000 | 13 |

Table 6-57-1: ABS Spectrum Emission Mask for 8.75 MHz Bandwidth Band Class 1-Korea, Ptx≥40 dBm

| Segment Number | Offset f from channel center (MHz) | Measurement bandwidth (KHz) | Allowed emission as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 4.77 to  22.5 | 100 | -56.9 dBc |

Table 6-57-2: ABS Spectrum Emission Mask for 8.75 MHz Bandwidth Band Class 1-Korea, 29≤Ptx<40 dBm

| Segment Number | Offset f from channel center (MHz) | Measurement bandwidth (KHz) | Allowed emission as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 4.77 to  22.5 | 100 | -53.9 dBc |

Table 6-57-3: ABS Spectrum Emission Mask for 8.75 MHz Bandwidth Band Class 1-Korea, Ptx≤29 dBm

| Segment Number | Offset f from channel center (MHz) | Measurement bandwidth (KHz) | Allowed emission as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 4.77 to  22.5 | 100 | -14.5 |

# ABS Band Class 3 – TDD

Table 6-58: ABS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 2.5  f  3.5 MHz | 13 dBm | 50 kHz |
| 3.5  f  12.5 MHz | 13 dBm | 1 MHz |

Table 6-59: ABS Spectrum Emission Mask for 10 MHz Bandwidth Band Class

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 5  f  6 MHz | 13 dBm | 100 kHz |
| 6  f  25 MHz | 13 dBm | 1 MHz |

Table 6-60: ABS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 3

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 10  f  11 MHz | 13 dBm | 200 kHz |
| 11  f  50 MHz | 13 dBm | 1 MHz |

Table 6-61: ABS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3-Japan

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 7.5 MHz  f < 12.25 | 151.4 × (f 7.5) dBm | 1 MHz |
| 12.25  f  22.5 MHz | 22 dBm | 1 MHz |

Table 6-62: ABS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3-Japan

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 15  f  25 MHz | 22 dBm | 1 MHz |

# ABS Band Class 3 – FDD

Table 6-63: ABS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 3 - United States

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 2.5  f  3.5 MHz | 13 dBm | 50 kHz |
| 3.5  f  12.5 MHz | 13 dBm | 1 MHz |

Table 6-64: ABS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 3 - United States

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 5  f  6 MHz | 13 dBm | 100 kHz |
| 6  f  25 MHz | 13 dBm | 1 MHz |

Table 6-65: ABS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 3 - United States

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 10  f  11 MHz | 13 dBm | 200 kHz |
| 11  f  50 MHz | 13 dBm | 1 MHz |

# ABS Band Class 5 – TDD

The Spectrum Emission Mask for 5, 10 and 20 MHz bandwidth sizes are specified in Tables 6-66 and 6-67. Table 6-66 specifies breakpoints of the underlying piecewise linear power spectral density mask. This mask is a relative mask and conditionally applicable depending on the ABS Pnom power level. Table 6-67 specifies the emission levels of an underlying piecewise step function applicable conditionally only to some of Pnom power levels.

Table 6-66: ABS Relative Transmit Spectral Power Density Mask

| Power | Frequency Offset |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0.5*BW | 0.71*BW | 1.06*BW | 2.0*BW | 2.5*BW |
| 39 dBm  Pnom | -20 dB | -27 dB | -32 dB | -50dB | -50dB |
| 33 dBm  Pnom ≤39 dBm | -20 dB | -27 dB | -32 dB | -50 dB + (39 dBm - Pnom) | Refer to Table 6-67 |

Table 6-67: ABS Absolute Spectral Emission Mask

| Power | Frequency Offset |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0.50 BW  f  0.71 BW | 0.71 BW  f  1.06 BW | 1.06 BW  f  2.00 BW | 2.00 BW  f  2.50 BW |
| 33 dBm  Pnom ≤ 39 dBm | Refer to Table 6-66 | Refer to Table 6-66 | Refer to Table 6-66 | -21 + x dBm/MHz |
| Pnom ≤ 33 dBm | -5.5 dBm/MHz | -5.5 dBm/MHz | -23.5 dBm/MHz | -23.5 dBm/MHz |

Notes: In Table 6-67, x = -10 log(BW/10).

# ABS Band Class 6 – Sub-band 1710-1755/2110-2155 MHz

The Spectrum Emission Mask of Table 6-68, Table 6-69, and Table 6-70 apply to the United States.

Table 6-68: ABS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 5 (1710-1755/2110-2155 MHz)

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/integration BW) at the antenna port. |
| --- | --- | --- | --- |
| 1 | 2.5 to < 3.5 | 50 | -13 |
| 2 | 3.5 to  12.5 | 1000 | -13 |

Table 6-69: ABS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 5 (1710-1755/2110-2155 MHz)

| Segment Number | Offset from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to < 6 | 100 | -13 |
| 2 | 6 to  25 | 1000 | -13 |

Table 6-70: ABS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 5 (1710-1755/2110-2155 MHz)

| Frequency offset from centre | Allowed emission level | Integration Bandwidth |
| --- | --- | --- |
| 10  f  11 MHz | 13 dBm | 200 kHz |
| 11  f  50 MHz | 13 dBm | 1 MHz |

# ABS Band Class 7

The Spectrum Emission Mask of Table 6-71, Table 6-72, and Table 6-73 apply to the United States.

- Table 6-71: ABS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7 - United States

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to < 2.6 | 30 | -13 |
| 2 | 2.6 to 12.5 | 100 | -13 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 2.515 MHz; the last is at f equals to 2.585 MHz. The first measurement position with a 100 kHz filter is at f equals to 2.650 MHz; the last is at f equals to 12.450 MHz.

- Table 6-72: ABS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 - United States

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5.0 to < 5.1 | 30 | -13 |
| 2 | 5.1 to  25.0 | 100 | -13 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 5.015 MHz; the last is at f equals to 5.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 5.150 MHz; the last is at f equals to 24.950 MHz.

- Table 6-73: ABS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 - United States

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10.0 to <10.1 | 30 | -13 |
| 2 | 10.1 to  50.0 | 100 | -13 |

- Notes:

- f is the separation between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 30 kHz filter is at f equals to 10.015 MHz; the last is at f equals to 10.085 MHz. The first measurement position with a 100 kHz filter is at f equals to 10.150 MHz; the last is at f equals to 50.950 MHz.

The Spectrum Emission Mask of Table 6-74, Table 6-75, and Table 6-76 apply to Europe.

Table 6-74: ABS Spectrum Emission Mask for 5 MHz Bandwidth Band Class 7 - Europe

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 2.5 to <7.5 | 100 | -7-7(∆f-2.55)/5 |
| 2 | 7.5 to 12.5 | 100 | -14 |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 100 kHz filter is at f equals to 2.550 MHz; the last is at f equals to 12.450 MHz.

- Integration Bandwidth refers to the frequency range over which the emission power is integrated.

Table 6-75: ABS Spectrum Emission Mask for 10 MHz Bandwidth Band Class 7 - Europe

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 5 to <10 | 100 | -7-7(∆f-5.05)/5 |
| 2 | 10 to <15 | 100 | -14 |
| 3 | 15 to 25 | 100 | -13 |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 100 kHz filter is at f equals to 5.05 MHz; the last is at f equals to 24.95 MHz.

- Integration Bandwidth refers to the frequency range over which the emission power is integrated.

Table 6-76: ABS Spectrum Emission Mask for 20 MHz Bandwidth Band Class 7 - Europe

| Segment Number | Frequency offset f from channel center (MHz) | Integration Bandwidth (kHz) | Allowed Emission Level (dBm/Integration Bandwidth) as measured at the antenna port |
| --- | --- | --- | --- |
| 1 | 10 to <15 | 100 | -7-7(∆f-10.05)/5 |
| 2 | 15 to <20 | 100 | -14 |
| 3 | 20 to 50 | 100 | -13 |

Notes:

- f is the absolute value of separation in MHz between the carrier frequency and the centre of the measuring filter.

- The first measurement position with a 100 kHz filter is at f equals to 10.05 MHz; the last is at f equals to 49.95 MHz.

Integration Bandwidth refers to the frequency range over which the emission power is integrated.

_________________