| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to section 16.2.5.2.3.1 AES CCM |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Erik Colban Huawei Technologies, Ltd | Voice: +1 858 775 2494 E-mail: ecolban@huawei.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot – P802.16m IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems - Amendment: Advanced Air Interface |  |
| Abstract | This document shows suggested changes to P802.16m Draft D6 related to AES-CCM message encryption and authentication. |  |
| Purpose | Review and approve as a remedy to related Sponsor Ballot comment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to section 16.2.5.2.3.1 AES CCM

Erik Colban, Bob Sultan

Huawei Technologies, Ltd

# Overview

Section 16.2.5.2.3.1, which specifies AES-CCM payload encryption, has several deficiencies. This contribution proposes several changes that would improve the standard.

# Proposed changes

## Change 1

Add reference in Normative Reference section:

NIST Special Publication 800-38C—Recommendation for Block Cipher Modes of Operation: The CCM Mode for Authentication and Confidentiality, May 2004.

## Change 2

Edit text in section 16.2.5.2.3.1 as shown below.

16.2.5.2.3 Cryptographic Methods

16.2.5.2.3.1 Payload encryption methods

AES-CCM (refer to NIST Special Publication 800-38C and FIPS 197 Advanced Encryption Standard (AES)) may be used to encrypt unicast control connections. Unicast transport connections may be encrypted with AES-CTR (refer to NIST Special Publication 800-38A) or AES-CCM.

16.2.5.2.3.1.1 AES-CCM

PDU payload format

The MAC PDU payload shall be prepended with a 2-bit EKS and a 22-bit PN (Packet Number). The EKS and PN shall not be encrypted. The plaintext PDU shall be encrypted and authenticated using the active TEK, according to the CCM specification. This includes appending an integrity check value (ICV) to the end of the payload and encrypting both the plaintext payload and the appended ICV where the size of ICV is decided by either 4 or 8 byte during key agreement procedure in network entry. The processing yields a payload that is 7 or 11 bytes longer than the plaintext payload.

The processing yields a payload that is 7 or 11 bytes longer than the plaintext payload.

Packet number (PN)

The PN associated with an SA shall be set to 0x000001 for DL and 0x200001 for UL when the SA is established and when a new TEK is installed. After each PDU transmission, the PN shall be incremented by 1. Any pair value of {PN, TEK} shall not be used more than once for the purposes of transmitting data. The AMS shall ensure that a new TEK is derived on both sides before the PN paired with either TEK for downlink or TEK for uplink reaches maximum value 0x1FFFFF or 0x3FFFFF, respectively. If the PN reaches maximum value without new TEKs being installed, transport communications on that SA shall be halted until new TEKs are installed.

CCM algorithm

NIST Special Publication 800-38C defines a number of algorithm parameters. Those parameters shall be fixed to specific values as follows.

The payload, P, shall be the connection payload, i.e., the PDU excluding the MAC header and any extended headers as well as the EKS, PN, and the Integrity Check Value (ICV) fields (refer to Figure 394).

The message authentication code, T, is referred herein as the ICV. The length of T in bytes, t, is either 4 or 8.

The cipher block key, K, is the current TEK as defined herein.

The associated data, A, is the empty string. The length of the associated data, a, is zero.

The nonce, N, shall be as shown in Table <xyz>. The MAC header, which fills bytes 1 and 2, may be the AGMH or the SPMH. If the STID or the FID has not been assigned, then the corresponding field shall be set to all zeroes. When payloads with different FIDs are multiplexed into the MAC PDU, the FID field of the nonce shall be set to the FID of the first payload connection.

<Insert Figure 402 here, but replace “NONCE” by “Nonce”.  (nonce is not an acronym, but a complete word). Replace “Figure 402” by “Table <xyz>”, where <xyz> is the appropriate table number (reason = tables are normative, figures are informative).>

The block cipher algorithm, CIPHK, is AES as specified in FIPS 197 Advanced Encryption Standard (AES).

The formatting function and the counter generation function are specified in NIST Special Publication 800-38C, Appendix A. In adopting Appendix A, the Length field Q shall be two bytes long, i.e., the octet length of the Length field, q, is 2. It follows that the Flags byte in the initial block, B0, has value 0x09 if a 4-byte ICV is used, or 0x19 if an 8-byte ICV is used. The initial block B0 is shown in Figure 403.

<Insert Figure 403 here, but replace “Flag’ with ‘Flags’ (the byte contains 4 flags) in byte 0 and “NONCE” by “nonce”.>

The j’th counter block, Ctrj, is shown in Figure 404. Note that the Flags byte in the counter blocks, which is different from the Flags byte in the initial block B0, has constant value 0x01.

<Insert Figure 404 here, but replace “Flag’ with ‘Flags’ in byte 0 and “NONCE” by “nonce”.>

<Delete the following text up to, but not including, line 45 on page 254. >