| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for change in Co-located Coexistence (CLC) message attribute (Section 16.2.3.17) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Siddharth Shetty, Punit Rathod, Abhay Karandikar TICET, Indian Institute of Technology Bombay Zhu Jing, Shantidev Mohanty Intel Corp. | Voice: +91 22 2576 4473 E-mail: shettysid@iitb.ac.in, punit@cse.iitb.ac.in, karandi@ee.iitb.ac.in |
| Re: | IEEE 802.16m Sponsor Ballot: P802.16m/D6 |  |
| Abstract | The contribution proposes a change in CLC report attribute |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposal for change in Co-located Coexistence (CLC) message attribute

Siddharth Shetty, Punit Rathod (Indian Institute of Technology, Bombay)

Zhu Jing, Shantidev Mohanty (Intel Corp.)

# Introduction

The contribution proposes addition of a bit in the Report Type attribute contained in the CLC Report attribute sent by the AMS to ABS in the AAI_CLC-REQ message.

The justification behind distinguishing between the two interference cases (i.e. collocated and non-collocated) is that the ABS can make an informed decision about necessary actions to prevent performance degradation. Note that performance degradation in the two cases may be distinct in nature i.e. collocated cases will tend to be more static since both radios exist on the same platform, whereas interference for non-collocated radios is more likely to be dynamic (e.g. the two non-collocated devices housing the radios may simply move away from each other and that would reduce interference)

Alternatively the ABS could share this information with a higher entity (e.g. SON server) which in turn can inform the non-802.16 network to take appropriate action(s) to mitigate interference (eg. channel change).

# References

[1] IEEE P802.16m/D6

# Proposed Modifications

Section 16.2.3.17, Page134 lines 39-63:

Add / modify text in Table 703 and lines 51-63 and shows below:

---------------------------------------Start of text for proposed Modification ---------------------------------------------

Table 703—CLC Report parameters

| M/O | Attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Report Type | 8 | 0: Interference Level 1: non collocated interference level Others: reserved | N.A. |
| O | Interference level | 8 | Signed integer: -127 ~ 127 | Present if (Report Type = = 0 OR Report Type == 1) |

Parameters shall be as follows:

Report Type

This field indicates which of the following fields are included.

Interference Level

This field is present if (Report Type == 0 OR Report Type == 1).

If Report Type == 0, this level indicates the average power level over one OFDMA symbol in unit of 1 dBm with the range from -127dBm to 127dBm received at the AMS from its co-located non 802.16 radios when they are active

If Report Type == 1, this level indicates the average power level over one OFDMA symbol in unit of 1 dBm with the range from -127dBm to 127dBm received at the AMS from its non co-located non 802.16 radios when they are active

---------------------------------------End of text for proposed Modification ----------------------------------------------