| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-Up Text for AAI_FFR-REP message format (16.2.3.19) |  |
| Date Submitted | 2010-07-12 |  |
| Source(s) | Jeongho Park Samsung Electronics | jeongho.jh.park@samsung.com |
| Re: | Proposed text changes to P802.16m/D6 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D7 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-Up Text for AAI_FFR-REP message format (16.2.3.19)

Jeongho Park

Samsung Electronics

- Introduction and Problem Definition

Current message format of AAI_FFR-REP is not correct for ASN.1.

- Proposed Text

Remedy #1: page 136-137, Modify Table 705 as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

Table 705—AAI_FFR-REP message format

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | frequencyPartitionBitMap | 4 | Each bit indicates the interference statistics report status of corresponding Frequency Partition. 0b0: no report interference statistics 0b1: report interference statistics Frequency Partition Bitmap shall contain at least one bit with value "1". The LSB indicates the lowest available FP and the MSB indicates the highest available FP where the size of an available FP is bigger than zero. | N.A. |
| M | reportType | 4 | Each bit indicates if one type of report is required to be sent by AMS. If one bit has value '1', it indicates the specific report type is being sent, otherwise it indicates the specific report type is not being sent. At least 1 bit needs to be set to value '1'. LSB#0: Interference-Mean LSB#1: Interference-Variance LSB#2: SINR-Mean LSB#3: SINR-Variance |  |
| O | FFR-REPORT | Variable |  |  |
| O | FFR-REPORT A)fp0Report | Variable | FFR-FeedbackIE | Present when 0-th LSB of frequencyPartitionBitMap is 0b1 |
| O | FFR-REPORT B)fp1Report | Variable | FFR-FeedbackIE | Present when 1-st LSB of frequencyPartitionBitMap is 0b1 |
| O | FFR-REPORT C)fp2Report | Variable | FFR-FeedbackIE | Present when 2-nd LSB of frequencyPartitionBitMap is 0b1 |
| O | FFR-REPORT D)fp3Report | Variable | FFR-FeedbackIE | Present when 3-th LSB of frequencyPartitionBitMap is 0b1 |
| O | FFR-FeedbackIE | Variable |  |  |
| O | FFR-FeedbackIE A)interferenceMean | 8 | Interference mean. This is noise plus inter-cell interference power level which is averaged over the frequency partition and divided by the number of subcarriers in the frequency partition. -134dBm to -30dBm in units of 1dB. -134dBm is encoded as 0x00, -30dB is encoded as 0x68, 0x69 to 0xFF is reserved. | Present when 0-th LSB of reportType is 0b1 |
| O | FFR-FeedbackIE B)interferenceVariance | 4 | Interference variance 0dB to 15dB in units of 1dB | Present when 1-st LSB of reportType is 0b1 |
| O | FFR-FeedbackIE C)sinrMean | 8 | SINR mean -16dB to 53dB in units of 0.5dB -16dB is encoded as 0x00, 53dB is encoded as 0x8A, 0x8B-0xFF are reserved | Present when 2-nd LSB of reportType |
| O | FFR-FeedbackIE D)sinrVariance | 4 | SINR variance 0dB to 15dB in units of 1dB. | Present when 3-rd LSB of reportType is 0b1. |

-------------------------------------------------Text Change END -----------------------------------------------------