| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reply for comment #209 (16.2.3.53) |  |
| Date Submitted | 2010-07-13 |  |
| Source(s) | Hyunjeong Kang Samsung Electronics Yuqin Chen ZTE | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | SB_16m on P802.16m/D6 |  |
| Abstract | The contribution proposes a table format for AAI_MC-RSP message. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reply for comment #209

Hyunjeong Kang

Samsung Electronics

Yuqin Chen

ZTE

- Introduction

Table 758 should be changed if the Table 757 is removed.

- Text proposal

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1: Modify Table 758 in page 217 as follows:]

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Global Assign | 1 | Indicates whether the ABS assigns all the carriers requested by AMS through AAI_MC-REQ 0b0: The ABS assigns a subset of the carriers requested by AMS 0b1: The ABS assigns all the carriers requested by AMS |  |
| O | Number of assigned carriers | 3 | Indicates the number of carriers to be assigned to the AMS | Shall be present when the Global Assign is zero |
| O | Physical carrier index | 6 | Indicates the carriers to be assigned to the AMS. Logical carrier index is assigned implicitly in the order of assigned physical carrier index | Shall be present when the Global Assign is zero and the Number of assigned carriers is not zero. |
| M | Support of data transmission over guard sub-carrier | 1 | Indicates the support of guard carrier for data 0b0: not supported 0b1: supported | N.A. Shall be present when the Global Assign is zero and the Number of assigned carriers is not zero. |
| O | DU/UL indicator | 1 | Indicates whether the carrier is assigned for both DL and UL 0b0: The carrier is assigned for both DL and UL 0b1: The carrier is assigned for DL only. | Shall be present when the Global Assign is zero and the Number of assigned carriers is not zero. |

[Remedy 2: Delete Table 757 in page 216:]

-------------------------------  Text End  ---------------------------------------------------