| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Definitions of “capability class 0” and parameters for 802.16m system |  |
| Date Submitted | 2010-07-14 |  |
| Source(s) | Sassan Ahmadi Shaocheng Wang Intel | E-mail: Sassan.Ahmadi@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to fix AAI_SBC-REQ/RSP issues |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Definitions of “capability class 0” and parameters for 802.16m system

- Sassan Ahmadi, Shaocheng Wang

- Intel

- Introduction

This contribution is intended to provide definitions for “capability class 0” and parameters for 802.16m system.

- Proposed text

II.1 Proposed text 1

[Note to the editor: create a new appendix and insert the following table. Also add the following text to the end of Ln 12, page 380. (section 16.2.15.4)

The content of “Capability Class 0” is defined in Appendix X of this specification.

------------------------------------------------- Start of proposed text I--------------------------------------------------

Table xxx—Default capability class and parameters for 802.16m system

| Feature | Feature Subset1 | Feature Subset2 | Description | Standard reference (802.16m/D6) |
| --- | --- | --- | --- | --- |
| 4.1.3 DL PHY Structure | 4.1.3.x Subframe type support |  | PRU for different subframe types (Ref: added Frame_Structure reference later) |  |
| 4.1.3 DL PHY Structure | 4.1.3.1 Frequency partitioning |  |  |  |
|  |  |  | Reuse-1 with one freq partition for SFH only | 16.3.6.2.1 |
|  | 4.1.3.2 DL subchannelization and resource unit |  | Distributed LRU (DLRU) | 16.3.5.1.1 |
|  |  |  | A-MAP logical resource unit (MLRU) | 16.3.5.3.4 |
|  |  |  | Subchannelization for SFH | 16.3.6.2.1 |
|  | 4.1.3.3 Pilot pattern |  |  |  |
|  |  |  | 2 stream pilot patterns for DLRU & CLRU with interlaced pilots | 16.3.5.4.1, 16.3.5.4.3 |
|  | 4.1.3.4 MIMO midamble |  | Support of MIMO Midamble | 16.3.5.4.2 |
|  | 4.1.3.5 Traffic guard subcarrier in multicarrier |  |  |  |
| 4.1.4 UL PHY Structure | 4.1.4.1 Frequency partitioning |  |  |  |
|  | 4.1.4.2 UL subchannelization and resource unit |  | Distributed LRU (DLRU) | 16.3.8.1.1 |
|  |  |  | Subband Contiguous LRU (SLRU) | 16.3.8.1.2 |
|  |  |  | UL Control Channels - Bandwidth request channels | 16.3.8.3.3.1 |
|  |  |  | UL Control Channels - HARQ feedback (ACK/NACK) channels | 16.3.8.3.3.2 |
|  |  |  | UL Control Channels - synchronized ranging channels | 16.3.9.1.4 |
|  | 4.1.4.3 Pilot pattern |  | 1 stream pilot patterns for DLRU & CLRU | 16.3.8.4 |
|  | 4.1.4.4 WirelessMAN-OFDMA Systems Support |  |  |  |
|  | 4.1.4.5 Uplink sounding |  |  |  |
|  | 4.1.4.6 Traffic guard subcarrier in multicarrier |  |  |  |
| 4.1.5 Fractional Frequency Reuse | 4.1.5.1 Downlink FFR |  |  |  |
|  | 4.1.5.2 Uplink FFR |  |  |  |
| 4.1.10 Channel Coding | 4.1.10.1 Data Channel | 4.1.10.1.1 Encoder | Data Burst CRC encoding | 16.3.11.1.1 |
|  |  |  | Data Burst partition process | 16.3.11.1.2 |
|  |  |  | FEC Block randomization | 16.3.11.1.3 |
|  |  |  | FEC Block CRC encoding | 16.3.11.1.4 |
|  |  |  | FEC Encoding | 16.3.11.1.5 |
|  |  |  | Bit selection and repetition | 16.3.11.1.6 |
|  |  |  | Bit Collection | 16.3.11.1.7 |
|  |  | 4.1.10.1.2 Modulation | DL QPSK | 8.4.9.4.2 |
|  |  |  | DL 16QAM | 8.4.9.4.2 |
|  |  |  | UL QPSK | 8.4.9.4.2 |
|  |  |  | UL 16QAM | 8.4.9.4.2 |
|  | 4.1.10.2. Control Channel | 4.1.10.2.1 Encoder | FEC Encoding (TBCC) | 16.3.11.2.1 |
|  |  |  | Bit Separation | 16.3.11.2.2 |
|  |  |  | subblock interleaver | 16.3.11.2.3 |
|  |  |  | Bit grouping | 16.3.11.2.4 |
|  |  |  | Bit Selection | 16.3.11.2.5 |
|  | 4.1.10.3 HARQ | 4.1.10.3.1 IR HARQ | HARQ Redundancy selection - DL | 16.3.11.4.1 |
|  |  |  | HARQ Redundancy selection - UL | 16.3.11.4.1 |
|  |  | 4.1.10.3.2 Constellation Rearrangement | Constellation rearrangement version (MIMO stream = 1) | 16.3.11.4.2 |
|  |  |  | Constellation rearrangement version (MIMO stream > 1) | 16.3.11.4.2 |
|  |  |  | DL CoRe version signaling | 16.3.11.4.2 |
|  |  |  | UL CoRe version signaling | 16.3.11.4.2 |
|  | 4.1.10.4 Subcarrier Randomization |  | PRBS for subcarrier randomization | 16.3.11.3.1 |
|  |  |  | Data subcarrier randomization | 16.3.11.3.2 |
|  |  |  | Pilot subcarrier randomization | 16.3.11.3.3 |
| 4.1.6 DL Control Structure | 4.1.6.1 Advanced Preamble (A-Preamble) | 4.1.6.1.1 Primary Advanced Preamble (PA-Preamble) | Primary A-Preamble | 16.3.6.1.1 |
|  |  | 4.1.6.1.2 Secondary Advanced Preamble (SA-Preamble) | Secondary A-Preamble | 16.3.6.1.2 |
|  | 4.1.6.2 Superframe Header (SFH) | 4.1.6.2.1 Primary Superframe Header (P-SFH) | Tx period (every superframe) | 16.3.6.2.1.1 |
|  |  |  | P-SFH code rate | 16.3.6.3.1.1 |
|  |  | 4.1.6.2.2 Secondary Superframe Header (S-SFH) | S-SFH code rate | 16.3.6.3.1.2 |
|  |  |  | Tx period of SP1 IE | 16.3.6.2.1.2 |
|  |  |  | Tx period of SP2 IE | 16.3.6.2.1.2 |
|  |  |  | Tx period of SP3 IE | 16.3.6.2.1.2 |
|  | 4.1.6.3 Advanced MAP (A-MAP) | 4.1.6.3.1 Location of A-MAP Region for FFR |  |  |
|  |  | 4.1.6.3.2 A-MAP Transmission/Reception | Support of Non-User Specific A-MAP IE | 16.3.6.5.2.1 |
|  |  |  | Support of DL Basic Assignment A-MAP IE | 16.3.6.5.2.4.1 |
|  |  |  | Support of Broadcast Assignment A-MAP IE | 16.3.6.5.2.4.13 |
|  |  |  | Assignment A-MAP IE, Code rate | 16.3.6.3.2.4 |
|  |  |  | Support of HARQ Feedback A-MAP IE | 16.3.6.5.2.2 |
|  |  |  | Support of Power Control A-MAP IE | 16.3.6.5.2.3 |
|  |  | 4.1.6.3.3 A-MAP Power Boosting |  | 16.3.6.4.1 |
|  |  | 4.1.6.3.4 A-MAP Resource Index | Assignment A-MAP (group size indication, and MCRC) | 16.3.6.3.2.4 |
|  |  | 4.1.6.3.5 HARQ Feedback A-MAP Resource Index |  |  |
| 4.1.7 UL Control Strcuture | 4.1.7.1 UL control channel for FFR | 4.1.7.1.1 Location of UL control channel for FFR |  |  |
|  | 4.1.7.2 Primary Fast Feedback Channel | 4.1.7.2.1 Physical Channel Structure |  |  |
|  |  | 4.1.7.2.2 Information Content | Reference to 4.1.8.3 |  |
|  | 4.1.7.3 Secondary Fast Feedback Channel | 4.1.7.3.1 Physical Channel Structure |  |  |
|  |  | 4.1.7.3.2 Information Content | Reference to 4.1.8.3 |  |
|  | 4.1.7.4 HARQ Feedback Channel | 4.1.7.4.1 Physical Channel Structure | Coding, modulation and subcarrier mapping of HARQ feedback channel | 16.3.9.2.2 |
|  |  | 4.1.7.4.2 Information Content | HARQ ACK/NACK Feedback | 16.3.9.2.2 |
|  | 4.1.7.5 Sounding Channel | 4.1.7.5.1 Physical Channel Structure |  |  |
|  | 4.1.7.6 Ranging Channel for Non-synchronized AMSs | 4.1.7.6.1 Physical Channel Structure |  |  |
|  | 4.1.7.7 Ranging Channel for Synchronized AMSs | 4.1.7.7.1 Physical Channel Structure | Synchronized ranging physical channel structure in 16m-only mode | 16.3.9.1.4.2 |
|  | 4.1.7.8 Bandwidth Request Channel | 4.1.7.8.1 Physical Channel Structure | Coding, modulation and subcarrier mapping of bandwidth request channel preamble part | 16.3.9.2.5 |
|  |  | 4.1.7.8.2 Information Content | Support of preamble only (5 step) | 16.2.11.1.1 |
| 4.1.X Control Mechanism | 4.1.X.1 Synchronization |  |  |  |
|  | 4.1.X.2 Uplink Power Control | 4.1.X.2.1 Uplink Power control | Support of uplink power control | 16.3.9.4 |
|  |  | 4.1.X.2.2 Control Channel TX PC correction by PC-A-MAP | Support of PC-A-MAP | 16.3.9.4 |
|  |  | 4.1.X.2.3 MS TX Status Report | Support of Period-based report | 16.3.9.4 |
|  |  |  | Support of event driven report | 16.3.9.4 |
|  |  |  | Support of ULPC Parameters Updating Indicator | 16.2.2.1.3.6 |
|  |  | 4.1.X.2.4 Initial ranging power control |  |  |
|  |  | 4.1.X.2.x Initial network entry/re-entry power control |  |  |
|  |  | 4.1.X.2.y Handover power control |  |  |
|  |  | 4.1.X.2.5 Sounding channel power control |  |  |
|  |  | 4.1.X.2.6 Concurrent transmission of uplink control channel and data |  |  |
|  |  | 4.1.X.2.7 Power control parameter signaling |  |  |
|  |  |  | Support of NI broadcast by AAI_ULPC_NI message | 16.2.3.31 |
|  |  |  | Support of power control parameter broadcast by AAI_SCD message | 16.2.3.30 |
|  |  |  | Support of power control offset unicast by AAI_UL POWER ADJ | 16.2.3.32 |
|  | 4.1.X.3 Ranging | 4.1.X.3.1 Non-dynamic ranging | Support of non-dynamic ranging | 16.3.6.5.2.4.13 |
|  |  | 4.1.X.3.2 Dynamic ranging |  |  |
| 4.1.Y Channel Measurement | 4.1.Y.1 Channel measurement |  |  |  |
|  | 4.1.Y.2 RSSI measurement |  |  |  |
|  | 4.1.Y.3 Interference measurement |  |  |  |
| 4.1.x A-MAP and HARQ operation | 4.1.x.2 A-MAP relevance and HARQ timing | 4.1.x.2.1 Processing Time | TUL_Rx_Processing to support F = 8 or D+U = 8 | 16.2.14.2.1 |
|  |  |  | TUL_Tx_Processing to support F = 8 or D+U = 8 | 16.2.14.2.1 |
|  |  |  | TDL_Rx_Processing to support F = 8 or D+U = 8 | 16.2.14.2.1 |
|  |  | 4.1.x.2.2 A-MAP relevance | DL Default TTI A-MAP Relevance | 16.2.14.2.2 |
|  |  |  | UL Default TTI A-MAP Relevance | 16.2.14.2.2 |
|  |  | 4.1.x.2.3 HARQ Feedback delay | DL Default TTI HARQ Feedback Delay | 16.2.14.2.2 |
|  |  |  | UL Default TTI HARQ Feedback Delay | 16.2.14.2.2 |
|  | 4.1.x.3 HARQ | 4.1.x.3.1 Support for Downlink HARQ Incremental Redundancy | DL maximum retransmission number (DL N_MAX_ReTx) | 16.2.14.2.1.1 |
|  |  |  | DL retransmission max interval (T_ReTx_Interval) | 16.2.14.2.1.1 |
|  |  | 4.1.x.3.2 Support for Uplink HARQ Incremental Redundancy | UL maximum retransmission number (UL N_MAX_ReTx) | 16.2.14.2.1.2 |
|  | 4.1.x.4 Persistent Allocation (PA) |  |  |  |
|  | 4.1.x.5 Group Resource Allocation (GRA) |  |  |  |
|  |  | 4.1.x.5.1 Group configuration |  |  |
|  | 4.1.x.6. S-SFH Update | 4.1.x.6.1 S-SFH Update procedure |  | 16.2.24 |
| 4.1.8 DL MIMO | 4.1.8.1 DL MIMO General | 4.1.8.1.1 Number of Tx antenna | 2 Transmit antennas | 16.3.7.2.5.5 |
|  |  | 4.1.8.1.2 Number of streams for SU-MIMO |  |  |
|  |  | 4.1.8.1.3 Number of streams for MU-MIMO |  |  |
|  |  | 4.1.8.1.4 Tx/Rx antenna/RF chain calibration at BS |  |  |
|  | 4.1.8.2 DL MIMO mode | 4.1.8.2.2 Mode 0 (TxD, SFBC, non-adaptive precoder, vertical encoding) | Support of MIMO Mode 0 | 16.3.7.2.1 |
|  |  |  | [Support of 2Tx] | 16.3.7.2.1 |
|  |  |  | OL SU-MIMO, DLRU, outside OL Region | 16.3.7.2.1 |
|  |  | 4.1.8.2.7 Mode 5 (TxD, CDR with non-adaptive precoder, vertical encoding) |  |  |
|  | 4.1.8.3 Feedback support for DL MIMO | 4.1.8.3.1 Base mode | Support of Base mode | 16.3.7.2.5.5 |
|  |  | 4.1.8.3.2 Transformation mode | Support of Transformation mode | 16.3.7.2.5.5.1 |
|  |  | 4.1.8.3.12 Covariance matrix feedback | Support of Covariance matrix feedback | 16.3.7.2.5.5.1 |
| 4.1.9 UL MIMO | 4.1.9.1 UL MIMO General | 4.1.9.1.1 Number of Tx antenna | 1 Transmit antennas | 16.3.10.1.1 |
|  | 4.1.9.2 UL MIMO mode | 4.1.9.2.2 Mode 0 (TxD, SFBC, non-adaptive precoder, vertical encoding) | Support of MIMO mode 0 | 16.3.10.2.1 |
|  |  | 4.1.9.2.3 Mode 1 (SM with non-adaptive precodder, vertical encoding) | Support of MIMO mode 1 | 16.3.10.2.1 |
|  |  |  | [Support of 1Tx] | 16.3.10.2.1 |
| 4.1.XX DL Multi-BS MIMO |  |  |  |  |
| 4.1.YY UL Multi-BS MIMO |  |  |  |  |
| 5.1.8 Bandwidth Request (BR) and Allocation | 5.1.8.1 Contention-based random access BR procedure |  |  |  |
|  |  |  | Support of 3-step BR procedure | 16.2.11.1.1 |
|  |  |  | Support of 5-step BR procedure | 16.2.11.1.1 |
|  |  |  | Support of BR ACK A-MAP IE | 16.2.11.1.1 |
|  |  |  | Support of predefined BR index | 16.2.11.1.5.1 |
|  |  |  | Support for multiple BR ACK MAP IE in a frame | 16.2.11.1.1 |
|  | 5.1.8.2 Standalone BR |  | Support of Standalone BR Header | 16.2.11.1.2 |
|  | 5.1.8.4 BR using Fast Feedback Channel |  | Support of Indication flag feedback | 16.2.11.1.4.1 |
| 5.1.9 Service Flow Management |  |  |  |  |
| 5.1.10 Quality of Service | 5.1.10.1 Support of Legacy scheduling services |  | Support of legacy scheduling services | 16.2.12.2 |
|  | 5.1.10.2 Support of Adaptive granting and polling service (aGPS) |  | Support of QoS mapping during Handover | 16.2.12.3.1.1 |
| 5.1.1 PHS |  |  |  |  |
| 5.1.2 CS Options and functions |  |  |  |  |
| 5.1.3 MAC Header formats |  |  |  |  |
| 5.1.4 Construction and Re-assembly of MAC PDU |  |  |  |  |
| 5.1.5 MAC control messages |  |  |  |  |
| 5.1.6 ARQ |  |  | ARQ implementation | 16.2.13 |
|  |  |  | Cumulative ARQ feedback | 16.2.13.2.1,16.2.13.5.3.2 |
|  |  |  | Selective ARQ feedback | 16.2.13.2.1,16.2.13.5.3.2 |
|  |  |  | ARQ feedback polling mechanism | 16.2.13.2.2 |
|  |  |  | Unsolicited bandwidth allocation for ARQ feedback |  |
|  |  |  | ARQ triggering conditions | 16.2.13.2.3 |
|  |  |  | ARQ rearrangement | 16.2.13.1.2 |
|  |  |  | HARQ local NAK in the ARQ tx. state machine | 16.2.13.5.2.2 |
|  |  |  | ARQ purging | 16.2.13.5.3.4 |
|  |  |  | ARQ discard | 16.2.13.5.3.3 |
|  |  |  | ARQ Synch loss | 16.2.13.5.5 |
|  |  |  | ARQ Reset | 16.2.13.5.4 |
|  |  |  | ARQ buffer management | 16.2.13.5.6 |
|  |  |  | ARQ Error detection | 16.2.13.5.3.2 |
|  |  |  | SDU in-order delivery | 16.2.13.5.3.5 |
|  |  |  | Piggybacked ARQ Feedback | 16.2.13.2.1 |
|  |  |  | ARQ Feedback using control message | 16.2.13.2.1 |
| 5.1.7 MAC support for H-ARQ |  |  | H-ARQ Support | 16.2.14 |
|  |  |  | Capability of DL HARQ channels Number negotiation | 16.2.14.2.1.1 |
|  |  |  | Capability of UL HARQ channels Number negotiation | 16.2.14.2.1.2 |
|  |  |  | Supported HARQ ACK delay in DL transmission | 16.2.14.2.1.1 |
|  |  |  | Supported HARQ ACK delay in UL transmission | 16.2.14.2.1.2 |
|  |  |  | Combined Feedback scheme for ROHC and HARQ |  |
| 5.1.8 Reliability of MAC control message |  |  |  |  |
| 5.1.9 Emergency Service Support |  |  |  |  |
| 5.1.10 L2 transfer message |  |  |  |  |
| 5.1.11 GRA |  |  |  |  |
| 5.1.4 Network Entry | 5.1.4.1 MS scanning and synchronization |  | Support for MS scanning and synchronization | 16.2.15.1, 16.2.8.2.3 |
|  | 5.1.4.2 MS DL/UL parameter acquisition |  | Support for DL/UL parameter acquisition | 16.2.15.2 |
|  | 5.1.4.3 MS initial ranging |  | Contention-based initial ranging and automatic adjustments | 16.2.15.3 |
|  | 5.1.4.4 MS basic capability negotiation |  | MS capability negotiation | 16.2.15.4 |
|  | 5.1.4.5 MS authorization and key exchange |  | MS authentication/authorization and key exchange | 16.2.15.5 |
|  | 5.1.4.6 MS capability exchange and registration |  | capability exchange and registration | 16.2.15.6, 16.2.8.2.3 |
|  |  |  | CRID assignment during network entry | 16.2.15.6 |
|  | 5.1.4.7 Network entry with multi-carrier supports |  |  |  |
|  | 5.1.4.8 IP address acquistion |  |  |  |
| 5.1.15 Sleep Mode |  |  |  |  |
|  | 5.1.15.1 Sleep Mode Initiation |  |  |  |
|  | 5.1.15.2 Sleep Cycle Operation |  |  |  |
|  | 5.1.15.3 Listening Windows operations |  |  |  |
|  |  |  | Error handling for the lost AAI-TRF-IND | 16.2.17.2.3.1 |
|  |  |  | ACK of SCH during Sleep Mode | 16.2.2.1.3.4 16.2.2.2.5 16.2.17.2.4 |
|  | 5.1.15.4 Sleep Cycle setting update |  |  |  |
|  | 5.1.15.5 Interruption to Sleep Mode |  |  |  |
|  | 5.1.15.6 CQI operation during Sleep Mode |  |  |  |
|  | 5.1.15.7 Sleep mode termination |  |  |  |
| 5.1.16 Idle Mode |  |  |  |  |
|  | 5.1.16.1 Paging group assignment |  |  |  |
|  | 5.1.16.2 Idle Mode Initiation |  | Maintain MS' connection information at BS with Management Resource Holding Timer during Idle Mode initiation | 16.2.18.1.1 |
|  |  |  | Support of De-registration ID | 16.2.3.21 |
|  | 5.1.16.3 Location Update |  | Paging group location update without PG_LU-timer | 16.2.18.4.1.1 |
|  |  |  | Power down location update | 16.2.18.4.1.3 |
|  | 5.1.16.4 Paging operation |  |  |  |
|  | 5.1.16.5 Services in idle mode |  |  |  |
|  | 5.1.16.6 Expedited NE from Idle Mode |  |  |  |
| 5.1.17 DCR Mode | 5.1.17.1 DCR mode initiation |  |  |  |
|  | 5.1.17.2 DCR mode operation |  |  |  |
|  | 5.1.17.3 DCR mode termination |  |  |  |
|  | 5.1.17.4 Network re-entry from DCR mode |  |  |  |
| 5.1.21 Co-located Coexistence |  |  |  |  |
|  | 5.1.21.1 CLC mode activation |  |  |  |
|  | 5.1.21.2 CLC mode operation |  |  |  |
|  | 5.1.21.3 CLC mode deactivation |  |  |  |
| 5.1.22 Power Management in Connected mode |  |  |  |  |
| 5.1.23. Coverage Loss |  |  |  |  |
| 5.1.18 Security features | 5.1.18.1 Authorization Policy Support |  | 802.16m "EAP based" authorization support | 16.2.3.4 |
|  | 5.1.18.2 PKM Version 3 Support |  | PMK derivation and management | 16.2.5.2.1.1.1 |
|  |  |  | AK derivation and management | 16.2.5.2.1.1.2 |
|  |  |  | CMAC key derivation management | 16.2.5.2.1.1.3 |
|  |  |  | TEK derivation and management | 16.2.5.2.1.1.4 |
|  |  |  | Key agreement procedure | 16.2.5.2.1.4 |
|  |  |  | ICV size neogotiation during key agreement in network entry | 16.2.3.42.4 |
|  |  |  | Support of CCM SA | 16.2.5.2.2 |
|  |  |  | Support of CTR SA | 16.2.5.2.2 |
|  |  |  | Key update during Lzone-to-Mzone zone switch | 16.2.5.2.1.5.6 |
|  |  |  | Key update during Mzone-to-Lzone zone switch | 16.2.5.2.1.5.7 |
|  |  |  | Key update during Handover | 16.2.5.2.1.5.4 |
|  |  |  | Key usage during Location Update and Network reentry from Idle mode | 16.2.5.2.1.5.5 |
|  | 5.1.18.3 cryptographic suites |  | No data encryption, no data authentication | 16.2.5.2.2 |
|  |  |  | CCM-Mode 128-bit AES | 16.2.5.2.3.1.1 |
|  |  |  | CTR Mode 128-bit AES, no data authentication | 16.2.5.2.3.1.2 |
|  |  |  | CMAC calculation | 16.2.5.2.3.2 |
|  | 5.1.18.4 Message Encryption/Authentication |  | Support of signaling messages integrity protection | 16.2.5.3.3 |
|  |  |  | Support of signaling messages confidentiality and integrity protection | 16.2.5.3.3 |
|  | 5.1.18.5 MS privacy |  | Support of MS privacy | 16.2.5.3 |
| 5.1.11 Neighbor Advertisement |  |  |  | 16.2.6.1 |
|  | 5.1.11.1 AAI_NBR-ADV |  |  | 16.2.3.12 |
|  |  |  | Broadcast transmission | 16.2.6.1.1 |
|  | Note is needed |  | Capability of encoding full SFH SPs information | 16.2.3.12 |
|  |  |  | Capability of encoding delta SFH information | 16.2.3.12 |
|  |  |  | Support of broadcasting different NBR-ADV segments independently | 16.2.6.1.1 |
|  | 5.1.11.2 AAI_MC-ADV |  |  |  |
|  |  |  | Support of broadcasting the available carrier information | 16.2.8.2.3.1 |
| 5.1.12 Scanning | 5.1.12.1 Scanning initiation |  | AMS initiated Scanning (SCN-REQ) | 16.2.6.1.2 |
|  |  |  | ABS initiated Scanning (SCN-RSP) | 16.2.6.1.2 |
|  |  |  | Event Triggered Scan request | 16.2.6.1.2 |
|  | 5.1.12.2 Scan Reporting Type Support |  | Periodic scan report | 16.2.6.2 |
|  |  |  | Event triggered scan report | 16.2.6.2 |
| 5.1.13 HO Trigger | 5.1.13.1 Trigger Metrics |  | BS CINR mean | 16.2.6.2 |
|  |  |  | BS RSSI mean | 16.2.6.2 |
|  |  |  | Number of missed frames | 16.2.6.2 |
|  | 5.1.13.2 Complex Trigger conditions |  |  |  |
| 5.1.14 HO Procedure | 5.1.14.1 Generic HO features |  | Support of HO Re-entry Mode 0: General HO support | 16.2.6.3.2 |
|  |  |  | HO initiated by AMS | 16.2.6.3.2 |
|  |  |  | HO initiated by ABS | 16.2.6.3.2 |
|  |  |  | Preallocation of STID | 16.2.6.3.3 |
|  |  |  | HO rejection | 16.2.6.3.4 |
|  |  |  | Capability of omitting CDMA HO ranging | 16.2.6.3.5.1 |
|  |  |  | Contention based CDMA HO Ranging | 16.2.6.3.5.1 |
|  |  |  | Dynamic ranging opportunity allocation in HO-CMD | 16.2.6.3.5.1 |
|  |  |  | Dedicated ranging code allocation in HO-CMD | 16.2.3.11 |
|  |  |  | Send AAI_HO-IND in case of All target ABSs in AAI_HO-CMD are unreachable. | 16.2.6.3.4 |
|  |  |  | Send AAI_HO-IND in case of HO cancel without target ABS SFH mismatch | 16.2.6.3.4 |
|  |  |  | Omit AAI_SBC-REQ/RSP management messages during reentry processing | 16.2.3.11 (no text description) |
|  |  |  | Omit PKM Authentication phase | 16.2.3.11 (no text description) |
|  |  |  | Seamless HO (Allowing data communications before completion of AAI_RNG-REQ/RSP transaction) | 16.2.3.11 (no text description) |
|  |  |  | Omit AAI_REG-REQ/RSP message and higher layer protocol triggering (for IP address refresh) during reentry processing. | 16.2.3.11 (no text description) |
|  |  |  | Full service and operational state transfer or sharing between Serving BS and Target BS | 16.2.3.11 |
|  | 5.1.14.2 Entry-Before-Break HO (HO Re-entry Mode 1) |  |  |  |
|  | 5.1.14.3 HO support from WirelessMAN-OFDMA Reference System to Advanced WirelessMAN-OFDMA System |  | Indication support of deployed system version by MAC version | 16.2.6.4.1.1.1 |
|  |  |  | Support of Zone Switch from LZone to MZone | 16.2.6.4.1.2.1 |
|  |  |  | BS initiated Zone Switch | 16.2.6.4.1.2.1 |
|  |  |  | Switch to Mzone after network entry to Lzone | 16.2.6.4.1.2.1 |
|  |  |  | Support of Zone Switch with Zone Swith Mode 0: Breaks communcation in LZone before performing network reentry in Mzone | 16.2.6.4.1.2.1 |
|  |  |  | Context Mapping during Zone Switch | 16.2.6.4.1.3 |
|  | 5.1.14.4 HO support from Advanced WirelessMAN-OFDMA System to WirelessMAN-OFDMA Reference System |  | HO from MZone in mixed-mode ABS to YBS | 16.2.6.4.2 |
|  |  |  | HO from AAI-only ABS to YBS | 16.2.6.4.2.4 |
|  | 5.1.14.5 InterRAT HO |  | Inter-RAT HO support using other RAT's message using L2 transfer message | 16.2.6.5.2.1 |

------------------------------------------------- End of proposed text I --------------------------------------------------