| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Definitions of “capability class 0” and parameters for 802.16m PHY system |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Sassan Ahmadi Shaocheng Wang Intel | E-mail: Sassan.Ahmadi@intel.com Shaocheng.wang@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to propose definitions of “capability class 0” and parameters for 802.16m PHY system |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Definitions of “capability class 0” and parameters for 802.16m PHY system

- Sassan Ahmadi, Shaocheng Wang

- Intel

- Introduction

This contribution is intended to provide definitions for “capability class 0” and parameters for 802.16m PHY system.

- Proposed text

II.1 Proposed text 1

[Note to the editor]: Add the following text to the end of Ln 19, page 388. (section 16.2.15.4)

------------------------------------------------- Start of proposed text I--------------------------------------------------

The content of “Capability Class 0” is defined in Appendix X of this specification.

------------------------------------------------- End of proposed text I --------------------------------------------------

II.1 Proposed text 2

[Note to the editor: create a new appendix and insert the following table

------------------------------------------------- Start of proposed text I--------------------------------------------------

Table xxx—Default capability class and parameters for 802.16m PHY system

| Feature | Feature Subset1 | Feature Subset2 | Description | Standard reference (802.16m/D6) |
| --- | --- | --- | --- | --- |
| 4.1.3 DL PHY Structure | 4.1.3.x Subframe type support |  | PRU for different subframe types (Ref: added Frame_Structure reference later) |  |
| 4.1.3 DL PHY Structure | 4.1.3.1 Frequency partitioning |  | Reuse-1 with one freq partition (FP0, FP1, FP2, FP3) =(1:0:0:0) |  |
|  |  |  | Reuse-1 with one freq partition for SFH only | 16.3.6.2.1 |
|  | 4.1.3.2 DL subchannelization and resource unit |  | Distributed LRU (DLRU) | 16.3.5.1.1 |
|  |  |  | A-MAP logical resource unit (MLRU) | 16.3.5.3.4 |
|  |  |  | Subchannelization for SFH | 16.3.6.2.1 |
|  | 4.1.3.3 Pilot pattern |  |  |  |
|  |  |  | 2 stream pilot patterns for DLRU & CLRU with interlaced pilots | 16.3.5.4.1, 16.3.5.4.3 |
|  | 4.1.3.4 MIMO midamble |  | Support of MIMO Midamble | 16.3.5.4.2 |
| 4.1.4 UL PHY Structure | 4.1.4.1 Frequency partitioning |  | Reuse-1 with one freq partition (FP0, FP1, FP2, FP3) =(1:0:0:0) |  |
|  | 4.1.4.2 UL subchannelization and resource unit |  | Distributed LRU (DLRU) | 16.3.8.1.1 |
|  |  |  | Subband Contiguous LRU (SLRU) | 16.3.8.1.2 |
|  |  |  | UL Control Channels - Bandwidth request channels | 16.3.8.3.3.1 |
|  |  |  | UL Control Channels - HARQ feedback (ACK/NACK) channels | 16.3.8.3.3.2 |
|  |  |  | UL Control Channels - synchronized ranging channels | 16.3.9.1.4 |
|  |  |  | UL Control Channels - asynchronized ranging channels | 16.3.9.1.4 |
|  | 4.1.4.3 Pilot pattern |  | 1 stream pilot patterns for DLRU & CLRU | 16.3.8.4 |
| 4.1.10 Channel Coding | 4.1.10.1 Data Channel | 4.1.10.1.1 Encoder | Data Burst CRC encoding | 16.3.11.1.1 |
|  |  |  | Data Burst partition process | 16.3.11.1.2 |
|  |  |  | FEC Block randomization | 16.3.11.1.3 |
|  |  |  | FEC Block CRC encoding | 16.3.11.1.4 |
|  |  |  | FEC Encoding | 16.3.11.1.5 |
|  |  |  | Bit selection and repetition | 16.3.11.1.6 |
|  |  |  | Bit Collection | 16.3.11.1.7 |
|  |  | 4.1.10.1.2 Modulation | DL QPSK | 8.4.9.4.2 |
|  |  |  | DL 16QAM | 8.4.9.4.2 |
|  |  |  | UL QPSK | 8.4.9.4.2 |
|  |  |  | UL 16QAM | 8.4.9.4.2 |
|  | 4.1.10.2. Control Channel | 4.1.10.2.1 Encoder | FEC Encoding (TBCC) | 16.3.11.2.1 |
|  |  |  | Bit Separation | 16.3.11.2.2 |
|  |  |  | subblock interleaver | 16.3.11.2.3 |
|  |  |  | Bit grouping | 16.3.11.2.4 |
|  |  |  | Bit Selection | 16.3.11.2.5 |
|  | 4.1.10.3 HARQ | 4.1.10.3.1 IR HARQ | HARQ Redundancy selection - DL | 16.3.11.4.1 |
|  |  |  | HARQ Redundancy selection - UL | 16.3.11.4.1 |
|  |  | 4.1.10.3.2 Constellation Rearrangement | Constellation rearrangement version (MIMO stream = 1) | 16.3.11.4.2 |
|  |  |  | Constellation rearrangement version (MIMO stream > 1) | 16.3.11.4.2 |
|  |  |  | DL CoRe version signaling | 16.3.11.4.2 |
|  |  |  | UL CoRe version signaling | 16.3.11.4.2 |
|  | 4.1.10.4 Subcarrier Randomization |  | PRBS for subcarrier randomization | 16.3.11.3.1 |
|  |  |  | Data subcarrier randomization | 16.3.11.3.2 |
|  |  |  | Pilot subcarrier randomization | 16.3.11.3.3 |
| 4.1.6 DL Control Structure | 4.1.6.1 Advanced Preamble (A-Preamble) | 4.1.6.1.1 Primary Advanced Preamble (PA-Preamble) | Primary A-Preamble | 16.3.6.1.1 |
|  |  | 4.1.6.1.2 Secondary Advanced Preamble (SA-Preamble) | Secondary A-Preamble | 16.3.6.1.2 |
|  | 4.1.6.2 Superframe Header (SFH) | 4.1.6.2.1 Primary Superframe Header (P-SFH) | Tx period (every superframe) | 16.3.6.2.1.1 |
|  |  |  | P-SFH code rate | 16.3.6.3.1.1 |
|  |  | 4.1.6.2.2 Secondary Superframe Header (S-SFH) | S-SFH code rate | 16.3.6.3.1.2 |
|  |  |  | Tx period of SP1 IE | 16.3.6.2.1.2 |
|  |  |  | Tx period of SP2 IE | 16.3.6.2.1.2 |
|  |  |  | Tx period of SP3 IE | 16.3.6.2.1.2 |
|  | 4.1.6.3 Advanced MAP (A-MAP) | 4.1.6.3.1 Location of A-MAP Region for FFR | Non-user specific, HARQ feedback, Power control A-MAPs in the reuse-1 freq partition | 16.3.6.2.2 |
|  |  | 4.1.6.3.2 A-MAP Transmission/Reception | Support of Non-User Specific A-MAP IE | 16.3.6.5.2.1 |
|  |  |  | Support of DL Basic Assignment A-MAP IE | 16.3.6.5.2.4.1 |
|  |  |  | Support of UL Basic Assignment A-MAP IE | 16.3.6.5.2.4.2 |
|  |  |  | Support of Broadcast Assignment A-MAP IE | 16.3.6.5.2.4.13 |
|  |  |  | Assignment A-MAP IE, Code rate | 16.3.6.3.2.4 |
|  |  |  | Support of HARQ Feedback A-MAP IE | 16.3.6.5.2.2 |
|  |  |  | Support of Power Control A-MAP IE | 16.3.6.5.2.3 |
|  |  | 4.1.6.3.5 HARQ Feedback A-MAP Resource Index | HARQ Feedback A-MAP Index Parameter with STID | 16.3.6.3.2.2 |
| 4.1.7 UL Control Strcuture | 4.1.7.1 UL control channel for FFR | 4.1.7.1.1 Location of UL control channel for FFR | UL fast feedback, HARQ feedback, Bandwidth request channel in the reuse-1 freq partition | 16.3.8.3.3 |
|  | 4.1.7.2 Primary Fast Feedback Channel | 4.1.7.2.1 Physical Channel Structure |  |  |
|  |  | 4.1.7.2.2 Information Content | Reference to 4.1.8.3 |  |
|  | 4.1.7.3 Secondary Fast Feedback Channel | 4.1.7.3.1 Physical Channel Structure |  |  |
|  |  | 4.1.7.3.2 Information Content | Reference to 4.1.8.3 |  |
|  | 4.1.7.4 HARQ Feedback Channel | 4.1.7.4.1 Physical Channel Structure | Coding, modulation and subcarrier mapping of HARQ feedback channel | 16.3.9.2.2 |
|  |  | 4.1.7.4.2 Information Content | HARQ ACK/NACK Feedback | 16.3.9.2.2 |
|  | 4.1.7.7 Ranging Channel for Synchronized AMSs | 4.1.7.7.1 Physical Channel Structure | Synchronized ranging physical channel structure in 16m-only mode | 16.3.9.1.4.2 |
|  | 4.1.7.8 Bandwidth Request Channel | 4.1.7.8.1 Physical Channel Structure | Coding, modulation and subcarrier mapping of bandwidth request channel preamble part | 16.3.9.2.5 |
|  |  | 4.1.7.8.2 Information Content | Support of preamble only (5 step) | 16.2.11.1.1 |
| 4.1.X Control Mechanism | 4.1.X.2 Uplink Power Control | 4.1.X.2.1 Uplink Power control | Support of uplink power control | 16.3.9.4 |
|  |  | 4.1.X.2.2 Control Channel TX PC correction by PC-A-MAP | Support of PC-A-MAP | 16.3.9.4 |
|  |  | 4.1.X.2.3 MS TX Status Report | Support of Period-based report | 16.3.9.4 |
|  |  |  | Support of event driven report | 16.3.9.4 |
|  |  |  | Support of ULPC Parameters Updating Indicator | 16.2.2.1.3.6 |
|  |  |  | Support of NI broadcast by AAI_ULPC_NI message | 16.2.3.31 |
|  |  |  | Support of power control parameter broadcast by AAI_SCD message | 16.2.3.30 |
|  |  |  | Support of power control offset unicast by AAI_UL POWER ADJ | 16.2.3.32 |
|  | 4.1.X.3 Ranging | 4.1.X.3.1 Non-dynamic ranging | Support of non-dynamic ranging | 16.3.6.5.2.4.13 |
| 4.1.x A-MAP and HARQ operation | 4.1.x.2 A-MAP relevance and HARQ timing | 4.1.x.2.1 Processing Time | TUL_Rx_Processing | 16.2.14.2.1 |
|  |  |  | TUL_Tx_Processing | 16.2.14.2.1 |
|  |  |  | TDL_Rx_Processing | 16.2.14.2.1 |
|  |  | 4.1.x.2.2 A-MAP relevance | DL Default TTI A-MAP Relevance | 16.2.14.2.2 |
|  |  |  | UL Default TTI A-MAP Relevance | 16.2.14.2.2 |
|  |  | 4.1.x.2.3 HARQ Feedback delay | DL Default TTI HARQ Feedback Delay | 16.2.14.2.2 |
|  |  |  | UL Default TTI HARQ Feedback Delay | 16.2.14.2.2 |
|  | 4.1.x.3 HARQ | 4.1.x.3.1 Support for Downlink HARQ Incremental Redundancy | DL maximum retransmission number (DL N_MAX_ReTx) | 16.2.14.2.1.1 |
|  |  |  | DL retransmission max interval (T_ReTx_Interval) | 16.2.14.2.1.1 |
|  |  | 4.1.x.3.2 Support for Uplink HARQ Incremental Redundancy | UL maximum retransmission number (UL N_MAX_ReTx) | 16.2.14.2.1.2 |
|  | 4.1.x.6. S-SFH Update | 4.1.x.6.1 S-SFH Update procedure |  | 16.2.24 |
| 4.1.8 DL MIMO | 4.1.8.1 DL MIMO General | 4.1.8.1.1 Number of Tx antenna | 2 Transmit antennas | 16.3.7.2.5.5 |
|  |  | 4.1.8.1.2 Number of streams for SU-MIMO | Support of 2 streams | 16.3.7.1.3, 16.3.7.2.4 |
|  | 4.1.8.2 DL MIMO mode | 4.1.8.2.2 Mode 0 (TxD, SFBC, non-adaptive precoder, vertical encoding) | Support of MIMO Mode 0 | 16.3.7.2.1 |
|  |  |  | OL SU-MIMO, DLRU, outside OL Region | 16.3.7.2.1 |
| 4.1.9 UL MIMO | 4.1.9.1 UL MIMO General | 4.1.9.1.1 Number of Tx antenna | 1 Transmit antennas | 16.3.10.1.1 |
|  |  | 4.1.9.1.2 Number of streams for SU-MIMO | Support of 1 stream | 16.3.10.1.3, 16.3.10.2.4 |
|  | 4.1.9.2 UL MIMO mode | 4.1.9.2.3 Mode 1 (SM with non-adaptive precodder, vertical encoding) | Support of MIMO mode 1 | 16.3.10.2.1 |

------------------------------------------------- End of proposed text I --------------------------------------------------