| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Definitions of “capability class 0” and parameters for 802.16m system |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Sassan Ahmadi Shaocheng Wang Intel Youngkyo Baek Samsung Electronics | E-mail: Sassan.Ahmadi@intel.com Shaocheng.wang@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to propose definitions of “capability class 0” and parameters for 802.16m PHY system |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Definitions of “capability class 0” and parameters for 802.16m system

- Sassan Ahmadi, Shaocheng Wang

- Intel

- Youngkyo Baek

- Samsung Electronics

- Introduction

This contribution is intended to provide definitions for “capability class 0” and parameters for 802.16m PHY system.

- Proposed text

II.1 Proposed text 1

[Note to the editor]: Add the following text to the end of Ln 41, page 419. (section 16.2.15.4)

------------------------------------------------- Start of proposed text I--------------------------------------------------

The content of “Capability Class 0” is defined in Appendix X of this specification.

------------------------------------------------- End of proposed text I --------------------------------------------------

II.1 Proposed text 2

[Note to the editor: create a new appendix and insert the following two tables

------------------------------------------------- Start of proposed text II--------------------------------------------------

Table xxx—Default capability class and parameters for 802.16m PHY system

| Feature | Feature Subset1 | Feature Subset2 | Description | Standard reference (802.16m/D8) |
| --- | --- | --- | --- | --- |
| 8.4.9.4.2 Channel Coding | Data Channel | Modulation | DL QPSK | 8.4.9.4.2 |
|  |  |  | DL 16QAM | 8.4.9.4.2 |
|  |  |  | UL QPSK | 8.4.9.4.2 |
|  |  |  | UL 16QAM | 8.4.9.4.2 |

Table xxx—Default capability class and parameters for 802.16m MAC system

| Feature | Feature Subset1 | Feature Subset2 | Description | Standard reference (802.16m/D8) |
| --- | --- | --- | --- | --- |
| 5.2 CS Options and functions | Support of IPCS |  | Support of IPCS | 5.2.5 |
| 16.2.3.47 Service Flow Management | a. Service flow addition |  | Support of service flow addition (BS initiated) | 16.2.3.47 |
|  | b. Service flow change |  | Support of service flow change (BS initiated) |  |
|  | c. Service flow release |  | Support of service flow release (BS initiated) |  |
| 16.2.6 HO | a. scanning | i) Scanning initiation | AMS initiated Scanning (SCN-REQ) | 16.2.6.1.2 |
|  |  |  | ABS initiated Scanning (SCN-RSP) | 16.2.6.1.2 |
|  |  |  | Event Triggered Scan request | 16.2.6.1.2 |
|  |  | ii) Scan Reporting Type Support | Periodic scan report | 16.2.6.2 |
|  |  |  | Event triggered scan report | 16.2.6.2 |
|  | b. HO Trigger | i) Trigger Metrics | BS CINR mean | 16.2.6.2 |
|  |  |  | BS RSSI mean | 16.2.6.2 |
| 16.2.11 Bandwidth Request (BR) and Allocation | Standalone BR |  | Support of Standalone BR Header | 16.2.11.1.2 |
| 16.2.12 Quality of Service | Support of Legacy scheduling services |  | Support of legacy scheduling services | 16.2.12.2 |
| 16.2.22 Reliability of MAC control message | Support of message ACK | Support of AAI_MSG-ACK as message ACK | Support of message ACK using AAI_MSG-ACK | 16.2.22 |

------------------------------------------------- End of proposed text II --------------------------------------------------