| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Suggested Table Format for 802.16m MAC Control Message |  |
| Date Submitted | 2010-07-15 |  |
| Source(s) | Kelvin Chou, I-Kang Fu, Yih-Shen Chen, Alex Hsu, Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | P802.16m/D6 |  |
| Abstract | Suggested Table Format for 802.16m MAC Control Message |  |
| Purpose | For TGm discussion and adoption into P802.16m/D7 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Suggested Table Format for 802.16m MAC Control Message

Kelvin Chou, I-Kang Fu, Yih-Shen Chen, Alex Hsu, Paul Cheng

MediaTek Inc.

Table XXX—AAI_XXX-XXX MAC Control Message Format

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| MAC Control Message Type | 8 |  |  |
| Global carrier configuration change count | 3 | Indicates the value of Global carrier configuration change count of the network, will be incremented each time the contents of this message change. Value range is 1-7 |  |
| Number of Carrier Groups | 4 | Groups of contiguous carriers. In case of FDD, only DL carrier groups are included. |  |
| For (i=0; i< Number of Carrier Groups; i++){ |  |  |  |
| Duplexing Mode | 1 | “0” for TDD “1” for FDD |  |
| Multi-Carrier Configuration Index Across the Network | 6 | Index associated to Table 803 |  |
| Start Frequency | 18 | Center frequency (in unit of 50kHz) of the carrier located at lowest frequency position in the i-th carrier group |  |
| Number of Carriers | 3 |  |  |
| For(j=0;j<Number of Carriers; j++) { |  |  |  |
| Physical Carrier Index | 3 | Index of the physical carrier |  |
| Carrier Type | 1 | Indicates the type of this physical carrier. 0b0: Fully configured carrier 0b1: Partially configured carrier | Presented when Duplexing Mode = 1 |
| Reference Carrier Indicator | 1 | 0b0: Non-reference carrier 0b1: Reference carrier of current carrier group |  |
| } |  |  |  |
| } |  |  |  |

---------------------------------------------------------Start of the Text-----------------------------------------------------------