| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | BR Index Assignment (16.2.12) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI | jscha@etri.re.kr, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE Sponsor Ballot Recirculation #1 on P802.16m/D7 |  |
| Abstract | This contribution provides a Predefined BR Index Assignment Method. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

BR Index Assignment (16.2.12)

Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Introduction

During connection establishment procedure, an ABS may assign predefined BR index for 3-step BR. However, the current draft does not describe how to assign the BR index in detail except one restriction, which is “If BR action is 0b10 (BR), ABS shall assign different BR index to service flows whose UL Grant Scheduling Type is different”.

Let’s consider the following two scenarios, which follow the assignment rule for predefined BR index defined in the current draft.

| Scenario1 | Service flow 1 | Service flow 2 |
| --- | --- | --- |
|  | - UL Scheduling Type: aGP - BR Index: 1 - BR action: 0b01 (switch to primary QoS parameters) | - UL Scheduling Type: aGP - BR Index: 1 - BR action: 0b01 (switch to primary QoS parameters) |
| Scenario2 | Service flow 1 | Service flow 2 |
|  | - UL Scheduling Type: nrtPS - BR Index: 2 - BR action: 0b10 (BR) - BR Size: 30 bytes | - UL Scheduling Type: nrtPS - BR Index: 2 - BR action: 0b10 (BR) - BR Size: 60 bytes |

In case of scenario 1, the same BR index is assigned to two aGP service flows. Note that this assignment does not violate the rule because BR action is not set to 0b10 and UL Scheduling Types of two service flows are same each other. In this case, when an ABS receives a quick access message with BR index set to 1, the ABS does not know which service flow needs QoS set switch.

In scenario 2, the same BR index is assigned to two nrtPS service flows, which also keeps the assignment rule because UL Scheduling Types of two service flows are same each other. If an ABS receives a quick access message with BR index set to 2, then how much bandwidth the ABS should allocate? 30 bytes or 60 bytes?

In order to avoid such ambiguity, the following restrictions are needed

- If BR Action is set to 0b00 or 0b01 (ertPS or aGP), then the same BR Index shall not be assigned to different service flows.  BR Index per ertPS or aGP service flow

- If BR Action is set to 0b10 (BR), then ABS shall assign different BR index to service flows whose UL Grant Scheduling Type is different and shall not assign the same BR index with different BR Size to different service flows whose UL Scheduling Type is same  BR Index set per UL Scheduling Type (BE, nrtPS, etc)

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify Table 786 as follows]

Table 786 – Predefined BR Index Parameters

| M/O | SyntaxAttributes / Array of attributes | Size (bit) | Value / Notes | Conditions |
| --- | --- | --- | --- | --- |
|  | Predefined BR Index Parameter () { |  |  |  |
| O | Predefined BR Index array [0..15] |  |  | May be present if 3-step BR is supported by AMS and ABS |
|  | Number of predefined BR Indexes (K) | 4 | 0-15 |  |
|  | For (i=1 to K+1) { |  |  |  |
| M | A. Predefined BR Index | 4 | If BR Action is 0b00 or 0b01, the same BR Index shall not be assigned to different service flows. If BR action is 0b10 (BR), ABS shall assign different BR index to service flows whose UL Grant Scheduling Type is different and shall not assign the same BR index with different BR Size to different service flows whose UL Scheduling Type is same. | N.A. |
| M | B. BR Action | 2 | 0b00: ertPS service flow requests to resume to maximum sustained rate 0b01: aGP service flow requests to switch to Primary QoS parameters 0b10: BR 0b11: reserved | N.A. |
|  | If (BR Action == 0b10) { |  |  |  |
| O | C. BR size | 11 | Number of bytes requested by the predefined BR index | Present when BR Action == 0b10 |
|  | } |  |  |  |
|  | } |  |  |  |
|  | } |  |  |  |

[Modify texts on page 188, line 21 as follows]

Following parameters may be included in the AAI_DSA-REQ message.

  - SCID: To switch sleep cycle setting

  - Predefined BR index parameters: Predefined BR index parameters define the mapping from predefined BR index(es) to BR action and BR size, which is used in 3-step Bandwidth Request procedure, and are only included in ABS initiated DSA-REQ. They are flow specific and are determined based on the QoS parameters of the service flow in the AAI-DSx messages. If BR Action is 0b00 or 0b01, the same BR Index shall not be assigned to different service flows. If BR action is 0b10 (BR), ABS shall assign different BR index to service flows whose UL Grant Scheduling Type is different and shall not assign the same BR index with different BR Size to different service flows whose UL Scheduling Type is same.

[Modify texts on page 202, line 40 as follows]

Following parameters may be included in the AAI_DSC-REQ message.

  - SCID: To switch sleep cycle setting

  - Predefined BR index parameters: Predefined BR index parameters define the mapping from predefined BR index(es) to BR action and BR size, which is used in 3-step Bandwidth Request procedure, and are only included in ABS initiated DSC-REQ. They are flow specific and determined based on the QoS parameters of the service flow in the AAI-DSx messages. If BR Action is 0b00 or 0b01, the same BR Index shall not be assigned to different service flows. If BR action is 0b10 (BR), ABS shall assign different BR index to service flows whose UL Grant Scheduling Type is different and shall not assign the same BR index with different BR Size to different service flows whose UL Scheduling Type is same.

---------------------------------------------------------End of the Text----------------------------------------------------------