| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of AAI_NBR-ADV Message (16.2.3) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI | jscha@etri.re.kr, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE Sponsor Ballot Recirculation #1 on P802.16m/D7 |  |
| Abstract | This contribution provides a cleanup of AAI_NBR-ADV Message. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of AAI_NBR-ADV Message (16.2.3)

Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify Table 692 as follows]

Table 692 – AAI_NBR-ADV parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| M | D.4) S-SFH change count | 4 | Indicates the value of S-SFH change count of this neighbor ABS |  |
| O | D.5) Paging carrier indication | 1 | Indicates whether the carrier is a paging carrier in ABS or not 0: no paging carrier 1: paging carrier | May be present when multiple carrier operation is applied. |
| M | D.6) PGID | 16 | Indicates paging group identifier where the carrier belongs | Shall be included for each carrier of each neighbor ABS |
| … | … | … | … | … |
| O | H) SFH Subpkt 3 | variable | - | Present if SFH_encoding_format is set to 0b00 |
| O | I) Delta Information | Variable | Indicates the delta encoding, between the reference carrier and the current carrier where this message is transmitted if SFH_encoding formatSFH_encoding_format = 0b01, or between the reference carrier and the preceding carrier if SFH_encoding_format = 0b11. Delta information contains SFH SP1, SP2, and SP3 attributes as defined in Table 840837, Table 841838, and table 842839, respectively. Each delta information attribute is optional. | Present if SFH_encoding_format is set to 0b01 or 0b11 |
| O | J) Neighbor-specific trigger | Variable | Optional neighbor-specific triggers with encoding defined in Table 772 – Trigger description | May be included for each neighbor ABS |
| O | NBR-ADV offset | 4 | The frame number offset the MOB_NBR-ADV is sent from the start point of frame counting. Unit: Frame. | May be included to indicate the existence of neighbor R1 BS or LZone of ABS. |
| O | NBR-ADV interval | 4 | The interval of MOB_NBR-ADV is sent in LZone. Unit: Frame. | May be included to indicate the existence of neighbor R1 BS or LZone of ABS. |
| O | R1 NBR ABS per segment array [1..255] | Variable | The list of release 1 neighbor ABS in a segment | Present if R1 NBR ABS list is available when NBR-ADV offset and NBR-ADV interval are present. |
| M | A) Neighbor BSID | 48 | Neighbor Base Station ID |  |
| … | … | … | … | … |
| O | IDCell range end | 10 |  | For ABS type whose system info are not included in AAI_NBR-ADV |
| O | Neighbor-specific trigger | Var. | Optional neighbor-specific triggers with encoding defined in Table 761 – Trigger description | May be included for each neighbor BS |
| O | NBR-ADV offset | 4 | The frame number offset the MOB_NBR-ADV is sent from the start point of frame counting. Unit: Frame. | It shall be included when the existence of neighbor R1 BS or LZone of ABS are indicated in AAI_NBR-ADV |
| O | NBR-ADV interval | 4 | The interval of MOB_NBR-ADV is sent in LZone. Unit: Frame. | It shall be included when the existence of neighbor R1 BS or LZone of ABS are indicated in AAI_NBR-ADV |
| O | paging carrier indication | 1 | Indicates whether the carrier is a paging carrier in ABS or not 0: no paging carrier 1: paging carrier | May be present when multiple carrier operation is applied. |
| M | PGID | 16 | Indicates paging group identifier where the carrier belongs | Shall be included for each carrier of each neighbor BS |
| O | Serving ABS LZone Preamble Index | 7 | Serving ABS LZone Preamble Index for AMS to synchronize with Serving ABS LZone to receive MOB_NBR-ADV | It shall be included if NBR-ADV offset and NBR-ADV interval are present. |
| … | … | … | … | … |

---------------------------------------------------------End of the Text----------------------------------------------------------