| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of MAEH Format (16.2.2) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI | jscha@etri.re.kr, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE Sponsor Ballot Recirculation #1 on P802.16m/D7 |  |
| Abstract | This contribution clarifies the MAEH format. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of MAEH Format (16.2.2)

Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify Section 16.2.2.2.5 as follows]

16.2.2.2.5 MAC Control ACK Extended Header (MAEH)

The MAEH format is defined in Table 672. This header may be used by ABS and AMS to indicate the reception of a specific, previously received MAC control message, BR without STID header (see 16.2.2.1.3.2). When an ABS or AMS receives a MAC control message or MAC control message fragment with the Polling bit set to 1 in the MCEH, the ABS or AMS shall transmit an MAEH or AAI_MSG-ACK message as an acknowledgement after receiving the complete message with the SN of the MAC control message PDU or the SN of the last received fragment if fragmented.

Table 672 – MAEH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MAEH () { |  |  |
| Type | 4 | Extended header type = 0b0100 (MAEH type) |
| MAEH sub-type | 1 | 0b0: Indicates reception of a control message 0b1: Indicates reception of a Service Specific Scheduling Control header |
| If (MAEH sub-type == 0b0) { |  |  |
| ACK_SN | 68 | SN retrieved from the MCEH of the MAC PDU with the Polling bit set to 1 |
| Control Connection Channel ID (CCC ID) | 1 | Control Connection Channel ID (CCC ID) that the MAC control message is received. |
| } |  |  |
| Reserved | Variable3 | For byte alignment |
| } |  |  |

[Modify Table 719 as follows]

Table 719 – AAI_MSG-ACK

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | ACK_SN | 68 | SN retrieved from MCEH of the received MAD PDU. | N.A. |
| M | CCC ID | 1 | Control Connection Channel ID (CCC ID) that the MAC management message is received. | N.A. |

---------------------------------------------------------End of the Text----------------------------------------------------------