| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup of AAI_HO-REQ Message (16.2.3) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI | jscha@etri.re.kr, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE Sponsor Ballot Recirculation #1 on P802.16m/D7 |  |
| Abstract | This contribution provides a cleanup version of AAI_HO-REQ Message. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Cleanup of AAI_HO-REQ Message (16.2.3)

Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify Table 690 as follows]

Table 690 – AAI_HO-REQ parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | AAI_NBR-ADV Change Count | 8 | AAI_NBR-ADV change count last received from the serving ABS | Shall be included if this message contains one or more recommended ABSs identified by using Neighbor_ABS_Index.when N_New_ABS_Index > 0 |
| O | Recommended ABS array [0..63] | Variable | The list of recommended ABS recommended for HO | Present if there is one or more recommended ABSs for HO |
| O | A) Neighbor_ABS_Index | 8 | ABS index of the recommended ABS assigned in the last received corresponds to AAI_NBR-ADV | Shall be included for each listed ABS which is presented Present if the recommended ABS is presented in the AAI_NBR-ADV message. |
| O | B) Neighbor_ABS_ID | 48 | The 48 bits neighbor ABSID | Shall be included for each listed ABS which is absent Present if the recommended ABS is absent in the AAI_NBR-ADV message. |
| O | C) ABS CINR mean | 8 | CINR mean of the new ABS | May be included for each listed ABS. |
| O | D) ABS RSSI mean | 8 | RSSI mean for the new ABS | May be included for each listed ABS. |
| O | E) Physical_Carrier_Index | 6 | Physical carrier index of the ABS | May be included if the ABS is multi-carrier ABS. |
| O | Carrier Preassignment Indication | 1 | Indicates whether AMS needs pre-assignment of secondary carriers at the Target ABS. | May be included when AMS supports MC mode = 0b010 or 0b100 |
| O | Physical_Carrier_Index | 6 | Physical carrier index of the ABS | May be included for each ABS with ABS Index or ABS ID when the ABS is multi-carrier ABS. |

---------------------------------------------------------End of the Text----------------------------------------------------------