| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean up for MAC control messages (16.2.3) |  |
| Date Submitted | 2010-08-11 |  |
| Source(s) | Chia-Lung Tsai, Ping-Heng Kuo, Rong-Terng Juang, Yu-Tao Hsieh, Pang-An Ting ITRI | E-mail: ythsieh@itri.org.tw |
| Re: | Comments on IEEE P802.16m/D7 for IEEE 802.16 WG SB_16m |  |
| Abstract | This contribution proposes renew text for relay support. |  |
| Purpose | Discussion and adoption by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean up for MAC control messages (16.2.3)

Chia-Lung Tsai, Ping-Heng Kuo, Rong-Terng Juang, Yu-Tao Hsieh, Pang-An Ting

ITRI

Introduction

Table 759 have been modified by two different contributions at the same time: C80216m-10_0848 (option 1) and C80216m-10_0870 (option 2). Consequently, the bit number and the descriptions of some fields in the table are not consistent. Therefore, in this contribution, we provide both option 1 and option 2 in the proposed text #1, either one can be used to solve the inconsistency issues. On the other hand, proposed text #2 is suggested to correct some typos.

Proposed text

Black text: text in the subclause 16.2.3 of P802.16m/D7

Red Text: delete text

Blue text: new text

---------------------------------------------------------Start of the Text #1--------------------------------------------------------

[Note to Editor – Modify the text in section 16.2.3.57 AAI_Global-CFG Message in page 225 as follows]

Option 1:

Table 759- AAI_Global-CFG MAC Control Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| MAC Control Message Type | 8 |  |
| Global Carrier Configuration Change Count | 3 | Indicates the value of Global carrier configuration change count of the network, will be incremented each time the contents of this message change. Value range is 1-7 |
| Number of Carrier Groups | 4 | Groups of contiguous carriers In case of FDD, only DL carrier groups are included. |
| For (i=0; i< Number of Carrier Groups; i++){ |  |  |
| Duplexing Mode | 1 | “0” for TDD “1” for FDD |
| Number of Carriers | 3 4 | Number of component carriers in this configuration. Value Range is 1(0x0)~16 (0xF) |
| If(Number of Carriers ==0x0){ |  |  |
| Frequency Assignment Index | 8 | Frequency Assignment Index of the carrier |
| Physical Carrier Index | 4 6 | Index of the physical carrier |
| If(Duplexing_Mode==1){ |  |  |
| Carrier Type | 1 | Indicates the type of this physical carrier. 0b0: Fully configured carrier 0b1: Partially configured carrier |
| } |  |  |
| }else { |  |  |
| Configuration Type | 1 | 0b0 : All carriers have same bandwidth in the carrier group 0b1: Multiple bandwidths are mixed for the carrier group |
| If(Configuration Type ==0b0){ |  | Single bandwidth in the carrier group |
| Bandwidth | 3 | Bandwidth of the component carrier 0b000 : 5MHz 0b001 : 10MHz 0b010 : 20MHz 0b011 : 7MHz 0b100 : 8.75MHz 0b101~: reserved |
| } |  |  |
| Start Frequency Start Frequency Assignment Index | 18 8 | Center frequency (in unit of 50 kHz) of the carrier located at lowest frequency position in the i-th carrier group Frequency Assignment Index of the first carrier in the i-th carrier group |
| Number of Carriers | 3 |  |
| For(j=0;j<Number of Carriers; j++) { |  |  |
| Physical Carrier Index | 4 6 | Index of the physical carrier |
| If(Configuration Type ==0b1){ |  | Multiple bandwidth in this group |
| Bandwidth | 6 2 | Bandwidth of the component carrier 0b00 : 5MHz 0b01 : 10MHz 0b10 : 20MHz 0b11 : reserved |
| } |  |  |
| If(Duplexing_Mode==1){ |  |  |
| Carrier Type | 1 | Indicates the type of this physical carrier. 0b0: Fully configured carrier 0b1: Partially configured carrier |
| } |  |  |
| Center Frequency Shift | 3 | The value of center frequency shift of the current carrier {0, 1.5625, 3.125, 4.6875, 0, -1.5625, -3.125, -4.6875}kHz |
| } |  |  |
| } |  |  |

[Note to Editor – Modify the text in line#60, page 929 as follows]

-- AAI_Global-ConfigCFG message

-- Global Configuration Message

AAI-Global- ConfigCFG ::= SEQUENCE {

}

Option 2:

16.2.3.57 AAI_Global- CFGonfig (global carrier configuration) Message

The AAI_Global- CFGonfig message provides the carrier information for all available carriers in the network. The ABS transmits the AAI_Global- CFGonfig message to an AMS after transmitting the AAI_REG-RSP message.

Table 759- AAI_Global-CFGonfig MAC Control Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| MAC Control Message Type | 8 |  |
| Global Carrier Configuration Change Count | 3 | Indicates the value of Global carrier configuration change count of the network, will be incremented each time the contents of this message change. Value range is 1-7 |
| Number of Carrier Groups | 4 | Groups of contiguous carriers In case of FDD, only DL carrier groups are included. |
| For (i=0; i< Number of Carrier Groups; i++){ |  |  |
| Duplexing Mode | 1 | “0” for TDD “1” for FDD |
| Multi-Carrier Configuration Index Across the Network | 6 | Index associated to Table 803 |
| Number of Carriers | 3 | Number of component carriers in this configuration. Value Range is 1(0x0)~16 (0xF) |
| If(Number of Carriers ==0x0){ |  |  |
| Frequency Assignment Index | 8 | Frequency Assignment Index of the carrier |
| Physical Carrier Index | 4 | Index of the physical carrier |
| If(Duplexing_Mode==1){ |  |  |
| Carrier Type | 1 | Indicates the type of this physical carrier. 0b0: Fully configured carrier 0b1: Partially configured carrier |
| } |  |  |
| }else { |  |  |
| Configuration Type | 1 | 0b0 : All carriers have same bandwidth in the carrier group 0b1: Multiple bandwidths are mixed for the carrier group |
| If(Configuration Type ==0b0){ |  | Single bandwidth in the carrier group |
| Bandwidth | 3 | Bandwidth of the component carrier 0b000 : 5MHz 0b001 : 10MHz 0b010 : 20MHz 0b011 : 7MHz 0b100 : 8.75MHz 0b101~: reserved |
| } |  |  |
| Start Frequency | 18 | Center frequency (in unit of 50 kHz) of the carrier located at lowest frequency position in the i-th carrier group |
| Number of Carriers | 3 |  |
| For(j=0;j<Number of Carriers; j++) { |  |  |
| Physical Carrier Index | 4 3 | Index of the physical carrier |
| If(Configuration Type ==0b1){ |  | Multiple bandwidth in this group |
| Bandwidth | 6 | Bandwidth of the component carrier 0b00 : 5MHz 0b01 : 10MHz 0b10 : 20MHz 0b11 : reserved |
| } |  |  |
| If(Duplexing_Mode==1){ |  |  |
| Carrier Type | 1 | Indicates the type of this physical carrier. 0b0: Fully configured carrier 0b1: Partially configured carrier |
| } |  |  |
| Center Frequency Shift | 3 | The value of center frequency shift of the current carrier {0, 1.5625, 3.125, 4.6875, 0, -1.5625, -3.125, -4.6875}kHz |
| Reference Carrier Indicator | 1 | 0b0: Non-reference carrier 0b1: Reference carrier of current carrier group |
| } |  |  |
| } |  |  |

---------------------------------------------------------End of the Text #1---------------------------------------------------------

---------------------------------------------------------Start of the Text #2--------------------------------------------------------

[Note to Editor – Modify the text in section 16.2.3.58 AAI_ARS-CONFIG-CMD message format in page 227 as follows]

Table 760- AAI_ARS-CONFIG-CMD message format

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | 16m_DL_Relay_zone_duration |  | 3 | The duration of the 16m DL Relay zone in units of AAI subframes. It has 3 bits to represent the values {1, 2, 3, 4, 5. 6, 7, 8} | N.A. |
| M | 16m_UL_Relay_zone_duration |  | 3 | The duration of the 16m UL Relay zone in units of AAI subframes. It has 3 bits to represent the values {1, 2, 3, 4, 5. 6, 7, 8} | N.A. |
| M | 16m_Relay_zone_AMS_allocation_indicator |  | 1 | The indicator which signalizes if the ABS may allocate resources to the AMS in the 16m DL Relay zone and 16m UL Relay zone. It has 1 bit to represent the values {0, 1}. If it is set to 1 then the AMSs are allocated in Relay zones | N.A. |
| M | MIMO Midamble indication in AAI DL Relay zone |  | 1 | 0b0: MIMO midamble is not transmitted in AAI DL Relay zone 0b1: MIMO midamble is transmitted in AAI DL Relay zone | N.A. |
| O | 16m_DL_Relay_zone_Cell_Specific_Parameters | R_DCASSB0SB0 | 5/4/3 | See 16.6.3.3.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits | Present if 16m_Relay_zone_AMS_allocation_indicator == 0 |
| O | 16m_DL_Relay_zone_Cell_Specific_Parameters | R_DCASMB0MB0 | 5/4/3 | See 16.6.3.3.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits | Present if 16m_Relay_zone_AMS_allocation_indicator == 0 |
| O | 16m_DL_Relay_zone_Cell_Specific_Parameters | R_DCASi i | 3/2/1 | See 16.6.3.3.1 Cell-specific resource mapping For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit | Present if 16m_Relay_zone_AMS_allocation_indicator == 0 |
| O | 16m_UL_Relay_zone_Cell_Specific_Parameters | R_UCASSB0SB0 | 5/4/3 | See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits | Present if 16m_Relay_zone_AMS_allocation_indicator == 0 |
| O | 16m_UL_Relay_zone_Cell_Specific_Parameters | R_UCASMB0MB0 | 5/4/3 | See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits | Present if 16m_Relay_zone_AMS_allocation_indicator == 0 |
| O | 16m_UL_Relay_zone_Cell_Specific_Parameters | R_UCASi i | 3/2/1 | See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit | Present if 16m_Relay_zone_AMS_allocation_indicator == 0 |

---------------------------------------------------------End of the Text #2---------------------------------------------------------

References

[1] IEEE P802.16m/D7, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”, July 2010.