| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed MAC Control Signaling Header for Inviting AMS to Conduct Periodic Ranging in the Coverage Loss Detection Procedure(16.26.1) |  |
| Date Submitted | 2010-08-11 |  |
| Source(s) | Lei Wang InterDigital Communications, LLC | Voice : +1 858 205-7286 E-mail: leiw@billeigean.com |
| Re: | IEEE Standards Sponsor Ballot Recirculation-1 for P802.16m/D7. |  |
| Abstract | The contribution proposes a MAC control signaling header to be used by the ABS to invite an AMS to conduct periodic ranging in the coverage detection procedure (16.26.1). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed MAC Control Signaling Header for Inviting AMS to Conduct Periodic Ranging in the Coverage Loss Detection Procedure (16.26.1)

Lei Wang

InterDigital Communications, LLC

- Introduction

In the 802.16m/D7, there are multiple questions/issues around the usage of AAI_RNG-RSP message in subsection 16.2.26.1, e.g.,

- Is the 1-bit "Ranging Request bit" field the only information needed to be included in the AAI_RNG-RSP for this coverage loss detection usage?  if so, why do we need such a complicated message to carry 1-bit information? if not, then what are the other field that are needed?

- The unsolicited AAI_RNG-RSP usage is not specified in the definition of AAI_RNG-RSP in section 16.2.3.2, where it actually says AAI_RNG-RSP shall be sent as a response to AAI_RNG-REQ;

- When the ABS invites the AMS to do periodic ranging, the ABS actually knows the AMS's ID. If the ABS can keep the knowledge of the AMS's ID info during this coverage loss detection required periodic ranging process, then the steps for AMS to send its ID info after a successful periodic ranging can be saved.

This contribution is intended to address the above questions/issues.

- Discussions

We propose to design a separate MAC control signaling header for the ABS to invite an AMS to conduct periodic ranging in the coverage loss detection procedure, based on the following considerations:

- In current 16m spec [2], the AAI_RNG-RSP message is already a heavily-loaded message. For the purpose of inviting an AMS to conduct periodic ranging in the coverage loss detection procedure, only an 1-bit information field is used in the current design.

- The AAI_RNG-RSP is designed as a response of ABS when receiving an AAI_RNG-REQ message.

- Considering MAC control message vs. MAC control signaling header, we propose to use MAC control signaling header, because the size of the periodic ranging invitation signaling information is small and no encryption or MAC control ACK is needed. Note that the periodic ranging invitation is used when the ABS failed to receive the UL burst in a unicast UL allocation allocated to the AMS during the coverage loss detection procedure.

In addition, we propose that the ABS can optionally provide a dedicated periodic ranging code in given periodic ranging channel or channels, such that the AMS ID information can be kept known to the ABS. In this way, after a successful periodic ranging, the AMS does not need to send its ID information to the ABS in the coverage loss detection procedure, thus resulting in an overhead reduction.

- Suggested changes in the 802.16m/D7

The following is the proposed change in the 802.16m/D7. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1: line 29, page 55

Insert the following subsection:

16.2.2.1.3.9  Periodic Ranging Invitation (PR-IVT) MAC Signaling Header

The format of the Periodic Ranging Invitation (PR-IVT) MAC signaling header is defined in Table 664a. This MAC signaling header is used by the ABS to invite an AMS to conduct periodic ranging as needed in the coverage loss detection procedure. When the dedicated periodic ranging is provided to the AMS, the AMS shall use the given dedicated periodic ranging code in the given periodic ranging channel to conduct periodic ranging.

Table 664a – Periodic Ranging Invitation (PR-IVT) Signaling Header Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| PR-IVT Signaling header() { |  |  |
| FID | 4 | Flow Identifier for MAC signaling header. Set to 0b0010. |
| Type | 5 | MAC signaling header type = 0b01000. |
| Length | 3 | Indicates the length in bytes of this MAC signaling header. |
| Dedicated PR flag | 1 | Indicates whether the dedicated Periodic Ranging (PR) opportunity is provided. 0b0: dedicated PR not provided 0b1: dedicated PR provided |
| If (Dedicated PR flag ==1) { |  |  |
| PR code | 6 | Indicates the dedicated PR code assigned to the AMS for this invited periodic ranging process. |
| Number of PR channels (n) | 2 | Indicate that the above given dedicated PR code is assigned to the AMS in the next (n+1)-th PR channel. |
| } |  |  |
| reserved | 3 | For byte alignment |
| } |  |  |

Suggested change #2: line 25, page 46, Table 656

Change the last row in Table 656 as follows:

| Type Field (5 bits) | MAC Signaling Header Type |
| --- | --- |
| 01000 - 11111 | Reserved Periodic Ranging Invitation (PR-IVT) |
| 01001 - 11111 | Reserved |

Suggested change #3: line 47, page 433

Change the paragraph in line 47 on page 433 as follows:

The ABS shall send a Periodic Ranging Invitation (PR-IVT) MAC signaling header unsolicited AAI_RNG-RSP message to request the AMS to perform ranging using periodic ranging codes by setting the Ranging Request bit to one and start T58 timer. If the dedicated periodic ranging code and channel are provided in the PR-IVT MAC signaling header, upon sending a RNG-ACK with “success” status to the AMS, the ABS shall restart the active_ABS_timer and stop T58 timer. Otherwise, upon Upon receiving an AAI-RNG-CFM message with AMS’s STID, which indicates a successful periodic ranging initiated by this PR-IVT MAC signaling header, unsolicited AAI-RNG-RSP, the ABS shall restart the active_ABS_timer and stop T58 timer. If the ABS does not receive the AAI_RNG-CFM message upon Upon the expiration of T58, the ABS shall start the Resource Retain Time.

Suggested change #4: line 20, page 434

Change the paragraph in line 20 on page 434 as follows:

The AMS upon receiving a Periodic Ranging Invitation (PR-IVT) MAC signaling header an unsolicited AAI_RNG-RSP message with Ranging Request bit to be one which indicates that the ABS requests ranging procedure to detect coverage loss, the AMS shall perform periodic ranging. If the dedicated periodic ranging code and channel are provided in the PR-IVT MAC signaling header, the AMS shall send the give periodic ranging code in the give periodic ranging channel; otherwise the AMS shall send a randomly chosen periodic ranging code in the next available periodic ranging channel. Upon successful ranging initiated by a Periodic Ranging Invitation (PR-IVT) MAC signaling header an unsolicited AAI_RNG-RSP message with Ranging Request bit to be one, indicated by receiving an AAI_RNG-ACK message with a success status and corresponding periodic ranging code and channel, if a randomly chosen periodic ranging code was used, the AMS shall request bandwidth and send back a AAI_RNG-CFM message that includes its STID. Upon exhausted HARQ retransmissions of the AAI_RNG-CFM message, the AMS considers that it is not connected with the ABS anymore, and the AMS shall perform coverage loss recovery procedure as indicated in Section 16.2.26.3.

Suggested change #5: line 1, page 725

Change the paragraph in line 1 on page 725 as follows:

rs0 is the start root index. Ms is the number of cyclic shift per ZC root index and defined by Ms = 1/G . Ncont is the number of periodic RP codes per sector (0~NPE -1) which is defined by Table 920 for contention-based approach, and Ndedi is the total number of dedicated periodic RP codes (Ncont~ Ncont+Ndedi-1) per sector where maximum possible Ndedi per sector is 32. For femtocell, Ncont is the total number of initial (0~NIN-1), handover (NIN~NIN+NHO-1) and periodic RP codes (NIN + NHO ~ NIN + NHO + NPE - 1) per sector which is defined by Table 921 for contention-based approach. Ndedi is the total number of dedicated handover and periodic ranging RP codes (Ncont~ Ncont+Ndedi-1) per sector where maximum possible Ndedi per sector is 32, where the femto ABS assigns the dedicated PR codes for HO and periodic ranging in a non-overlapping way .

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D7, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”