| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On S-SFH SP1 IE Femto Indicator (16.3.5.5.1.2, Table 837) |  |
| Date Submitted | 2010-08-16 |  |
| Source(s) | Ying Li, Sudhir Ramakrishna, Kaushik Josiam , Zhouyue Pi, Anshuman Nigam, Jung Je Son, Young Bo Cho Samsung Electronics Pei-Kai Liao MediaTek Inc. | Phone: +1-972-761-7903 E-mail: yli2@sta.samung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> pk.liao@mediatek.com |
| Re: | IEEE 802.16m D6 change requests. |  |
| Abstract | This contribution is to propose changes in Table 840 to save the overhead of S-SFH SP1. |  |
| Purpose | To be discussed and adopted by WG SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# On S-SFH SP1 IE Femto Indicator (16.3.5.5.1.2, Table 837)

Ying Li, Sudhir Ramakrishna, Kaushik Josiam,

Zhouyue Pi, Anshuman Nigam, Jung Je Son

Samsung Electronics

Pei-Kai Liao

MediaTek Inc

# Introduction

- In the current draft D7, it uses a bit ‘femto indicator’ in S-SFH SP1 IE, for the purpose of indicating different ranging configuration of femto and other types of the cells.

- We think the key reason to use different ranging configuration is because of the different size or Tx power of the cell. Not only femtocells, but also other small-sized cells can use the same ranging configuration as femto, different from macrocell.

- Hence we want to change the femto indicator to the indication of ranging configuration.

- Make the ranging configuration more flexibly related to the cell size, not just the type of the cell.

# Proposed Text

Please revise Table 837 in D7 as follows.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.3.5.5.1.2 S-SFH IE

[Modify texts as suggested below: Page 553, Line 47]

Table 837—S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| MSB of superframe number | 8 | Remaining bit of SFN except LSB of SFN in P-SFH |
| …… |  |  |
| ABS EIRP | 6 | Unsigned integer from 1 to 64 in units of 1 dBm, where 000000=1 dBm and 111111=64 dBm. |
| …… |  |  |
| Femto Indicator Ranging configuration type | 1 | Indicates whether the ABS is a Femto ABS. 0b0: ABS not a Femto ABS. 0b1: ABS is a femto ABS. Indicates the ranging configuration type. 0b0: ranging configuration type being the one for Femto ABS 0b1: ranging configuration type being the one for Macro ABS |
| …… |  |  |
| if(Femtocell Ranging configuration type ==0b0) { |  | for AAI Femtocell ranging configuration type being the one for Femto ABS |
| Allocation periodicity of the S-RCH | 2 | Indicates the periodicity of the S-RCH allocation according to the Table 926. |
| …… |  |  |
| } else { |  |  |
| Allocation periodicity of the NS-RCH | 2 | Indicates the periodicity of the NS-RCH allocation according to the Table 923. |
| …… |  |  |
| NS-RCH formats | 1 | Indicates the NS-RCH formats number of Table 916 |
| } |  |  |
| ……. |  |  |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------