| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the Minimal HO Reentry Interleaving Interval (16.2.3.7) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Kelvin Chou MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D7) |  |
| Abstract | Clarification on the Minimal HO Reentry Interleaving Interval in AAI_REG-REQ |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_RNG-REQ Message and ASN.1 Clean Up (16.2.3.1)

Kelvin Chou

MediaTek Inc.

Introduction

The parameter Minimal HO Reentry Interleaving Interval in AAI_REG-REQ message needs to be clarified:

1. Minimal HO Reentry Interleaving Interval is counted in unit of frame and hence one bit is not enough. Suggest to use at least 2 bits (up to four frames)

2. Minimal HO Reentry Interleaving Interval cannot be 0 if AMS only support carrier switching.

Proposed Text

Remedy 1:

In the P802.16m/D7, page 97, line 57, modify Table 686 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 686—AAI_REG-REQ message Field Description

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | O.1) Minimal HO Reentry Interleaving Interval | 12 | The minimal HO_Reentry_Interleaving_Interval measured in frames. For MC HO capable AMS capable of Carrier Aggregation (Multicarrier Capability = 0b010 or 0b100), this value shall be 0. Shall be included only if EBB support is set to 1 | Present if needed |

--------------------------------------------------  Text End  -----------------------------------------------------------------