| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to subband index of RCH (16.3.8.2.4) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Chiwoo Lim, Jeongho Park, Jaeyun Ko, Hwasun Yoo, Jaeweon Cho, Hokyu Choi, Heewon Kang Samsung Electronics | chiwoo.lim@samsung.com |
| Re: | Sponsor ballot comment on P802.16m/D7 Target topic: “IEEE P802.16m/D7, section 16.3.8.2.4”. |  |
| Abstract | The contribution proposes to change the subband index of RCH of the 802.16m draft amendment, IEEE P802.16m/D7. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to subband index of RCH (16.3.8.2.4)

Chiwoo Lim, Jeongho Park, Jaeyun Ko, Hwasun Yoo, Jaeweon Cho, Hokyu Choi, Heewon Kang
Samsung Electronics

- Introduction

The contribution proposes to change the subband index of RCH in the 802.16m amendment. In the current draft amendment [1], the subband index of RCH (ISB) is calculated by the IDcell and the allocated number of subbands YSB according to the following equation.

ISB = mod (IDcell, YSB)

Where YSB = ∑LSB-CRU,FPi /4, i=0,1,2,3. LSB-CRU,FPi is the number of allocated subband CRUs in each frequency partition i.

The problem is current ISB can indicate the non-power boosted reuse 3 frequency partition when UL FFR is applied. We need to clarify this problem.

- References

[1] IEEE P802.16m/D7, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, July 2010

- Proposed changes for 802.16m draft amendment

--------------------------------------------------- Text Start ---------------------------------------------------

[Remedy-1: Modify the texts on page 722, line 63 ~ page 723, line 11 as following]

The information for ranging frequency resource allocation, i.e., the subband index for ranging resource allocation is determined by the IDcell and the allocated number of subbands YRSB according to the Equation (283), where IDcell is defined in 16.3.5.1.2 and YRSB is LSB-CRU,FPi /4, where is defined in 16.3.5.5.2.4.3 with exception of LSB-CRU,FPi  is the number of allocated subband CRUs in 16.3.7.3 and FPi denotes reuse 1 partition or power-boosted reuse 3 partition. The frequency partition where the ranging channel is located is indicated by the ABS through S-SFH SP1 IE.

ISB = mod (IDcell, YRSB)                            (283)

where ISB denotes the subband index (0, …, YRSB -1) for ranging resource allocation among YRSB subbands.

[Remedy-2: Modify the text on page 725, line 63~ page 726, line 47 as following]

The information for ranging frequency resource allocation, i.e., the subband index for ranging resource allocation is determined by the IDcell and the allocated number of subbands YRSB according to the Equation (286), where IDcell is defined in 16.3.5.1.2 and YRSB is LSB-CRU,FPi /4 is defined in 16.3.5.5.2.4.3 except in the UL, where LSB-CRU,FPi is the number of allocated subband CRUs in 16.3.7.3 and FPi denotes reuse 1 partition or power-boosted reuse 3 partition. The frequency partition where the ranging channel is located is indicated by the ABS through S-SFH SP1 IE.

ISB = mod (IDcell+1, YRSB)                            (286)

where ISB denotes the subband index (0, …, YRSB -1) for ranging resource allocation among YRSB subbands.

--------------------------------------------------- Text End ---------------------------------------------------