| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_SCD Configuration Change Count in AAI_E-MBS-CFG (16.9.3.1) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D7,” in response to the IEEE 802.16 Working Group Sponsor Ballot Recirc #1 |  |
| Abstract | E-MBS Configuration applying indication in AAI_SCD supporting E-MBS on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# AAI_SCD Configuration Change Count in AAI_E-MBS-CFG (16.9.3.1)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Instructions

IEEE 802.16m Amendment Draft Standard [1] describes the Enhanced Multicast Broadcast Service (E-MBS).

According to the IEEE 802.16m Amendment Draft Standard [1], some E-MBS Configuration Parameters transmitted via AAI_SCD message have been proposed as follows.

- Zone_Allocation-Bitmap

- ZF

- MSI Length(NMSI)

- E-MBS AAI frame offset

Although those parameters are essential for the E-MBS operation, any E-MBS AMSs should receive the AAI_E-MBS-CFG message. Furthermore, whenever any parameter of them in AAI_SCD changes, some parameters (e.g., E-MBS MAP Resource Index) in AAI_E-MBS-CFG may change. Therefore, the AMS should wait until the LIFETIME expires. The interval between the time applying the configuration from AAI_SCD and upcoming AAI_E-MBS-CFG to update makes complicated operation for AMSs due to deciding one of either previous AAI_SCD or new AAI_SCD during the interval.

Therefore, applying the configuration from AAI_SCD should be defined clearly and some operations are proposed in [2][3].

- AAI_SCD update indicator in E-MBS MAP [2]

- AAI_SCD update indicator in AAI_E-MBS-CFG [3]

Either [2] or [3] may be an example to update AAI_SCD.

However, AAI_SCD update indicator in E-MBS MAP should be careful due to AAI_SCD is cell-specific information.

AAI_SCD update indicator in AAI_E-MBS may be reasonable but still needs to be modified to resolve the precise applying and the AAI_SCD update should be predicted in the previous time transmit AAI_E-MBS-CFG message.

In this contribution, we propose AAI_SCD configuration change count in AAI_E-MBS-CFG message to resolve the ambiguity which AAI_SCD and when the changed configuration of AAI_SCD.

# References

- IEEE P802.16m/D7, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” July 2010

- IEEE C802.16m-10/0793, Proposed Text for updating E-MBS configuration parameter in AAI_SCD message (16.3.6.2.3), July 2010

- IEEE C802.16m-10/0898, E-MBS Related SCD information update indication and action time, July 2010

# Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy: Adapt the following change in 16.9.3.1 of 802.16m/D7]

16.9.3.1 E-MBS Configuration Indicators

The information regarding E-MBS configuration is transmitted periodically to the AMSs interested in E-MBS using a MAC control message called the AAI-E-MBS_CFG message. The E-MBS configuration indicators specify the resources reserved for E-MBS traffic in the downlink physical resources and additional information necessary for E-MBS operation.

The E-MBS configuration information necessary for E-MBS operation include E-MBS-Zone_IDs of serving and neighboring ABSs, E-MBS MAP time offset (frequency offset), E-MBS MAP resource allocation, E-MBS MAP Isize-offset, E-MBS MAP MIMO Mode and E-MBS ID and FID mappings between serving E-MBS Zone and neighboring E-MBS Zone for the same content. AAI_E-MBS-CFG shall be advertised in the superframe before the beginning of the largest MSI (16 superframes)

An AMS movinges across cell boundary is expected to update the AAI_E-MBS-CFG message from the target ABS.

To apply the updated AAI_SCD when any of E-MBS configuration information of AAI_SCD changes, the AMS, receiving changed AAI_SCD, checks the Applied AAI_SCD Configuration Change Count of every AAI_E-MBS-CFG until it is the same as Configuration Change Count of AAI_SCD.

Table 979—AAI_E-MBS-CFG message Format

| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | AAI_E-MBS-CFG | N.A. |
| M | E-MBS_CFG_LIFETIME (m) | 4 | Indicates the duration of E-MBS_CFG_LIFETIME for which the E-MBS configuration information of the zone do not change. AMS shall decode the E-MBS configuration message when its Nsuperframe meets the following condition. Nsuperframe modulo 16(m+1) == 0 |  |
| M | Applied AAI_SCD Configuration Change Count | 4 | To adapt AAI_SCD whose Configuration Change Count |  |
| M | fullNumEMBSZoneID[1..8] |  |  |  |
| M | A) E-MBS_Zone_ID | 7 | The E-MBS_Zone_ID to which this E-MBS MAP applies. |  |
| M | B) E-MBS MAP Resource Index | 11 | Resource index includes location and allocation size. |  |
| M | C) E-MBS MAP ISizeOffset | 5 | Offset used to compute burst size of E-MBS MAP |  |
| O | D) E-MBS ID and FID Mappings List |  |  | Present when the serving ABS is in the zone boundary and to support inter-zone service continuity |
| M | D.1) Num_Neighbor_E-MBS_Zones | 3 | Number of neighbor E-MBS Zones |  |
| M | D.2) AAI_NBR-ADV Change Count | 3 | Indicates the value of AAI_NBR-ADV change count |  |
| O | D.3) fullNumNeighborEMBSZones[1°K…8] |  |  | Present when the Num_Neighbor_E-MBS_Zones > 0 |
| M | D.3.a) Neighbor_E-MBS_Zone_ID | 7 | Neighbor E-MBS Zone ID |  |
| O | D.3.b) Physical Carrier Index | 6 | Target carrier which the AMS switches or is redirected by ABS to | Present only if the neighbor E-MBS Zone ID is served on a different carrier from the previous one at the neighbor ABS |
| M | D.3.c) Num_Neighbor_ABS | 8 | Number of Neighbor ABSs |  |
| O | D.3.d) fullNumNeighborABS[1°K…256] |  |  | Present when the Num_Neighbor_ABS > 0 |
| M | D.3.d.1) ABSIndex | 8 | Index of Neighbor ABS |  |
| M | D.3.e) Number of E-MBS_FID Mappings (M) | 4 | Number of E-MBS ID_FID Mappings between serving E-MBS Zone and neighbor E-MBS Zone |  |
| O | D.3.f) fullNumEMBSFidMappings[1..16] |  |  | Present when the Number of E-MBS_FID Mappings > 0 |
| M | D.3.f.1) Current_E-MBS ID and FID and New_E-MBS_ID and FID | Variable (32xM) | Mapping of current E-MBS ID and FID and new E-MBS ID and FID between serving E-MBS Zone and neighbor E-MBS Zone. Based on the value of Number of E-MBS ID_FID Mappings |  |

Applied AAI_SCD Configuration Change Count: Applied Configuration Change Count consists of the change count of AAI_SCD message for the unicast/multicast zone. If an AMS receives AAI_SCD with new configuration count (n+1) but the applied AAI_SCD Configuration Change Count(n), the AMS should preserve the previous configuration of AAI_SCD till receives the same applied AAI_SCD Configuration Change Count(n+1) in order to adapt the configuration of AAI_SCD as well as AAI_E-MBS-CFG message.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]