| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to description of subcarrier alignment for multi-carrier operation(16.2.3.57) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Soojung Jung, Eunkyung Kim, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI | sjjung@etri.re.kr |
| Re: | IEEE Sponsor Ballot Recirculation #1 on P802.16m/D7 |  |
| Abstract | This contribution provides to clean up the texts for subcarrier alignment. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to description of subcarrier alignment for multicarrier operation (16.2.3.57)

Soojung Jung, Eunkyung Kim, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Introduction

According to the section 16.3.3.6.2, the AMS can obtain the frequency offset Δfc to be applied over based on reference carrier indicator and multicarrier configuration index and Table 803.

However, in current draft (D7), the frequency offset Δfc and bandwidth of each carrier are provided through the AAI_Global-CFG message. And reference carrier indicator is not provided through the AAI_Global-CFG message.

Therefore, this contribution proposes to clean up the descriptions that are used to derive the information for sub-carrier alignment.

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify Table 759 as follows]

Table 759 – AAI_Global-CFG MAC Control Message Format

| Feild | Size(bit) | Description |
| --- | --- | --- |
| MAC Control Message Type | 8 |  |
| Global carrier configuration change count | 3 | Indicates the value of Global carrier configuration change count of the network, will be incremented each time the contents of this message change. Value range is 1-7 |
| Number of Carrier Groups | 4 | Groups of contiguous carriers In case of FDD, only DL carrier groups are included |
| For(i=0; i<Number of Carrier Groups; i++){ |  |  |
| Duplexing Mode | 1 | “0” for TDD “1” for FDD |
| Number of Carriers | 43 | Number of component carriers in this configuration. Value Range is 1(0x0) ~ 16(0xF) |
| If(Number of Carriers == 0x0){ |  |  |
| Frequency Assignment Index | 8 | Frequency Assignment Index of the carrier |
| Physical Carrier Index | 4 | Index of the physical carrier |
| If(Duplexing_Mode ==1){ |  |  |
| Carrier Type | 1 | Indicates the type of this physical carrier. 0b0: Fully configured carrier 0b1: Partially configured carrier |
| } |  |  |
| }else{ |  |  |
| Configuration Type | 1 | 0b0: All carriers have same bandwidth in the carrier group 0b1: Multiple bandwidths are mixed for the carrier group |
| If(Configuration Type == 0b0){ |  | Single bandwidth in the carrier group |
| Bandwidth | 3 | Bandwidth of the component carrier 0b000: 5MHz 0b001: 10MHz 0b010: 20MHz 0b011: 7MHz 0b100: 8.75MHz 0b101 ~ : reserved |
| } |  |  |
| Start frequency | 18 | Center frequency(in unit of 50 kHz) of the carrier located at lowest frequency position in the i-th carrier group |
| Number of Carriers | 3 |  |
| For(j=0;j<Number of Carriers; j++){ |  |  |
| Physical Carrier Index | 4 | Index of the Physical carrier |
| If(Configuration Type ==0b1){ |  | Multiple bandwidth in this group |
| Bandwidth | 6 | Bandwidth of the component carrier 0b00: 5MHz 0b01:10MHz 0b10:20MHz 0b11:reserved |
| } |  |  |
| If(Duplexing _Mode ==1){ |  | Multiple bandwidth in this group |
| Carrier Type | 1 | Indicates the type of this physical carrier 0b0: Fully configured carrier 0b1: Partially configured carrier |
| } |  |  |
| Center Frequency Shift Frequency offset | 3 | The value of center frequency shift of the current carrier for sub-carrier alignment {0, 1.5625, 3.125, 4.6875, 0, -1.5625,- 3.125, -4.6875}kHz 0b000:0 kHz 0b001: 1.5625 kHz 0b010: 3.125 kHz 0b011: 4.6875kHz 0b100: -1.5625kHz 0b101: - 3.125kHz 0b110: -4.6875kHz 0b111: reserved. |
| } |  |  |
| } |  |  |
| } |  |  |

[Modify texts on page 462, from 49 to line 65 as follows]

During the network entry procedure (defined in 16.2.8.2.3.), the AMS is notified of the frequency offset to be applied over each carrier for sub-carrier alignment through an AAI_Global-configCFG message. According to the multicarrier configuration index, the center frequency of the first carrier in each carrier group, frequency offset Δfc and the Physical Carrier Index in the AAI_Global-ConfigCFG message and the AAI_MC-ADV message, AMS can derive the center frequency of the available carriers by the associated frequency offset Δfcusing Table 803. The frequency offsets specified in Table 803 are calculated based on Equation (178).

[Modify texts on page 463, from line 34 to line 41 as follows]

Based on the center frequency of the first carrier in each carrier group and the bandwidth of each carrier that an AMS received from the AAI_Global-CFGConfig message and the bandwidth of each carrier, the center frequency of each carrier before sub-carrier alignment can be derived. Then the AMS can obtain the frequency offset Δfc to be applied over each carrier from the AAI_Global-CFG message based on the multicarrier configuration index, the reference carrier indicator and Table 803. So that AMS can obtain the correct center frequency offset before activating multi-carrier operations.

[Delete Table 803 as follows]

Table 803 – Multicarrier configuration index

| Index | Multi-carrier configuration(MHz) | Frequncy OffsetΔf(kHz) |
| --- | --- | --- |
| 1 | {5} | {0} |
| 2 | {7} | {0} |
| 3 | {8.75} | {0} |
| 4 | {10} | {0} |
| 5 | {20} | {0} |
| 6 | {10,10} | {0, -3.1248} |

---------------------------------------------------------End of the Text----------------------------------------------------------