| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for Power Control A-MAP IE (16.3.5.5.2.3) |  |
| Date Submitted | 2010-08-17 |  |
| Source(s) | Xianming Chen, Ding Ning, Yanfeng Guan, Bo Sun ZTE Corporation Jeongho Park Samsung Electronics Wookbong Lee LG Electronics | chen.xianming1@zte.com.cn jeongho.jh.park@samsung.com wookbong.lee@lge.com |
| Re: | Proposed text changes to P802.16m/D7 |  |
| Abstract | This contribution proposes text on Power Control A-MAP IE. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D8 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for Power Control A-MAP IE (16.3.5.5.2.3)

Xianming Chen, Ding Ning, Yanfeng Guan, Bo Sun

ZTE Corporation

Jeongho Park

Samsung Electronics

Wookbong Lee

LG Electronics

- References

[1] IEEE 802.16m/D7

- Introduction

In 16m session#68, the text related to Power Control A-MAP IE has been modified, but some confusing problems exist all the same. For example, 1) the number of HARQ regions in a UL AAI subframe is not considered by the proposed PC A-MAP text in D7; 2) the notes of “UL_FEEDBACK_SIZE” field in S-SFH SP3 is ambiguous; 3) the “Power control channel resource size indicator” field in S-SFH SP1 is not necessary.

This contribution provides some proposals to clear the text related to the above aspects.

- The Scope of Suggested Remedies

1. UL_FEEDBACK_SIZE

Currently, UL_FEEDBACK_SIZE is included in S-SFH SP3 IE (page 557, Table 839) which has 4 bit size representing 1 ~ 16 LRUs for the uplink control channel resource.

Feedback channels can be used for fast feedback and HARQ feedback, so the specific feedback channel type needs to be given.

2. Power Control A-MAP IE

Viewing from the current power control A-MAP IE text, each UL AAI subframe seems to own the identical number of HARQ FBCH and FFBCH. For example, in D7, there is such a description that “T: necessary number of PC-A-MAPs in a DL AAI subframe corresponding to the number of FFBCH, which is defined by T = ceil(U∙Q/D)”, here Q is total number of FFBCH in a UL AAI subframe, “U∙Q” is used in the formula “T = ceil(U∙Q/D)”, therefore, it can be inferred that each UL AAI subframe has Q FFBCHs.

But actually under the TDD mode, the number of HARQ FBCH and FFBCH in any UL AAI subframe shall depend on frame configuration, as shown Table 1 ~ 4. Here, “-” means that the corresponding configuration shall not occur.

Table 1 (D : U = 4 : 4): In the case, each UL AAI subframe contains one HARQ region.

| # of LRU for FBCH |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # of FBCH |  | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| # of H-FBCH in 5MHz per HARQ Region |  | Number of channels for FFBCH per UL AAI Subframe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0b00: 06 | UL SF 0 ~ 3 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 |
| 0b01: 12 | UL SF 0 ~ 3 | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
| 0b10: 18 | UL SF 0 ~ 3 | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 |
| 0b11: 24 | UL SF 0 ~ 3 | - | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
| # of H-FBCH in 10MHz per HARQ Region |  | Number of channels for FFBCH per UL AAI Subframe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0b00: 06 | UL SF 0 ~ 3 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 |
| 0b01: 12 | UL SF 0 ~ 3 | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
| 0b10: 24 | UL SF 0 ~ 3 | - | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
| 0b11: 30 | UL SF 0 ~ 3 | - | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 |
| # of H-FBCH in 20MHz per HARQ Region |  | Number of channels for FFBCH per UL AAI Subframe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0b00: 12 | UL SF 0 ~ 3 | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
| 0b01: 24 | UL SF 0 ~ 3 | - | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
| 0b10: 48 | UL SF 0 ~ 3 | - | - | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 |
| 0b11: 60 | UL SF 0 ~ 3 | - | - | - | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 |

Table 2 (D : U = 5 : 3): In the case, the 0th and 2nd UL AAI subframes contain two HARQ regions respectively, and the 1st UL AAI subframe contains one HARQ region.

| # of LRU for FBCH |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # of FBCH |  | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| # of H-FBCH in 5MHz per HARQ Region |  | Number of channels for FFBCH per UL AAI Subframe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0b00: 06 | UL SF 0,2 | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
|  | UL SF 1 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 |
| 0b01: 12 | UL SF 0,2 | - | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
|  | UL SF 1 | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
| 0b10: 18 | UL SF 0,2 | - | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 |
|  | UL SF 1 | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 |
| 0b11: 24 | UL SF 0,2 | - | - | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 |
|  | UL SF 1 | - | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
| # of H-FBCH in 10MHz per HARQ Region |  | Number of channels for FFBCH per UL AAI Subframe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0b00: 06 | UL SF 0,2 | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
|  | UL SF 1 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 |
| 0b01: 12 | UL SF 0,2 | - | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
|  | UL SF 1 | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
| 0b10: 24 | UL SF 0,2 | - | - | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 |
|  | UL SF 1 | - | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
| 0b11: 30 | UL SF 0,2 | - | - | - | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 |
|  | UL SF 1 | - | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 |
| # of H-FBCH in 20MHz per HARQ Region |  | Number of channels for FFBCH per UL AAI Subframe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0b00: 12 | UL SF 0,2 | - | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
|  | UL SF 1 | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
| 0b01: 24 | UL SF 0,2 | - | - | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 |
|  | UL SF 1 | - | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
| 0b10: 48 | UL SF 0,2 | - | - | - | - | - | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 |
|  | UL SF 1 | - | - | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 |
| 0b11: 60 | UL SF 0,2 | - | - | - | - | - | - | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 |
|  | UL SF 1 | - | - | - | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 |

Table 3 (D : U = 3 : 5): In the case, only the 1st, the 2nd and the 3rd UL AAI subframes contain one HARQ region respectively, and the 0th and the 4th UL AAI subframe contain no HARQ regions.

| # of LRU for FBCH |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # of FBCH |  | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| # of H-FBCH in 5MHz per HARQ Region |  | Number of channels for FFBCH per UL AAI Subframe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0b00: 06 | UL SF 1,2,3 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 |
|  | UL SF 0,4 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| 0b01: 12 | UL SF 1,2,3 | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
|  | UL SF 0,4 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| 0b10: 18 | UL SF 1,2,3 | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 |
|  | UL SF 0,4 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| 0b11: 24 | UL SF 1,2,3 | - | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
|  | UL SF 0,4 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| # of H-FBCH in 10MHz per HARQ Region |  | Number of channels for FFBCH per UL AAI Subframe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0b00: 06 | UL SF 1,2,3 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 |
|  | UL SF 0,4 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| 0b01: 12 | UL SF 1,2,3 | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
|  | UL SF 0,4 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| 0b10: 24 | UL SF 1,2,3 | - | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
|  | UL SF 0,4 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| 0b11: 30 | UL SF 1,2,3 | - | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 |
|  | UL SF 0,4 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| # of H-FBCH in 20MHz per HARQ Region |  | Number of channels for FFBCH per UL AAI Subframe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0b00: 12 | UL SF 1,2,3 | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
|  | UL SF 0,4 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| 0b01: 24 | UL SF 1,2,3 | - | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
|  | UL SF 0,4 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| 0b10: 48 | UL SF 1,2,3 | - | - | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 |
|  | UL SF 0,4 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| 0b11: 60 | UL SF 1,2,3 | - | - | - | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 |
|  | UL SF 0,4 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |

Table 4 (D : U = 6 : 2): In the case, each UL AAI subframe contains three HARQ regions.

| # of LRU for FBCH |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # of FBCH |  | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| # of H-FBCH in 5MHz per HARQ Region |  | Number of channels for FFBCH per UL AAI Subframe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0b00: 06 | UL SF 0,1 | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 |
| 0b01: 12 | UL SF 0,1 | - | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 |
| 0b10: 18 | UL SF 0,1 | - | - | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 |
| 0b11: 24 | UL SF 0,1 | - | - | - | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 |
| # of H-FBCH in 10MHz per HARQ Region |  | Number of channels for FFBCH per UL AAI Subframe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0b00: 06 | UL SF 0,1 | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 |
| 0b01: 12 | UL SF 0,1 | - | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 |
| 0b10: 24 | UL SF 0,1 | - | - | - | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 |
| 0b11: 30 | UL SF 0,1 | - | - | - | - | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 |
| # of H-FBCH in 20MHz per HARQ Region |  | Number of channels for FFBCH per UL AAI Subframe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0b00: 12 | UL SF 0,1 | - | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 |
| 0b01: 24 | UL SF 0,1 | - | - | - | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 |
| 0b10: 48 | UL SF 0,1 | - | - | - | - | - | - | - | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| 0b11: 60 | UL SF 0,1 | - | - | - | - | - | - | - | - | - | 0 | 3 | 6 | 9 | 12 | 15 | 18 |

3. Power control channel resource size indicator

This is the field name with 2bit included in S-SFH SP1 IE, which represents the total number of PC-A-MAP IE per DL AAI subframe. According to the field value, total 4 cases are defined for PC-A-MAP IE size.

But actually, it is the truth that through receiving “Number of UL HARQ feedback channels per HARQ feedback region” in S-SFH SP1 and “UL_FEEDBACK_SIZE” in S-SFH SP3, the necessary number of PC-A-MAPs in a DL AAI subframe (T) can be implicitly acquire by an AMS. Therefore, it shall not be necessary that an ABS explicitly informs AMSs of PC A-MAP channel size.

- Proposed Text

Remedy #1: page 565, Modify line 17~65 as follows:

-------------------------------------------------Text Change Start --------------------------------------------------------------

For FDD/H-FDD/TDD systems, when AMS transmits the q-th FFBCH at the m-th uplink subframe in the i-th frame, the corresponding PC-A-MAP IE to the AMS automatically follows the assignment location defined in Table 844.

N : total number of AAI subframes in an AAI frame (In TDD systems, N=D+U. In FDD/H-FDD, N=D=U)

D : total number of DL AAI subframes in an AAI frame

U : total number of UL AAI subframes in an AAI frame

Q : total number of FFBCH in an AAI frame, Q = U∙Qmax

Qmax : the maximum FFBCH number among total number of FFBCH in a UL AAI subframes given by Qmax = Nfb∙UL_FEEDBACK_SIZE – NHARQ-Region-min ∙LHFB/6 where Nfb is 3. When frame structure is supporting the WirelessMAN-OFDMA with FDM-based uplink PUSC zone, Nfb is 4. And LHFB is the number of UL HARQ channels per HARQ region defined in S-SFH SP1, NHARQ-Region-min is the minimum value among number of HARQ regions in the UL AAI frame shown in Table xxx.

Table xxx. The number of HARQ regions in each UL AAI subframe

| AAI DL subframe : AAI UL subframe | NHARQ-Region-min |
| --- | --- |
| 6:2 | 3 |
| 5:3 | 2 |
| 4:4 | 1 |
| 3:5 | 0 |
| 2:6 | 0 |

q : the feedback channel index specified in Feedback Allocation A-MAP IE (0≤q≤Q-1)

m : UL AAI subframe index (0≤m≤U-1)

T : necessary number of PC-A-MAPs in a DL AAI subframe corresponding to the total number of FFBCH in any UL AAI subframe, which is defined by T = ceil(U∙Q/D)

s: the feedback channel index in an AAI frame, i.e., the q-th feedback channel atin the m-th UL AAI subframe, s=Q∙m Qmax∙m +q

Table 844-PC-A-MAP IE location

|  | Frame index | Subframe index | Channel index |
| --- | --- | --- | --- |
| F-FBCH location | i | m | q |
| PC-A-MAP IE location for FDD/H-FDD systems | i+floor[(m+N/2)/N] | (floor(m+N/2)) mod N | q |
| PC-A-MAP IE location for TDD systems | i+1 | floor(s/T) | s mod T |

-------------------------------------------------Text Change END ---------------------------------------------------------------