| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Data Forwarding Scheme for Relay in IEEE P802.16m/D7 (16.6.2.3.1) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Kanchei(Ken) Loa, Yi-Ting Lin, Chun-Che Chien, Tsung-Yu Tsai, Youn-Tai Lee, Chiu-Wen Chen, Chun-Yen Hsu Institute for Information Industry (III) Hyunjeong Kang Samsung Electronics Co., Ltd. Xiangying Yang Intel Yuqin Chen ZTE Corporation | E-mail: loa@iii.org.tw |
| Re: | Sponsor ballot IEEE P802.16m/D7 : 16.6.2.3.1 |  |
| Abstract | This contribution proposes the text change for data forwarding scheme for relay. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Data Forwarding Scheme for Relay in IEEE P802.16m/D7 (16.6.2.3.1)

Kanchei(Ken) Loa, Yi-Ting Lin, Chun-Che Chien, Tsung-Yu Tsai, Youn-Tai Lee, Chiu-Wen Chen, Chun-Yen Hsu

Institute for Information Industry (III)

Hyunjeong Kang

Samsung Electronics Co., Ltd.

Xiangying Yang

Intel

Yuqin Chen

ZTE Corporation

# Introduction

In current IEEE 802.16m/D7, the Forwarding ID in ARFEH not only identifies the AMS transmitting or receiving the ASN data between ABS and ARS, but also identifies the tunnel forwarding the ASN data. In this contribution, the description of Forwarding ID is modified to expand on the cases.

# Reference

[1] IEEE P802.16m/D7, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” July 2010.

- Text proposal for inclusion in the 802.16m Draft 7

----------------------------------------------------------------------------------------------------------------------------------------------------------------

[Add the following text in Section 16.2.2.2]

Suggested Remedy #1

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.2.2.2.11 Advanced Relay Forwarding Extended Header (ARFEH)

The ARFEH may be used when the ASN data traffic between ABS and ARS is encapsulated into 16m MAC PDU. The ARFEH is used for identifying which AMS transmitting or receiving the MAC PDU. The ARFEH format is defined in Table 678.

Table 678—ARFEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| ARFEH() { |  |  |
| Type | 4 | Extended header type = 0b1010 (ARFEH Type) |
| Do { |  |  |
| Forwarding ID | 12 | Identity of the AMS, which transmits or receives MAC PDU the ASN data forwarded between ABS and ARS. |
| End | 1 | 0: one or more Forwarding ID follows. 1: This Forwarding ID is the last Forwarding ID. |
| }While (!End) |  |  |
| } |  |  |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------

[Add the following text in Section 16.6.2.3.1]

Suggested Remedy #2

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.6.2.3.1 Data Forwarding Scheme

For DL transmission via an ARS, when an ABS transmits data to AMSs via the ARS, the ABS shall encapsulate ASN data traffic of one or multiple target AMSs into a relay MAC PDU of the ARS where the advanced relay forwarding extended header (ARFEH) may be appended to identify the AMS receiving the ASN data traffic. The ARS shall decode the DL basic assignment A-MAP IE to receive relay MAC PDUs. The ARS shall de-encapsulate received relay MAC PDU and transmits the ASN data traffic to target AMSs in format of MAC PDU.

For UL transmission via an ARS, the ARS shall encapsulate the ASN data traffic from one or multiple AMSs into a relay MAC PDU where the advanced relay forwarding extended header (ARFEH) may be appended to identify the AMS transmitting the ASN data traffic.

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------