| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed MAC Control Message Specification for AAI_CLC-REQ (16.2.3.16) |  |
| Date Submitted | 2010-08-11 |  |
| Source(s) | Lei Wang InterDigital Communications, LLC | Voice : +1 858 205-7286 E-mail: leiw@billeigean.com |
| Re: | IEEE Standards Sponsor Ballot Recirculation-1 for P802.16m/D7. |  |
| Abstract | The contribution proposes the MAC control message specification for AAI_CLC-REQ message (16.2.3.16). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed MAC Control Message Specification for AAI_CLC-REQ (16.2.3.16)

Lei Wang

InterDigital Communications, LLC

- Introduction

In the 802.16m/D7, in the definition of AAI_CLC-REQ as shown in Table 696, there are two variable-size optional fields, CLC request and CLC report; and each of those two optional fields has its own condition of presence. Then question is how the receiver of this message knows one or two optional fields is included, if one, then which one.

Such a message design/specification generates either ambiguities or even not-decodable messages, when there are multiple optional fields, for which there is not deterministic way to determine whether or not those optional fields are included in. For example, if the optional fields have completely different sizes, then the receiver can guess which one / ones included; however, if the optional fields have variable sizes and similar sizes to each other, then the receiver has no way to tell.

Again, in order to properly specify the 16m MAC control messages, we strongly recommend using the 16e-style pseudo c-code to specify the MAC control messages, which gives deterministic definition of MAC control messages

- Suggested changes in the 802.16m/D7

The following is the proposed change in the 802.16m/D7. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1: line 1, page 131

Delete the entire Table 697

Suggested change #2: line 13, page 133

Delete the entire Table 698, plus line 26 on page 133

Suggested change #3: line 42, page 133

Delete the entire Table 699, plus line 57 on page 133

Suggested change #4: throughout the 16m draft standard

Replace the references to “Table 697” by “Table 696”;

Replace the references to “Table 698” by “Table 696”;

Replace the references to “Table 699” by “Table 696”;

Suggested change #5: line 12, page 130

Replace Table 696 by the following Table:

Table 696 – AAI_CLC-REQ Message

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI_CLC-REQ Message () { |  |  |
| MAC control message type | 8 | Identify this MAC control message with the type value assigned in Table 679. |
| CLC request/report indicator | 2 | Indicate the inclusion of CLC request and/or CLC report: 0b00: reserved 0b01: CLC request only 0b10: CLC report only 0b11: CLC request and report |
| If (CLC request/report indicator == 0b10 ¦¦ 0b11) { |  | CLC report |
| CLC report type | 8 | 0: collocated interference level 1: non collocated interference level 2: interference level (source unknown) Others: reserved |
| If (CLC report type == 0 ¦¦ 1 ¦¦ 2) { |  |  |
| Interference level | 8 | signed integer: -127 ~ 127 |
| } |  |  |
| } |  |  |
| If (CLC request/report indicator == 0b01 ¦¦ 0b11) { |  | CLC request |
| Request Action | 8 | Bitmap, each bit of this parameter indicates whether AMS requests to activate or deactivate the corresponding CLC class Bit #i set to 0: indicates that the AMS requests to terminate the existing CLC class with CLC ID =i; or the CLC class with CLC ID =i does not exist; Bit #i set to 1: indicates that the AMS requests to activate the CLC class with CLC ID = I if it does not exist. For existing CLC ID, AMS may keep existing configuration, request to re-configure or replace existing CLC class. |
| For (i=0; i<8; i++) { |  |  |
| If (Request Action [i] == 1) { |  |  |
| CLC ID | 3 | 0~7 |
| Start Superframe Number | 3 | the 3 LSB of the superframe number of CLC start time |
| Start Frame Index | 2 | the frame index of CLC start time |
| Scheduling Impact | 2 | 0b00: both DL and UL allocations are prohibited in CLC active interval 0b01: only DL allocations are prohibited in CLC active interval 0b10: only UL allocations are prohibited in CLC active interval 0b11: reserved |
| Flag | 3 | 0b000: Type I 0b001: Type II subtype 1 0b010: Type II subtype 2 0b011: Type II subtype 3 0b100: Type III Others: reserved |
| If (Flag == 0b000) { |  |  |
| Start AAI Subframe Index | 3 | the subframe index of the first subframe of the CLC active interval |
| CLC active interval of Type I CLC class | 8 | the number of AAI subframes of the CLC active interval |
| CLC active cycle of Type I CLC class | 21 | the number of microseconds of the CLC active cycle |
| } |  |  |
| If (Flag == 0b001) { |  |  |
| CLC active bitmap of Type II CLC class with subtype 1 | 8 | Setting a bit of the field to "1" indicates the corresponding AAI subframe in each frame is in CLC active interval |
| } |  |  |
| If (Flag == 0b010) { |  |  |
| Start AAI Subframe Index | 3 | the subframe index of the first subframe of the CLC active interval |
| CLC active interval of Type II CLC class with subtype 2 | 8 | the number of AAI subframes of CLC active interval |
| CLC active cycle of Type II CLC class with subtype 2 | 8 | the number of AAI frames of the CLC active cycle |
| } |  |  |
| If (Flag == 0b011) { |  |  |
| CLC active cycle of Type II CLC class with subtype 3 | 2 | x (0, 1 or 2): the length of Extended CLC active bitmap of Type II CLC class with subtype 3 is x + 2 |
| Extended CLC active bitmap of Type II CLC class with subtype 3 | variable | Setting a bit of the field to "1" indicates the corresponding AAI subframe in each CLC active cycle is in CLC active interval. If the CLC active cycle is x frames, and a frame consists of m AAI subframes, the length of the field shall be x x m. |
| } |  |  |
| If (Flag == 0b100) { |  |  |
| CLC active interval of Type III CLC class | 8 | the number of superframes of the CLC active interval |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| reserved | variable | For byte alignment |
| } |  |  |

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D7, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”