| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup for the AAI_LBS-IND MAC control message (16.8.2) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Lei Zhou, Xufeng Zheng, Hai Wang China Samsung Telecom R&D Center Sangheon Kim, Ji-Yun Seol, Jaeweon Cho, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. | l.zhou@samsung.com +86-10-59253333 |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D7 |  |
| Abstract | Cleanup for the AAI_LBS-IND message to support U-TDOA on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Cleanup for the AAI_LBS-IND MAC control message (16.8.2)

Lei Zhou, Xufeng Zheng, Hai Wang,

China Samsung Telecom R&D Center

Sangheon Kim, Ji-Yun Seol, Jaeweon Cho,Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

IEEE C802.16m-10/799r2 has been accepted in IEEE 802.16 #68 meeting. But unfortunately IEEE C802.16m-10/799r2 can’t be correctly implemented in IEEE 802.16m/D7. Some editorial errors exist in Figure 607&608 in IEEE 802.16m/D7. So this contribution  proposes to modify some editorial errors of figure 607&708 in IEEE 802.16m/D7.

Text to change in AWD

-------------------------------  Text Start  ---------------------------------------------------

-------------------------------  Text Proposal1 Start ---------------------------------------------------

[Adopt the following Figure modification started from line# 1 Page 862, Figure 607, in P802.16m/D7]

Figure 607—Operation description on AAI for General U-TDOA

-------------------------------  Text Proposal1 End ---------------------------------------------------

-------------------------------  Text Proposal2 Start ---------------------------------------------------

[Adopt the following Figure modification started from line# 13 Page 863, Figure 608, in P802.16m/D7]

Figure 608 Operation description on AAI for Special U-TDOA

---------------------------------  Text Proposal2 End ---------------------------------------------------------

-----------------------------------------  Text End ---------------------------------------------------------------