| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Control Signaling on triggering and reporting for Enhanced LBS Support (16.8.3) |  |
| Date Submitted | 2010-08-18 |  |
| Source(s) | Lei Zhou, Xufeng Zheng, Hai Wang China Samsung Telecom R&D Center Sangheon Kim, Seonghyeon Chae, Jaeweon Cho, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. Alexey Khorayev Muthaiah Venkatachalam Intel | l.zhou@samsung.com +86-10-59253333 |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D7 |  |
| Abstract | This contribution proposes solution of control signaling for E-LBS zone |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Control Signaling on triggering and reporting for Enhanced LBS Support (16.8.3)

Lei Zhou, Xufeng Zheng, Hai Wang

China Samsung Telecom R&D Center

Sangheon Kim, Seonghyeon Chae, Jaeweon Cho,Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes solution of control signaling for E-LBS zone based on 802.16m Draft Amendment, IEEE P802.16m/D7. We propose to use SCN-RSP/REP to trigger MS to measure E-LBS-zone and report measurement results.

The Proposed Text in AWD

-------------------------------  Text Start  ---------------------------------------------------------

-------------------------------  Text Proposal1 Start ---------------------------------------------------

[adopt the following text modification started from line# 43 Page 866 in P802.16m/D7]

16.8.3.4 D-LBS zone measurement and report

An ABS may trigger a location measurement on D-LBS zone by sending AAI_SCN-RSP to the AMS. The AMS shall respond to this with AAI_SCN-REP with the parameters that enable location determination. (See section 16.2.3.14 and 16.2.3.15)

-------------------------------  Text Proposal1 End ---------------------------------------------------

-------------------------------  Text Proposal2 Start ---------------------------------------------------

[adopt the following text modification started from line# 46 Page 123 in P802.16m/D7]

Table 694—AAI_SCN-RSP parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Scan duration | 8 | Duration (in units of AAI subframes) of the requested scanning period. |  |
| M | Report mode | 2 | 0b00:No report 0b01:Periodic report 0b10:Event-triggered report 0b11:One-time scan report |  |
| M | Report metric | 4 | Bitmap indicator of trigger metrics that the serving ABS requests the AMS to report. The serving ABS shall indicate only the trigger metrics agreed during AAI_SBC-REQ/RSP negotiation. Each bit indicates whether reports will be initiated by trigger based on the corresponding metric: Bit 0: ABS CINR mean Bit 1: ABS RSSI mean Bit 2: Relative delay Bit 3: ABS RTD |  |
| M | Report period | 8 | The period of AMS's report of scanning result when the AMS is required to report the value periodically or one-time. The period is calculated from the start of the first scan duration. If the ABS sends an unsolicited AAI_SCN-RSP message without assignment of a scanning interval and the scan duration is set to zero, the period is calculated from the frame the AMS receives the AAI_SCN-RSP message. For AMS scanning request denied by AAI_SCN-RSP with Scan Duration set to zero, Report period is the number of frames that ABS suggests to AMS before transmitting next AAI_SCN-REQ |  |
| O | Duration for measurements signal location parameters on D-LBS zone | 4 | Duration (in units of 4 superframes) of the requested scanning. | Present if ABS request AMS to scan D-LBS zone for signal location parameters measurements |
| O | start super frame number | 6 | Represents the 6 least significant bits of the absolute super frame number in which the first Scanning interval starts | Present only when Scan Duration > 0 |
| … | … | … | … | … |

-------------------------------  Text Proposal2 End ---------------------------------------------------

------------------------------------------------------  Text End  ------------------------------------------------