| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification on HO execution procedure |  |
| Date Submitted | 2010-8-17 |  |
| Source(s) | Guanghui Fan, Bin Chen Huawei Technologies Shahab Sayeedi Motorola Peretz Feder Alcatel-Lucent | E-mail: fanguanghui@huawei.com binchen@huawei.com shahab.sayeedi@motorola.com peretz.feder@alcatel-lucent.com |
| Re: | Call for SB on “ P802.16m/D7”: |  |
| Abstract | This contribution proposes about modification on HO execution procedure in IEEE 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Sponsor Ballot. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification on HO execution procedure

Guanghui Fan， Bin Chen

Huawei Technologies

Introduction

According to the spec text, the second AAI_HO-CMD may still have multiple candidates, then should the AMS send another AAI_HO-IND to tell its final choice? The further state flow is not clear enough and may induce implemental confusion. We propose that a second AAI_HO-IND shall be sent if the AMS decides to handover to any target in the second AAI_HO-CMD.

Text proposal for inclusion in the P802.16m/D7

========================== Start of Proposed Text ==============================

[Editor’s Note 1 : modify the text as following in page 286 line 19]

If all target ABSs included in the AAI_HO-CMD message are unreachable (as defined in this section) or if the AAI_HO-CMD message includes no target ABS, and if the AMS has a preferred target ABS it shall inform the serving ABS of its preferred target ABS by sending the AAI_HO-IND message with HO Event Code 0b001 prior to expiration of Disconnect Time. If the AMS has no preferred target ABS to include in the AAI_HO-IND message, it may perform HO cancellation as described in section 16.2.6.3.6.

If a serving ABS receives the AAI_HO-IND message with HO Event Code 0b001, it shall respond to the AMS by sending the AAI_HO-CMD message with target ABSs which may include the target ABS proposed by the AMS in the AAI_HO-IND message. The AMS shall wait to receive the AAI_HO-CMD message until expiration of Disconnect Time unless the AMS cannot maintain communication with the serving ABS (as defined in this section). If the AMS cannot maintain communication with the serving ABS, it may proceed to its preferred target ABS after sending the AAI_HO-IND message with its preferred target ABS to the serving ABS. If after receiving a second AAI_HO-CMD message none of the target ABSs proposed by the serving BS are reachable by the AMS, the AMS may proceed to its preferred target ABS or perform HO cancellation as described in section 16.2.6.3.6. If the AMS decides to handover to a target ABS included in the second AAI_HO-CMD message, it shall send a second AAI_HO-IND message with HO Event Code 0b000 to inform the serving ABS. If the AMS has no preferred target ABS to include in the AAI_HO-IND message, it may perform HO cancellation as described in section 16.2.6.3.6.

========================== End of Proposed Text ==============================