| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification for Paging Carrier Update in idle mode for Multi-carrier |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Jia Lin, Bin chen, Junxian Mo, Phillip Barber Huawei Technologies Hyunjeong Kang Samsung Electronics | E-mail: linjia@huawei.com binchen@huawei.com hyunjeong.kang@samsung.com |
| Re: | Call for SB on “ P802.16m/D7”: Target topic: “16.2.8.2.10.2” |  |
| Abstract | This contribution proposes amendment text describing trigger function in multi-carrier scenario in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Sponsor Ballot. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Clarification for Paging Carrier Update in idle mode for Multi-carrier

Jia Lin, Bin Chen, Junxian Mo, Phillip Barber

- Huawei Technologies

Hyunjeong Kang

- Samsung  Electronics

- Introduction

- Paging Carrier update in Idle mode for multicarrier has been defined in IEEE P802.16m/D7 as follow:

- An AMS may perform Location Update process to acquire its preferred carrier for the idle mode support when AMS cannot find the paging carrier. In this case, Ranging Purpose Indication = 0b0001 in AAI_RNG-REQ message is used for indicating paging carrier update in location update process, and the AMS may include Paging Carrier Update parameter in AAI_RNG-REQ message to inform its preferred carrier. The preferred carrier shall be continuously used in the subsequent paging until the preferred paging carrier is updated during the location update procedure.

- For cleanup its impacts in other sections of IEEE P802.16m/D7, a few modifications should be added into section 16.2.8.2.10.2 and Table 679.

- References

- IEEE P802.16m/D7, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for inclusion in the P802.16m/D7

========================== Start of Proposed Text ==========================

[Note to Editor: Add the following contents in section 16.2.8.2.10.2 and Table 679]

- 16.2.8.2.10.2 Idle mode

- An AMS may perform Location Update process to acquire its preferred carrier for the idle mode support when AMS cannot find the paging carrier. In this case, Ranging Purpose Indication = 0b0001 in AAI_RNG-REQ message is used for indicating paging carrier update in location update process, and the AMS may include Paging Carrier Update parameter in AAI_RNG-REQ message to inform its preferred paging carrier. Upon the AMS receiving an AAI_RNG-RSP message for the corresponding location update procedure, Tthe preferred paging carrier is updated and shall be continuously used in the subsequent paging listening intervaluntil the preferred paging carrier is updated during the location update procedure.

- Table 679—AAI_RNG-REQ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Power Cycle Change | 4 | PAGING_CYCLE requested by the AMS | It shall be included when the AMS in Idle Mode is attempting to change Paging Cycle during Location Update |
| O | Paging Carrier update | 4 | Preferred Paging carrier updateindex requested by the AMS | It shall be included when the AMS in Idle Mode is attempting to update Paging Carrier during Location Update |

========================== End of Proposed Text ===========================