| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Capability Negotiation on Battery Level Report (16.2.2.1.3) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao, Yu-Min Chiu ITRI | Voice : + 886 3-5914629 E-mail: mhtao@itri.org.tw |
| Re: | Category: SB_16m / Area: Chapter 16.2.2.1.3 (MAC Signaling Header) |  |
| Abstract | The contribution proposes some modifications to AMS Battery Level Report Signaling Header (16.2.2.1.3.5). |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Capability Negotiation on Battery Level Report (16.2.2.1.3)

Ming-Hung Tao, Ying-Chuan Hsiao, Yu-Min Chiu

ITRI

- Introduction

The AMS battery level report mechanism is missing in the current D7. It is required for the ABS and AMS to negotiate the battery report capacity. Then the AMS transmits the battery level report header according to the negotiated behavior. In case the ABS needs to poll the AMS about its battery information, some messages must be designed for this purpose. For simplicity, we suggest adding additional information in the battery level report header to enable this function.

- Proposed Text for IEEE P802.16m/D7

[Accept the following modifications marked in blue]

============================ Start of Proposed Text ==================================

[Accept the modified text from line 17, page 46 (16.2.2.1.3)]

#### 16.2.2.1.3 MAC Signaling Header

Table 656—Type field encodings for MAC signaling header type

| Type field (4 bits) | MAC Signaling Header Type |
| --- | --- |
| 00000 | BR with STID |
| 00001 | BR without STID |
| 00010 | Service Specific Scheduling Control Header |
| 00011 | Sleep Control |
| 00100 | AMS Battery Level Report |
| 00101 | Uplink Power Status Report |
| 00110 | Correlation Matrix Feedback |
| 00111 | MIMO Feedback |
| 01000 - 11111 | Reserved |

[Accept the modified text from line 37, page 51 (16.2.2.1.3.5)]

#### 16.2.2.1.3.5 AMS Battery Level Report Header

The AMS Battery Level Report Header should be used by the AMS to convey AMS its battery level information from AMS to the ABS, or may be used by the ABS to poll the AMS about the battery level information. Battery level is defined as the amount of energy remaining in the battery as a percentage of the rated capacity of the battery. The units and algorithm used by the AMS to determine the battery level are implementation specific. The format of the AMS Battery Level Report Header is given in Table 661.

Table 661—AMS Battery level Report header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AMS Battery Level Report header () { |  |  |
| FID | 4 | Flow Identifier. Set to 0001. |
| Type | 5 | MAC signaling header type = 0b00100 |
| Length | 3 | Indicates the length of the signaling header in bytes. |
| Polling Indication | 1 | 0b0: This is a report message from AMS to ABS 0b1: This is a polling message from ABS to AMS |
| If (Polling Indication = 0) { |  |  |
| AMS Battery Status | 1 | 0b0: The AMS is plugged into a power source 0b1: The AMS is not plugged into a power source |
| Battery Level Indication | 1 | 0b0: Detailed battery level report is not included. 0b1: Detailed battery level report is included. |
| If (Battery Level Indication = 1) { |  |  |
| AMS Battery Level | 3 | 0b000: Battery level is >75% and <= 100% 0b001: Battery level is >50% and <= 75% 0b010: Battery level is >25% and <= 50% 0b011: Battery level is >5% and <= 25% 0b100: Battery level is below 5% 0b110 – 0b111: Reserved |
| Reserved | 76 | Shall be filled by 0 |
| else { |  |  |
| Reserved | 21 | Shall be filled by 0 |
| } |  |  |
| } |  |  |
| Else { |  |  |
| Reserved | 3 | Shall be filled by 0 |
| } |  |  |
| } |  |  |

[Accept the modified text from line 20, page 89 (16.2.3.4)]

#### 16.2.3.4 AAI_SBC-REQ

Table 683—AAI_SBC-REQ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| O | MIH Capability Supported | 1 | If bit #0=1, the capability of IEEE 802.21 Media Independent Handover Services supports. |  |
| O | Battery Report Mode | 2 | 0b00: The AMS reports its battery information periodically 0b01: The AMS reports its battery information when its battery level or status changes 0b10: The AMS doesn’t report its battery information unless being polled by the ABS 0b11: The AMS doesn’t report its battery information |  |

[Accept the modified text from line 20, page 92 (16.2.3.5)]

#### 16.2.3.5 AAI_SBC-RSP

Table 684—AAI_SBC-RSP message Field Description

| M/O | Attributes / Array of attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| O | MIH Capability Supported | 1 | If bit #0=1, the capability of IEEE 802.21 Media Independent Handover Services supports. |  |
| O | Battery Report Mode | 2 | 0b00: The AMS reports its battery information periodically 0b01: The AMS reports its battery information when its battery level or status changes 0b10: The AMS doesn’t report its battery information unless being polled by the ABS 0b11: The AMS doesn’t report its battery information |  |

[Accept the modified text from line 56, page 883 (16.11)]

#### 16.11 Global Values

Table 983—Parameters and constants

| System | Name | Time Reference | Minimum Value | Default Value | Maximum Value |
| --- | --- | --- | --- | --- | --- |
| ABS, AMS | BR grant timer | UL allocation waiting time for BR header during network entry | - | - | - |
| AMS | Battery Report Timer | Nominal time between transmission of Battery Level Report Headers | - | - | - |

============================== End of Proposed Text ====================================

- References

- IEEE P802.16m/D6. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, May 2010.

- IEEE 802.16m-08/0034r2. IEEE 802.16m System Description Document, Sep 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.