| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Trigger function in Multi-carrier Operation in 802.16m |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Jia Lin, Bin Chen, Junxian Mo, Phillip Barber Huawei Technologies Hyunjeong Kang Samsung Electronics Kelvin Chou MediaTek | E-mail: linjia@huawei.com binchen@huawei.com hyunjeong.kang@samsung.com |
| Re: | Call for SB on “ P802.16m/D7”: Target topic: “16.2.6.2” |  |
| Abstract | This contribution proposes amendment text describing trigger function in multi-carrier scenario in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Sponsor Ballot. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed Text for Trigger function of Multicarrier Scenario

- in 802.16m

Jia Lin, Bin Chen, Junxian Mo, Phillip Barber

- Huawei Technologies

Hyunjeong Kang

- Samsung  Electronics

Kelvin Chou

- MediaTek

- Introduction

- This contribution proposes amendment text describing the Trigger function when a 16m ABS and AMSs support multiple carriers.

- In Multicarrier Scenario, an AMS may use multiple carriers for data transmission. When an AMS scans the carriers of the Serving ABS and Neighbor ABSs, it may trigger the actions such as “respond on trigger with AAI_SCN-REP”. Since the AMS communicates with the ABS on multiple carriers, if AMS find there are other carriers of Neighbor ABS or Serving ABS satisfying trigger functions, it is possible that AMS is triggered to report these conditions.

- In 16m/D7, only trigger conditions definitions that are applicable for a single carrier scenario, but not for a multicarrier scenario, are considered. In a multicarrier scenario, if currently defined trigger functions are used to trigger actions (e.g. respond on trigger with AAI_SCN-REP or AAI_HO-REQ), it may happen that the AMS will be triggered to send lots of AAI_SCN-REP messages that are not useful for the serving ABS. Since the AMS communicates with the serving ABS on multiple carriers, even though one neighbor ABS’s carrier metric is greater than a given absolute value, or greater than serving ABS’s carriers metric by relative value, there may be no reason for action since there are other carriers available for the AMS at the serving ABS. In this case, the serving ABS can provide high data traffic throughput for the AMS, and the AMS doesn’t need to send a scan report or HO-REQ.

- Conclusion

- We propose two trigger condition for the multicarrier scenario:

- Number of neighbor ABS’s carriers which are greater than absolute value is more than specific value

- Number of neighbor ABS’s carriers which are greater than absolute value is more than number of AMS’s active carriers.

- Benefits for introducing trigger functions for Respond on trigger with AAI_SCN-REP in MC scenario:

- Avoid useless trigger actions for MC scenario.

- Reduce signaling overhead.

- Reduce signaling overhead.

- References

- IEEE P802.16-2009 “IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access”.

- IEEE P802.16m/D7, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for inclusion in the P802.16m/D7

========================== Start of Proposed Text ==========================

[Note to Editor: Add the following contents in Table 773 in “16.2.6.2 Trigger condition definitions”]

- 16.2.6.2 Trigger condition definitions

Table 773—Trigger; Type/Function/Action Description

| Name | Size (Bit) | Value | Description |
| --- | --- | --- | --- |
| Type | 3(MSB) | Trigger metric type: 0x0: CINR 0x1: RSSI 0x2: RTD 0x3: Number of consecutive P-SFHs missed 0x4: RD 0x5-0x7: reserved |  |
| Function | 3 | Computation defining trigger condition: 0x0: Reserved 0x1: Metric of neighbor ABS is greater than absolute value 0x2: Metric of neighbor ABS is less than absolute value 0x3: Metric of neighbor ABS is greater than serving ABS metric by relative value 0x4: Metric of neighbor ABS is less than serving ABS metric by relative value 0x5: Metric of serving ABS greater than absolute value 0x6: Metric of serving ABS less than absolute value 0x7: (For Multicarrier scenario only): Number of carriers in neighbor ABS whose metric (CINR/RSSI) is greater than absolute value. The threshold value for the “Number of carriers” can be AMS specific or ABS specific. (When it is AMS specific, the threshold value is defined as the number of this AMS’s current carriers for data transmission. When it is ABS specific, the threshold value is configured by the serving ABS and signaled as a Trigger Value.) | NOTE-0x1-0x4 not applicable for RTD trigger metric When type 0x1 is used together with function 0x3 or 0x4, the threshold value shall range from -32 dB (0x80) to +31.75 dB (0x7F). When type 0x1 is used together with function 0x1, 0x2, 0x5 or , or 0x6, or 0x7, the threshold value shall be interpreted as an unsigned byte with units of 0.25 dB, such that 0x00 is interpreted as -103.75 dBm and 0xFF is interpreted as -40 dBm Type 0x3 can only be used together with function 0x5 or function 0x6 Fuction 0x7 can only be used for Action 0x1 in Multicarrier scenario. |
| Action | 3(LSB) | Action performed upon reaching trigger condition: 0x0: Reserved 0x1: Respond on trigger with AAI_SCN-REP 0x2: Respond on trigger with AAI_HO-REQ 0x3: Respond on trigger with AAI_SCN-REQ 0x4: Declare ABS unreachable 0x5: Cancel HO 0x6 and 0x7: Reserved | Action 0x3 applies to only Function 0x5 and 0x6. Action 0x4: If this ABS is the serving ABS(meaning the AMS is unable to maintain communication with the ABS), AMS sends AAI_HO-IND with HO Event Code 0b010 to the serving ABS and proceeds as specified in section 16.2.6.3.4. If this ABS is a target ABS, the AMS needs not take immediate action when this trigger condition is met for a single ABS. The AMS shall act only when this condition is met for all target ABSs included in AAI-HO-CMD during HO execution. The specific actions are described in section 16.2.6.3.4. |

16.2.8.2.9 Handover

The multicarrier handover (MCHO) is defined as the handover procedure which involves multiple radio carriers. An AMS with multicarrier capability may follow the single-carrier handover procedure per section 16.2.6. It may also decide to perform MCHO procedure as defined in this section.

For an AMS supported multicarrier aggregation, it shall use its Primary Carrier as reference for serving ABS scanning and follow the trigger/action defined for its Primary Carrier obtained by AAI_SCD broadcast on its Primary Carrier for handover or scanning related operations.

The AMS may perform scanning on carriers of neighbor ABSs autonomously, based on triggers, or by the instruction from the ABS. The trigger function 0x7 for trigger action 0x1 (Respond on trigger with AAI_SCN-REP) may be applied to reduce unnecessary scanning reports by considering the number of carriers of the neighbor ABSs whose CINR or RSSI measurement is greater than an absolute value (i.e., a threshold value), where the threshold value for the “number of carriers” is specific to each AMS and is equal to each AMS’s active carriers. Alternatively, the threshold value for the “number of carriers” is specified by the serving ABS as a Trigger Value.

========================== End of Proposed Text ===========================