| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Clarification of the Relay FDD Frame Structure (16.6.3.2.1) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Jinyoung Chun, Jin Sam Kwak LG Electronics | jiny.chun@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Sponsor Ballot Recirc #1 |  |
| Abstract | This contribution provides the clarification of the Relay FDD Frame Structure. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The Clarification of the Relay FDD Frame Structure (16.6.3.2.1)

Jinyoung Chun, Jin Sam Kwak

LG Electronics

- Introduction

In the last meeting, most of relay issues are solved. ARS’s TDD HARQ procedure is matched to ABS’s and the existence of additional MIMO midamble is defined in SFH SP2. But still the procedure related to ARS is unclear.

When AMS tries the network entry at first, the AMS doesn’t know whether the BS is ABS or ARS by detecting SA-Preamble. Then the AMS assumes that the BS is ABS and follows ABS operation with ABS frame structure. Then the BS can schedule the AMS as below:

- If the BS is ABS without ARSs, the operation is normal.

- If the BS is ABS with ARSs, the ABS doesn’t allocate the long TTI bursts to the AMS. (implementation issue)

- If the BS is ARS, the ARS doesn’t allocate the long TTI burst to the AMS and doesn’t allocate the burst in the last subframe in DL access zone if DL R-RTI is 1. (implementation issue)

After AMS receives AAI-SCD message, the AMS can interpret the BS type and the next operation is as below:

- If the BS is ABS, AMS know the existence of additional MIMO midamble in SFH SP2 and cell specific mapping in AAI-SCD message. AMS follows ABS’s operation based on the information.

- If the BS is ARS, AMS follows ARS’s operation such as long TTI allocation in access zone. For that, AMS needs to know the ARS’s DL R-TTI and UL Tadv.

In TDD, the HARQ operation is same with ABS and ARS. Therefore even if AMS misunderstands the BS type, there is no problem. But in FDD, the HARQ operation is different with ABS and ARS. It’s very critical problem to the operation of ABS and ARS. For example, when cell specific mapping is ‘on’ in FDD, AMS served by ABS transmits and receives in Access zone and can’t follow the FDD HARQ timing as Figure A.

Figure A. An example of FDD HARQ timing when ABS supports ARS

In this contribution, in order that ARS’s FDD HARQ timing is as same as the ABS’s one, we suggest that UL Relay zone precedes UL Relay zone in a UL FDD frame and the ratio of Access Zone and Relay Zone is fixed to 4:4.

- Text proposal in the P802.16m/D7

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

------------------------------------------------   Text Start  -----------------------------------------------------------

[Modify the text in line 7 of page 845 as below:]

When ARSs are supported the ABS frame is divided on AAI Access zone and AAI Relay zone. The AAI Access zone position precedes the AAI Relay zone position inside the TDD frame and FDD DL frame. The AAI Relay zone position precedes the AAI Access zone position inside the FDD UL frame. The duration of the AAI Access zone and AAI Relay zone may be different in DL and UL directions. The zone configuration of AAI Access zone and AAI Relay zone is informed to the ARS by the ABS in AAI_ARS-CONFIG-CMD message.

[Modify the text in line 39 of page 847 as below:]

When an ARS switches transceiver states from receive to transmit or from transmit to receive, an UL ARS radio frame in FDD system may have a R-RTI between AAI Access in the previous UL ARS frame and AAI Relay zone and a R-TTI between AAI Relay zone and AAI Access zone in the next UL ARS frame. The location of the R-RTI is the first OFDM symbol of the first AAI subframe of AAI Relay zone and the location of R-TTI is the last OFDM symbol of the last AAI subframe of AAI Relay zone. The duration of R-RTI and R-TTI in UL shall be calculated by following equations:

[Modify the figure 603 in line 1 of page 848 as below:]

Figure 603 – Example of ARS FDD frame structure with G=1/8 in 5/10/20MHz

-----------------------------------------------   Text End  -----------------------------------------------------------