| 2Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for clarifications of Contention-based random access bandwidth request (D7 - 16.2.11.1.1) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Heejeong Cho, Ki Seon Ryu, Youngsoo Yuk, Jin Sam Kwak LG Electronics | Voice : +82-31-450-1876 E-mail: heejeong.cho@lge.com |
| Re: | Sponsor Ballot on IEEE p802.16m D7 |  |
| Abstract | This contribution proposes some modification on section 16.2.11.1.1 for clarification. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for clarifications of Contention-based random access bandwidth request (D7 - 16.2.11.1.1)

Heejeong Cho, Ki Seon Ryu, Youngsoo Yuk, Jin Sam Kwak

LG Electronics

- Introduction

In the previous #68 session, several comments were submitted for clean-up and clarification of the contention-based random access bandwidth request in the IEEE 802.16m DRAFT Amendment [1]. However there are still ambiguous descriptions and incorrect figure. In this contribution, the proposed text about clarification of current contention-based random access bandwidth request is suggested.

- Modification/Rationale

- Clarify the description related to implicit-NACK (line 10 in page 331)

    - According to D7, at least one BR-ACK A-MAP IE shall be sent at the DL frame of the frame n+BR_ACK_Offset if the ABS detects at least one BR preamble sequence in the BR opportunities of frame n and does not grant UL resources to all the successfully received BR requests before or in the frame n+BR_ACK_Offset. If no BR-ACK A-MAP IEs are sent at the DL frame of frame n+BR_ACK_Offset, the AMS considers it as an implicit-NACK and may restart BR procedure. However, the AMS should not consider only no reception of the BR-ACK A-MAP IE when AMS decide it has received an implicit-NACK. That’s because BR-ACK A-MAP IE can be omitted by providing all UL BW allocation for all the successfully received BR requests before or in the frame n+BR_ACK_Offset.

    - Suggestion: Insert the sentence into the text as suggested in remedy-1

- Clarify the description related to time when BR-ACK A-MAP IE is sent (line 15 in page 331)

    - Because BR-ACK A-MAP IE is sent at the frame n+BR_ACK_Offset, not frame n+1, current text (from line 17 to 23 in page 331) is incorrect.

    - Each BR-ACK A-MAP IE should include the decoding status of all BR opportunities in frame n.

    - Suggestion: Modify the sentence as suggested in remedy-2

- Clean-up of the Figure the 428 (line 1 in page 333)

    - In 5-step random access BR procedure, ABS sends UL grant for UL data, not for standalone BR header at step-4. And then, AMS sends UL data, not BR header at step-5.

    - Suggestion: Modify the sentence as suggested in remedy-3

- References

[1] IEEE P802.16m/D7, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------

[Remedy 1 : on the IEEE P802.16m/D7, page 331, line 10, make the following changes to subclause 16.2.11.1.1]

In this case if no BR-ACK A-MAP IEs are sent at the DL frame of frame n+BR_ACK_Offset and the AMS does not receive any UL grant before or in frame n+BR_ACK_Offset, the AMS considers it as an implicit-NACK and may restart BR procedure.

[Remedy 2 : on the IEEE P802.16m/D7, page 331, line 15, make the following changes to subclause 16.2.11.1.1]

The BR-ACK A-MAP IE indicates

—The decoding status of each BR opportunity in the previous n-th frame (no or at least one BR preamble sequence is detected). Each BR-ACK A-MAP IE contains its own bitmap relating only to the preamble indices being acknowledged/granted in this A-MAP IE. Each BR-ACK A-MAP IE contains its own BR-ACK Bitmap of size equal to the number of BR opportunities in the n-th frame. In case of multiple BR-ACK A-MAP IEs are transmitted in a single subframe, the BR opportunities acknowledged in each BR-ACK A-MAP IE shall be mutual exclusive.

—The correctly received BR preamble sequences in the BR opportunities of the previous UL n-th frame being acknowledged/granted in this A-MAP IE, and

[Remedy 3 : on the IEEE P802.16m/D7, page 333, line 1, make the following changes to subclause 16.2.11.1.1]

Figure 428—Example of 5-step random access BR procedure

-----------------------------------------------------------End of the Text---------------------------------------------------------