| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for HARQ operation for carrier switching mode (16.9.2.4) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, youngsoo.yuk@lge.com, kiseon.ryu@lge.com |
| Re: | Sponsor ballot recirc #1 on P802.16m/D7 |  |
| Abstract | This contribution proposes the texts related to HARQ operation of carrier switching mode for E-MBS. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for HARQ operation for carrier switching mode (16.9.2.4)

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu and Jin Sam Kwak

LG Electronics

- Introduction

Interruption time from carrier switching operation yields abnormal HARQ operation. We propose a simple modification of HARQ operation to support carrier switching mode.

- Text proposal for the 802.16m DRAFT amendment

---------------------------------------------------------Start of the Proposed Text ---------------------------------------------------------

[Remedy: Modify and add the following text in line 33 page 870, section 16.9.2.4 of 802.16m/D7]

16.9.2.4 E-MBS Operation on the multi-carrier deployment

When E-MBS data, including E-MBS configuration message, E-MBS MAP, and E-MBS contents, is transmitted on an alternative carrier, i.e. other than the AMS's primary carrier where service flows are configured the AMS is redirected to relevant carrier through DSA as described in 16.9.2.1

The AMS with multiple transceivers may be able to receive E-MBS data while communicating with ABS on primary carrier.

The AMS with only one transceiver may perform primary to secondary carrier switching where E-MBS configuration message and the E-MBS data are transmitted on an alternative carrier to the AMS and other communication with ABS is performed in the primary carrier.

If the carrier switching from the primary carrier to the secondary carrier is required before HARQ re-transmission is finished, DL HARQ retransmission shall be skipped while UL HARQ retransmission shall be terminated. The HARQ burst shall be discarded if the HARQ transmission is not finished until T_ReTx_Interval*DL_N_MAX_ReTx frames after first transmission.

-----------------------------------------------------------End of the Proposed Text   --------------------------------------------------------