| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Editorial clean up for test vectors (Annex Q) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Youngkyo Baek Young-Kwan Choi Samsung Electronics Erik Colban Huawei | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot Recirc # 1 on P802.16m/D7 |  |
| Abstract | This contribution proposes correction on test vector for cryptographic methods to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by WG SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Fix test vectors for cryptographic methods (Annex Q)

Youngkyo Baek, Young-Kwan Choi

Samsung Electronics

Erik Colban

Huawei

# 1. Introduction

This contribution suggests fixing test vectors to prevent misunderstanding and some typos.

# 2. Text Proposal

Modify the sentences at page 931, line 1 as follows

======================== Start of Proposed Text =====================

Annex Q

(informative)

Test vectors

Q.1 Cryptographic method test vectors

Note that all values are shown in hexadecimal notation

Q.1.1 AES-CCM:

Q.1.1.1 Short payload and short ICV

  - Plaintext PDU

    - Advanced Generic MAC header =  0xD0 0D6

    - Payload = 0x 9c 05 3f 24

    - STID=0x 234, FID=0x D

  - Ciphertext PDU where TEK = 0x D50E18A844AC5BF38E4CD72D9B0942E5, EKS=0x 1 (2 bits), PN=0x17F6BC (22 bits) and ICV length is 4 bytesB:

    - Advanced Generic MAC header = D0 0D

    - Initial CCM block B0 (128 bits):
0x 09 D0 0D 23 4D 00 00 00 00 00 00 57 F6 BC 00 04

    - Encrypted payload MAC PDU consisting of unencrypted AGMH (2 bytes), unencrypted EKS+PN (3 bytesB), encrypted payload (4 bytesB) and, encrypted ICV (4 bytesB):

0x D0 0D 57 F6 BC   10 71 D1 B0   FF 70 71 B1

Q.1.1.2 Long payload and long ICV

  - Plaintext PDU

    - Advanced Generic MAC header = A0 CA

    - Payload  (200 bytesB):

00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F

10 11 12 13 14 15 16 17 18 19 1A 1B 1C 1D 1E 1F

20 21 22 23 24 25 26 27 28 29 2A 2B 2C 2D 2E 2F

30 31 32 33 34 35 36 37 38 39 3A 3B 3C 3D 3E 3F

40 41 42 43 44 45 46 47 48 49 4A 4B 4C 4D 4E 4F

50 51 52 53 54 55 56 57 58 59 5A 5B 5C 5D 5E 5F

60 61 62 63 64 65 66 67 68 69 6A 6B 6C 6D 6E 6F

70 71 72 73 74 75 76 77 78 79 7A 7B 7C 7D 7E 7F

80 81 82 83 84 85 86 87 88 89 8A 8B 8C 8D 8E 8F

90 91 92 93 94 95 96 97 98 99 9A 9B 9C 9D 9E 9F

A0 A1 A2 A3 A4 A5 A6 A7 A8 A9 AA AB AC AD AE AF

B0 B1 B2 B3 B4 B5 B6 B7 B8 B9 BA BB BC BD BE BF

C0 C1 C2 C3 C4 C5 C6 C7

    - STID=0x 234, FID=0x A

  - Ciphertext PDU where TEK = 0x B74EB0E4F81AD63D121B7E9AECCD268F, EKS=0x 3 (2bits), PN=0x3B5F11 (22 bits) and ICV length is 8 bytesB:

    - Advanced Generic MAC header = A0 D5

    - Initial CCM block B0 (128bits):
19 A0 D5 23  4A 00 00 00  00 00 00 FB  5F 11 00 C8

    - Encrypted payload MAC PDU consisting of unencrypted AGMH (2 bytes), unencrypted EKS+PN (3 bytesB), encrypted payload (200 bytesB) and, encrypted ICV (8 bytesB):

- A0 D5 FB 5F 11

EA 53 E1 74 89 B2 0B F3 F0 9B 0C 1B 84 9A A7 78

B8 D2 67 35 4F F6 95 D1 8B 60 79 F6 67 DB FF 3D

8C 76 AC C1 0C B5 A6 BB 6C 54 1B 61 FB 13 45 DA

4E A9 0A F4 B9 AC B5 AF 28 21 20 95 41 02 7B 4B

13 A8 BA 16 3B 9F 88 42 56 3E B4 0B 8C 4C EA 68

C0 74 F3 C1 CC BF D0 84 C2 7F D1 AC 48 44 E6 7D

63 63 1A F3 D9 39 F2 8F 6D F5 64 31 06 4B AA DE

2C AB C2 C9 8C BC 87 41 78 B7 85 27 C4 DD 33 D0

02 50 32 81 14 B2 32 8C 28 C7 11 72 75 CE FF 57

F2 E5 80 83 B2 08 24 4E 7A C4 18 63 3F CB 38 85

7C 7B DC AC E9 D1 1B 6B 8B EF E3 54 16 AE 3D 26

5A 10 7C FA 39 D6 51 17 67 16 46 3B 26 EE EF 85

EE 74 67 A7 13 DC 03 EF

2F 6B 08 CF 49 2A E1 04

- Q.1.2 AES-CTR:

Q.1.2.1 Short payload

  - Plaintext PDU

    - Advanced Generic MAC header = D0 06

    - Payload = 9c 05 3f 24

    - STID=0x 234, FID=0x D

  - Ciphertext PDU where TEK = 0x D50E18A844AC5BF38E4CD72D9B0942E5, EKS=0x 1 (2bits) and PN=0x 17F6BC (22 bits):

    - Advanced Generic MAC header = D0 06

    - Encrypted payload MAC PDU consisting of unencrypted AGMH (2 bytes), unencrypted EKS+PN (3 bytesB) and, encrypted payload (4 bytesB):

- D0 09 57 F6 BC 86 FB 65 B7

Q.1.2.2 Long payload

  - Plaintext PDU

    - Advanced Generic MAC header = A0 CA

    - Payload  (200B):

00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F

10 11 12 13 14 15 16 17 18 19 1A 1B 1C 1D 1E 1F

20 21 22 23 24 25 26 27 28 29 2A 2B 2C 2D 2E 2F

30 31 32 33 34 35 36 37 38 39 3A 3B 3C 3D 3E 3F

40 41 42 43 44 45 46 47 48 49 4A 4B 4C 4D 4E 4F

50 51 52 53 54 55 56 57 58 59 5A 5B 5C 5D 5E 5F

60 61 62 63 64 65 66 67 68 69 6A 6B 6C 6D 6E 6F

70 71 72 73 74 75 76 77 78 79 7A 7B 7C 7D 7E 7F

80 81 82 83 84 85 86 87 88 89 8A 8B 8C 8D 8E 8F

90 91 92 93 94 95 96 97 98 99 9A 9B 9C 9D 9E 9F

A0 A1 A2 A3 A4 A5 A6 A7 A8 A9 AA AB AC AD AE AF

B0 B1 B2 B3 B4 B5 B6 B7 B8 B9 BA BB BC BD BE BF

C0 C1 C2 C3 C4 C5 C6 C7

    - STID=0x 234, FID=0x A

  - Ciphertext PDU where TEK = 0x B74EB0E4F81AD63D121B7E9AECCD268F, EKS=0x 3 (2bits) and PN=0x 3B5F11 (22 bits):

    - Advanced Generic MAC header = A0 CD

    - Encrypted payload MAC PDU consisting of unencrypted AGMH (2 bytes), unencrypted EKS+PN (3 bytesB) and, encrypted payload (200 bytesB):

- A0 CD FB 5F 11

EC 86 6C FF 73 C8 CF A6 25 A6 2D E5 8E 68 0E 35

CD 0E AC 0F 0B A6 EE 50 6C CC 13 81 67 6C 85 6E

83 99 58 DF B8 BB 89 74 10 37 3A C3 37 0B 7D C6

BF 52 34 9C 85 25 92 27 79 85 D3 5C 62 F1 A9 67

DA 21 2B 87 04 D6 70 6C CC FD 2E B6 AD 27 64 CD

F9 DA AD 86 5B 20 5F 8D 20 37 BA 36 13 CD E8 E0

51 43 D4 C8 D5 CF 0B FA 92 8D 49 0F 91 2B 70 9A

6C 7C A0 9F FB 48 14 EB 08 03 DA 9E 13 A0 1C A3

E5 01 86 12 22 BD 1C 8A B5 E3 4E 17 A5 00 FC C7

91 DA F2 98 C5 A2 49 EC FC 92 39 ED 6B 4C F4 6A

2E 0D D2 58 55 0F DB 7F 97 A6 3B 3B 67 E3 BF 29

43 F6 7A 31 E2 6F 1B EB 51 12 D4 1C 07 F6 48 B0

A6 BF AB C6 77 2E 6E 27

- Q.1.3 AES-CMAC:

This CMAC calculation is performed according to the formula indicated in the section 16.2.5.2.3.2.

Q.1.3.1 Short MAC control message

  - Plaintext PDU

    - ASN.1 encoded MAC control message = 9c 05 3f 24

    - STID=0x 234, FID=0x D

  - Signature where CMAC_KEY= 0x D50E18A844AC5BF38E4CD72D9B0942E5, AKID=0x A67B1FE254CD290A (64bits) and CMAC_PN=0x 57F6BC (24 bits):

    - Message header (AKID | CMAC_PN |STID|FID|24-bit zero padding | ASN.1 encoded MAC_Control_Message) = 
A6 7B 1F E2  54 CD 29 0A  57 F6 BC 23  4D 00 00 00

    - CMAC value (8 bytesB)= 78 1C 63 71 6F 48 6A 6F

Q.1.3.2 Long MAC control message :

  - Plaintext PDU

    - ASN.1 encoded MAC control message (100 bytesB):

00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F

10 11 12 13 14 15 16 17 18 19 1A 1B 1C 1D 1E 1F

20 21 22 23 24 25 26 27 28 29 2A 2B 2C 2D 2E 2F

30 31 32 33 34 35 36 37 38 39 3A 3B 3C 3D 3E 3F

40 41 42 43 44 45 46 47 48 49 4A 4B 4C 4D 4E 4F

50 51 52 53 54 55 56 57 58 59 5A 5B 5C 5D 5E 5F

60 61 62 63

    - STID=0x ABC, FID=0x A

  - Signature where CMAC_KEY= 0x B74EB0E4F81AD63D121B7E9AECCD268F, AKID=0x D5F725AE30F45B3C (64bits) and CMAC_PN=0x 3B5F11 (24 bits):

    - Message header (AK ID | CMAC_PN |STID|FID|24-bit zero padding | ASN.1 encoded MAC_Control_Message) = 
D5 F7 25 AE  30 F4 5B 3C  3B 5F 11 AB  CA 00 00 00

    - CMAC value (8 bytesB)= DA 0A 50 5D  04 2A 08 38

============================== End of Proposed Text ===============

# 4. References

- IEEE P802.16m/D7. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, MAY 2010.