| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Capability negotiation about frame configuration to support R1 system (16.2.3.8) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Youngkyo Baek Samsung Electronics | E-mail: Youngkyo.baek@samsung.com |
| Re: | Sponsor Ballot Recirc on P802.16m/D8 |  |
| Abstract | This contribution proposes text to fix AAI_REG-REQ/RSP considering Capability negotiation about frame configuration to support R1 system. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Capability negotiation about frame configuration to support R1 system (16.2.3.8)

- Youngkyo Baek

- Samsung Electronics

- Introduction

AMS's capabilities are negotiated through SBC and REG messages during the network entry procedure.

Item 'Frame configuration to support legacy', which is the report to ABS about capability to support legacy R1 MS, is included in AAI_REG-REQ. It can be used for determining whether zone switching is possible or not.

In current D8 ,5 MHz and 10 MHz only are incorporated.

Hence, we suggest adding 8.75 and 7 MHz cases also .

- Proposed text

II.1 Proposed text 1

Modify the texts in line 22, page 109 as proposed text.

------------------------------------------------- Start of proposed text I--------------------------------------------------

Table 686—AAI_REG-REQ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| … | …. | …. | …. | …. |
| O | I) frame configuration to support legacy R1.0(5MHz) | 1 4 | If Bit#0 =1,it R1 5MHz supports(TDD only) If Bit#1 =1,R1 10MHz supports If Bit#2 =1,R1 8.75MHz supports If Bit#3 =1,R1 7MHz supports | Present if needed |
| O | J) frame configuration to support legacy(10MHz) | 1 | If Bit#0 =1,it supports(TDD only) | Present if needed |
| ….. | …… | …… | ….. | ………. |

[Editorial note : the above proposed table can be replaced with the following remedy, based on the reformatted control message format.]

| Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| …. | …. | …. | …. |
| frame configuration to support legacy R1.0(5MHz) | 1 4 | If Bit#0 =1,it R1 5MHz supports(TDD only) If Bit#1 =1,R1 10MHz supports If Bit#2 =1,R1 8.75MHz supports If Bit#3 =1,R1 7MHz supports | Present if needed |
| frame configuration to support legacy(10MHz) | 1 | If Bit#0 =1,it supports(TDD only) | Present if needed |
| …… | …… | ….. | ………. |

------------------------------------------------- End of proposed text I --------------------------------------------------