| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on MSID privacy disabled operation mode (16.2.3.51) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Youngkyo Baek Jicheol Lee Jungshin Park Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot Recirc # 1 on P802.16m/D7 |  |
| Abstract | This contribution proposes mode of MSID privacy disabled operation to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on MSID privacy disabled operation mode (16.2.3.51)

Youngkyo Baek, Jicheol Lee, Jungshin Park

Samsung Electronics

# 1. Introduction

Currently a legacy Access networks are deployed together with a legacy core networks (e.g. AAA server).

Due to development of the new 16m technologies we expect the 16m Access network will be deployed in some time.

So we may expect that 16m Access network is operating together with a legacy core network.

In those situations, MSID privacy may be disabled.

The AMSs need to know whether the network(e.g. NSP) supports MSID privacy or not.

Hence we suggest using AAI_SII-ADV message to notify the NSP's MSID privacy policy.

# 2. Solution

Regarding the NSP’s policy, we can use AAI_SII-ADV message which broadcast NSPs’ information which the ABS supports. By adding MSID privacy indication on AAI_SII-ADV message, the AMS can realize the mode of operation. i.e. AMS decide which it makes AAI_RNG-REQ contain MSID* or MSID.

# 3. Text Proposal

Modify the table at page 216, line 20 as follows

======================== Start of Proposed Text #1=====================

16.2.3.51 AAI_SII-ADV (Service Identity Information)

An ABS may use the AAI_SII-ADV message to broadcast a list of Network Service Provider (NSP) Identifiers and optionally whether MSID privacy is enabled or not. Assignment method, administration, and usage of NSP IDs are outside the scope of this standard. The list of NSP IDs to be included in this message and the message transmission frequency are programmable. The following parameters may be included in the AAI_SII-ADV message; at least one shall be included in an AAI_SII-ADV message.

Table 752— AAI_SII-ADV message format

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | nspArray [1..16] | variable | List of one or more Network Service Provider(NSP) 24 bit Identifiers. |  |
| M | A) nspIdentifier | 24 | NSP identifiers to report. The serving ABS shall indicate only the trigger |  |
| O | B)MSID privacy indication | 1 | It indicates whether each NSP supports MSID privacy. 0b0 : MSID privacy is disabled 0b1 : MSID privacy is enabled | It shall be included if ABS broadcasts MSID privacy support. |
| …… | …. | …… | …… | … |

============================== End of Proposed Text #1===============

# 4. References

- IEEE P802.16m/D7. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Mar. 2010.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.