| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal to announce MSID privacy policy of NSP (16.2.3.50) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Youngkyo Baek Jicheol Lee Jungshin Park Samsung Electronics Oleg Marinchenco Alvarion | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot Recirc on P802.16m/D8 |  |
| Abstract | This contribution proposes mode of MSID privacy disabled operation to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal to announce MSID privacy policy of NSP (16.2.3.50)

Youngkyo Baek, Jicheol Lee, Jungshin Park

Samsung Electronics

Oleg Marinchenco

Alvarion

# 1. Introduction

Currently MSID privacy can be enabled or disabled depending on the MSID privacy policy considering coexistence with a legacy core networks (e.g. AAA server).

AMS performs network entry according to the MSID privacy policy.

Provisioned AMS already knows Home NSP’s MSID privacy Policy, but the retail AMSs(i.e. un-provisioned AMSs) cannot know the Home NSP’s MSID privacy Policy. Hence during Network entry the retail AMS need to obtain the Home NSP’s privacy policy. But, how to learn privacy policy is still open in 16m/D8 draft[1].

In order that let all AMSs recognize the Home NSP’s MSID privacy policy, we suggest that ABS broadcast the NSP's MSID privacy policy using AAI_SII-ADV message.

# 2. Solution

Regarding the NSP’s policy, we can use AAI_SII-ADV message which broadcast NSPs’ information which the ABS supports. By adding MSID privacy indication on AAI_SII-ADV message, the AMS can realize the mode of operation. i.e. AMS decide which it makes AAI_RNG-REQ contain MSID* or MSID.

# 3. Text Proposal

Modify the table at page 233, line 50 as follows

======================== Start of Proposed Text #1=====================

16.2.3.50 AAI_SII-ADV (Service Identity Information)

An ABS may use the AAI-SII-ADV message to broadcast a list of Network Service Provider (NSP) Identifiers and optionally MSID privacy indication that the corresponding NSP supports MSID privacy or not. AMS, for which home NSP’s MSID privacy policy is not provisioned, can realize NSP’s privacy policy from MSID privacy indication.

Assignment method, administration, and usage of NSP IDs are outside the scope of this standard. The list of NSP IDs to be included in this message and the message transmission frequency are programmable. The following parameters may be included in the AAI-SII-ADV message; at least one shall be included in an AAI-SII-ADV message.

Table 747— AAI_SII-ADV message format

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | nspArray [1..16] | variable | List of one or more Network Service Provider(NSP) 24 bit Identifiers. |  |
| M | A) nspIdentifier | 24 | NSP identifier which ABS supports |  |
| O | B)MSID privacy indication | 1 | It indicates whether each NSP supports MSID privacy. 0b0 : MSID privacy is disabled 0b1 : MSID privacy is enabled | It shall be included if ABS broadcasts MSID privacy support. |
| …… | …. | …… | …… | … |

[Editorial note : the above table can be replaced with the following remedy, based on the reformatted control message format.]

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| Number of NSP IDs | 4 | Number of Network Service Providers |  |
| for (i=0; i<number of NSP IDs; i++) { |  | List of the Network Service Provider identifiers |  |
| NSP identifier | 24 | NSP identifier which ABS supports |  |
| MSID privacy indication | 1 | It indicates whether each NSP supports MSID privacy. 0b0 : MSID privacy is disabled 0b1 : MSID privacy is enabled | It shall be included if ABS broadcasts MSID privacy support. |
| } |  |  |  |
| …. | …… | …… | … |

============================== End of Proposed Text #1===============

Add some descriptions at line 56, page 298 as follows

======================== Start of Proposed Text #2=====================

AMSID privacy is applied in the following way;

• Home NSP’s MSID privacy policy is provisioned by MSID privacy indication, which is transmitted by AAI-SII-ADV or AAI-SBC-RSP messages to acknowledge NSP’s privacy policy to the un-provisioned AMSs.

•"AMSID privacy is mandatory to implement in ABS and AMS.

============================== End of Proposed Text #2===============

Add an attribute to the SBC-RSP message in table 684, page 103 as follows

======================== Start of Proposed Text #3=====================

Table 684—AAI-SBC-RSP message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | MIH Capability Supported | 1 | If bit #0=1, the capability of IEEE 802.21 Media Independent Handover Services supports. | Present as needed |
| O | nspArray [1..16] | Variable | List of one or more Network Service Provider 24 bit identifier | Present if SIQ bit 0 in AAI-SBC_REQ is set and if AAI-SII-ADV Message Pointer is not included. |
| O | A) nspIdentifier | 24 | NSP identifier which ABS supports |  |
| O | B)MSID privacy indication | 1 | It indicates whether each NSP supports MSID privacy. 0b0 : MSID privacy is disabled 0b1 : MSID privacy is enabled | It shall be included if ABS provides MSID privacy support. |

============================== End of Proposed Text #3===============

# 4. References

- IEEE P802.16m/D8. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Aug. 2010.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.