| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup for the section 16.2.15.4 Basic Capability Negotiation (16.2.15.4) |  |
| Date Submitted | 2010-08-15 |  |
| Source(s) | Youngkyo Baek, Hyunjeong Kang Samsung Electronics | E-mail: Youngkyo.baek@samsung.com |
| Re: | Sponsor Ballot Recirc # 1 on P802.16m/D7 |  |
| Abstract | This contribution proposes text to fix AAI_REG-REQ/RSP considering Capability negotiation about frame configuration to support R1 system. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Cleanup for the section 16.2.15.4 Basic Capability Negotiation (16.2.15.4)

- Youngkyo Baek, Hyunjeong Kang

- Samsung Electronics

- Introduction

The section 16.2.15.4 describes basic capability negotiation procedure during network entry.

However, the description follows the logic based on the assumption that capability class only is used for AMS capability negotiation.

Now, we have the concept of device class separately from the capability class and allow negotiation of configuration parameter and features one by one.

Hence, we need fix the current text as proposed text.

- Proposed text

Modify the texts in page 388 as proposed text.

------------------------------------------------- Start of proposed text I--------------------------------------------------

16.2.15.4 Basic Capability Negotiation

Immediately after completion of ranging, the AMS informs the ABS of its basic capabilities by transmitting an AAI-SBC-REQ message. A "Capability Class" is defined as a unique set of features and functions, configuration parameters, air-interface protocol revision, and/or services that can uniquely describe a mobile station implementation or configuration while operating in the network. Each Capability Class is indicated by its corresponding CAPABILITY_INDEX. The AMS, by default, shall support the basic capabilities associated with "Capability Class 0" (i.e. CAPABILITY_INDEX = 0). If the AMS is capable of supporting higher revisions of physical layer or medium access control layer protocols or further wishes to use enhanced features, it shall send an AAI_SBC-REQ message to the ABS indicating the Capability Class. If the AMS has higher capability than the capability suggested by CAPABILITY_INDEX=0, then the AMS may transmit a higher version of the capability index or the AMS may additionally include parameters in SBC-REQ message that represent the difference with respect to the transmitted capability index.

Upon receipt of the AAI_SBC-REQ message containing the "CAPABILITY_INDEX" from the AMS, the ABS determines whether it could allow, or could support the requested feature set and configuration parameters or MAC and/or PHY protocol revisions. If the ABS does support, or can allow the use of enhanced features and configuration parameters, it shall respond with an AAI_SBC-RSP message to inform the AMS of its decision. The ABS shall transmitsignal the AAI_SBC-RSP message containing a "CAPABILITY_INDEX" which may be numerically different than that requested by the AMS.

The "CAPABILITY_INDEX" values range from 0 to N, where N denotes the maximum CAPABILITY_INDEX value. The CAPABILITY_INDEX = 0 indicates the default capability index and basic feature set or configuration parameters.

The features and configuration parameters included in the Capability Class 0 shall be sufficient to meet the minimum performance requirements. In case of failure in any stages of operation, the AMS and ABS shall fall back to "Capability Class 0" and restart negotiations for a new "Capability Class", if necessary.

------------------------------------------------- End of proposed text I --------------------------------------------------