| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for AAI_DSA-RSP message (16.2.3.46.2) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Hyunjeong Kang, Yeongmoon Son, Jaehyuk Jang Samsung Electronics | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | Sponsor ballot recirc #1 on P802.16m/D7 |  |
| Abstract | The contribution proposes to clean up AAI_DSA-RSP and AAI_DSC-RSP messages for group parameter create/change. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for AAI_DSA-RSP message

Hyunjeong Kang, Yeongmoon Son, Jaehyuk Jang

Samsung Electronics

- Introduction

With Group Parameter Create/Change parameter, more than 1 service flows can be created or changed through one DSA or DSC transaction.

The transaction begins with an AAI_DSA-REQ or AAI_DSC-REQ message including the Group Parameter Create/Change for the service flows to create or change. Upon receipt of the AAI_DSA-REQ or AAI_DSC-REQ message, the receiver shall send an AAI_DSA-RSP or AAI_DSC-RSP with Confirmation code whether at least one service flow is successfully created or changed. If at least one service flow is successfully created or changed, then the AAI_DSA-RSP or AAI_DSC-RSP message includes a FID of the service flow created or changed successfully.

However, in current 16m/D7, any parameter for Group Parameter Create/Change is not defined in AAI_DSA-RSP and AAI_DSC-RSP message.

It is proposed to include Group Parameter Create/Change parameter in AAI_DSA-RSP and AAI_DSC-RSP message when a service flow for Group Parameter Create/Change is successfully created or changed.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1:]

[Add the following parameters into Table 741 of page 200 in subclause 16.2.3.46.2 AAI_DSA-RSP:]

| O | Group Parameter Create/Change | variable |  | Shall be included upon receiving an AAI_DSA-REQ including Group Parameter Create/Change parameter |
| --- | --- | --- | --- | --- |
| O | A) FID List | 4  N | This parameter indicates FIDs of service flows created successfully with this Group Parameter Create/Change. N is the number of successfully created service flow. | Shall be included when N > 0 |

[Remedy 2:]

[Add the following parameters into Table 744 on page 211 in subclause 16.2.3.46.5 AAI_DSC-RSP:]

| O | Group Parameter Create/Change | variable |  | Shall be included upon receiving an AAI_DSC-REQ including Group Parameter Create/Change parameter |
| --- | --- | --- | --- | --- |
| O | A) FID List | 4  N | This parameter indicates FIDs of service flows created successfully with this Group Parameter Create/Change. N is the number of successfully created service flow. | Shall be included when N > 0 |

-------------------------------  Text End  ---------------------------------------------------