| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Improving HARQ performance for Uplink Transmission (Section 16.2.14.1 – HARQ Functions – HARQ subpacket generation and transmission) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Anup Talukdar, Mark Cudak Motorola Inc. | Anup.Talukdar@motorola.com |
| Re: | Category: P802.16m/D7 comments for sb_16m Area: (Section 16.2.14.1 – HARQ Functions – HARQ subpacket generation and transmission) |  |
| Abstract | Incremental redundancy (IR) HARQ scheme is mandatory for unicast data traffic and unicast MAC control messages in both downlink and uplink. However, for uplink transmissions, benefits of IR HARQ technique can be limited, or even non-existent, when the ABS re-allocates resources for retransmissions. This contribution proposes a simple modification which will allow IR HARQ operation in the uplink when resource re-allocations for retransmissions are common for uplink data transmissions. The proposed scheme does not incur any additional overheads. |  |
| Purpose | Discuss and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Improving HARQ performance for Uplink Transmission 
(Section 16.2.14.1 – HARQ Functions – HARQ subpacket generation and transmission)

Anup Talukdar, Mark Cudak

Motorola, Inc.

# Introduction

Incremental redundancy (IR) HARQ scheme is mandatory for unicast data traffic and unicast MAC control messages in both downlink and uplink [1]. For IR scheme, each transmitted subpacket contains part of the codeword determined by a subpacket identifier (SPID). For uplink transmissions, the default retransmission scheme is synchronous where the AMS sends one of the subpackets labeled with SPID 0b00, 0b01, 0b10 and 0b11 in sequential order. However, ABS can re-allocate resources for retransmission by sending an UL basic Assignment A-MAP IE (with the same ACID and without toggling the AI_SN bit) to the AMS to override the synchronous HARQ retransmission. On receiving this A-MAP IE, the AMS is required to send the subpacket with SPID 0b00.

It can be observed that, re-allocation of resources for retransmission can help in eliminating ‘resource holes’ and in achieving better link adaptation in a broadband air-interface. However, in such cases, the benefits of IR HARQ technique for the uplink data transmissions can be very limited, and even non-existent, when for majority of the uplink retransmissions the ABS sends the UL basic Assignment A-MAP IE to re-allocate resources.

The proposed changes will allow use of IR HARQ scheme for the uplink data transmissions, even when the ABS re-allocates the resources for retransmissions by sending the UL basic Assignment A-MAP IE. In the proposed scheme, the UL basic Assignment A-MAP IE contains a single-bit parameter, called ‘Subpacket_Cycling’. The ABS should set this bit to ‘0’ to indicate that the AMS shall send the subpacket 0b00. The ABS should set this bit to ‘1’ to indicate that, the AMS shall send the next subpacket, in sequential order. The proposed ‘Subpacket_Cycling’ indication bit replaces the last ‘Reserved bit’ in UL basic Assignment A-MAP IE.; thus the size of the A-MAP remains same as before.

- Proposed Text Changes

Modify the Table 849 – UL Basis assignment A-MAP IE as follows (in Page 577, lines 31-43):

| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| --- | --- | --- |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| Reserved Subpacket cycling | 1 | Reserved bits 0b0 : OFF, 0b1: ON |
| } | - | - |

Modify the paragraph in page 367, line 55-62 in sub-section 16.2.14.1 as follows:

For Uplink,

a)  At the first transmission, AMS shall send the subpacket labeled 0b00.

b)  AMS shall send one among subpackets labeled 0b00, 0b01, 0b10 and 0b11 in sequential order.

c)  AMS shall send the subpacket labeled 0b00 when the AMS receives the UL Basic Assignment A-MAP IE() with the Subpacket_cycling set to 0b0; however, if the Subpacket_cycling is set to 0b1, the AMS shall send one among subpackets labeled 0b00, 0b01, 0b10 and 0b11 in sequential order.

# References

- IEEE P802.16m/D7, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”