| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | E-MBS resource configuration parameters update indication and action time |  |
| Date Submitted | 2010-08-17 |  |
| Source(s) | Kaiying Lv Nan Li Bo Sun ZTE Corporation | E-mail: lv.kaiying@zte.com.cn |
| Re: | Sponsor ballot comment on P802.16m/D7 Chapter 16.9.3.1.( E-MBS configuration indicator) |  |
| Abstract | This contribution proposes modification to AAI_E-MBS_CFG message by indicating update of E-MBS configuration parameters of SCD message. |  |
| Purpose | To be discussed and adopted. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

E-MBS Related SCD information update indication and action time

Kaiying Lv, Nan Li, Bo Sun

ZTE Corporation

# 1. Problem Statement

According to 802.16m/D7, E-MBS configuration parameters such as Zone_Allocation_Bit-MAP, Zone flag bit, MSI length etc. are transmitted in system configuration description message (AAI_SCD) in order for both unicast and E-MBS users to be informed of resource allocation information. In multicarrier scenario, normally carrier-switching capable users receive and decode updated AAI-SCD message on its primary carrier. However, when E-MBS services are deployed on the carrier other than its primary carrier, the carrier-switching capable E-MBS users may receive E-MBS and unicast services alternatively. In this case, according to the current system parameters update scheme, E-MBS users may receive updated AAI-SCD message on their primary carrier but cannot timely receive and decode updated AAI-SCD message on the E-MBS carrier once E-MBS configuration parameters are updated through AAI_SCD. For idle mode users, the same problem may happen.

On the other hand, since E-MBS configuration information in AAI_SCD message includes Zone resource allocation and MSI length which affect the burst allocation information, the updated information shall be applied at specific time which is related to MSI or AAI_E-MBS_CFG transmission period or AAI_E-MBS_CFG life time.

# 2. Proposed Schemes

We propose that E-MBS configuration message AAI_E-MBS_CFG includes one bit to indicate the update of E-MBS configuration information in AAI_SCD. The updated AAI_SCD shall start to be applied at the beginning of the first MSI within next AAI_E-MBS_CFG life time.

# 3. Text Proposal

======================== Start of Proposed Text =====================

[Change message format and text in P870,line 65, 16.9.3.1 E-MBS configuration indicators as follows:]

16.9.3.1 E-MBS Configuration Indicators

…

The E-MBS configuration information necessary for E-MBS operation include E-MBS-Zone_IDs of serving and neighboring ABSs, E-MBS MAP time offset (frequency offset), E-MBS MAP resource allocation, E-MBS MAP Isize-offset, E-MBS MAP MIMO Mode and E-MBS ID and FID mappings between serving E-MBS Zone and neighboring E-MBS Zone for the same content. AAI_E-MBS-CFG shall be advertised in the superframe before the beginning of the largest MSI (16 superframes). at the superframe whose number (Nsuperframe) from SFH meets the following condition.

Nsuperframe modulo 16 == 15

In the case of AAI_SCD update that includes any updated E-MBS configuration parameter, the updated E-MBS resource configuration parameters shall be applied at the beginning of the first MSI in next AAI_E-MBS_CFG life time cycle.

============================== End of Proposed Text ============================