| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to section 16.2.5.2.3.1 AES CCM |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Erik Colban Ronald Mao Bin Chen George Cummings Huawei Technologies, Ltd | Voice: +1 858 775 2494 E-mail: ecolban@huawei.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot – P802.16m IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems - Amendment: Advanced Air Interface |  |
| Abstract | This document shows suggested changes to P802.16m Draft D7 related to HO Event Code. |  |
| Purpose | Review and approve as a remedy to related Sponsor Ballot comment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes related to HO Event Code

Erik Colban

Huawei Technologies Co., Ltd

# Overview

During the first circulation of the sponsor ballot a comment to change the HO Event Code from 3 bits to two bits was accepted. Unfortunately, this comment failed to include changes in other parts of the standard that were dependent on the encoding of the HO Event Code. This contribution aims at correcting this situation.

# Proposed changes

## Change 1

On page 284, line 53:

For ABS initiated HO, the AMS may detect an SFH mismatch by referring to the S-SFH change count included in the AAI_HO-CMD message. The AMS should not select a T-ABS with mismatched SFH information. If the AMS does not have the latest SFH for any of the T-ABSs included in AAI_HO-CMD, the AMS should cancel the HO by sending AAI_HO-IND with HO event code 0b11 and SFH mismatch indication 0b1. When the serving ABS receives the AAI_HO-IND with HO event code 0b011 and SFH mismatch indication 0b1, it may send another AAI_HO-CMD message which includes all up-to-date delta SFH information of each recommended T-ABS. If pre-allocated at target ABS, the serving ABS shall include an STID to be used at target ABS in the AAI_HO-CMD message. The pre-allocated STID shall be used in the target ABS by the AMS to communicate with the target ABS. The FIDs which are used to distinguish different connections are not updated during the handover procedure. If the network decides that certain service flow will not exist at the target ABS, this shall also be indicated in the AAI_HO-CMD message.

## Change 2

On page 285, line 20:

When multiple target ABSs are included in the AAI_HO-CMD message, the HO preparation phase completes when the AMS informs the ABS of its target ABS selection via an AAI_HO-IND message with HO Event code 0b00.

## Change 3

On page 285, line 55:

If HO_Reentry_Mode is set to 0, the serving ABS stops sending DL data and providing UL allocations to the AMS after the disconnect time derived from Action Time and Disconnect Time Offset included in the AAI_HO-CMD message or upon receiving AAI_HO-IND with HO Event Code 0b10, whichever occurs first. If HO_Reentry_Mode is set to 1, the serving ABS stops sending DL data and providing UL allocations to the AMS after the disconnect time, upon receiving AAI_HO-IND with HO Event Code 0b10 or after receiving HO completion confirmation from target ABS, whichever occurs first. The serving ABS defines conditions based on which the AMS decides if it is unable to maintain communication with the serving ABS. If the AMS decides, based on these conditions, that it cannot maintain communication with the serving ABS until the expiration of Disconnect Time, the AMS may send an AAI_HO-IND message with HO Event Code 0b10 to the serving ABS. If the AAI_HO-CMD message includes more than one target ABS, and at least one of the target ABSs in the AAI_HO-CMD message is reachable, the AMS shall select one of these targets and inform the S-ABS of its selection by sending an AAI_HO-IND message with HO Event Code 0b00 to the S-ABS before the expiration of Disconnect Time. The serving ABS should prioritize the HO candidates by presenting the candidate ABSs in descending order of priority in the AAI_HO-CMD message.

## Change 4

On page 286, line 21:

If all target ABSs included in the AAI_HO-CMD message are unreachable (as defined in this section) or if the AAI_HO-CMD message includes no target ABS, and if the AMS has a preferred target ABS it shall inform the serving ABS of its preferred target ABS by sending the AAI_HO-IND message with HO Event Code 0b01 prior to expiration of Disconnect Time.

## Change 5

On page 291, line 60:

After HO is initiated, the handover could be canceled by AMS at any phase during HO procedure. The HO cancellation is initiated before the expiration of the Resource_Retain_Time. An AMS requests HO cancellation to the serving ABS by sending the AAI_HO-IND with HO Event Code 0b11 (HO cancel) with its current CMAC KEY COUNT after Disconnect Time.

## Change 6

On page 292, line 17:

When the AMS has detected a drop during network reentry with a target ABS, it may attempt network reentry with its preferred target ABS as through Cell Reselection, which may include resuming communication with the serving ABS by sending AAI_HO-IND message with HO Event Code = 0b11 (HO cancel) or performing network reentry at the serving ABS.